3GPP TSG RAN Meeting #109					RP-252147

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.3 Requirements for architecture and migration

Source:	Thales, ESA, Airbus DS, Fraunhofer IIS, Fraunhofer HHI, Eutelsat, Iridium, Gatehouse Satcom, Novamint, Echostar, ST Engineering iDirect

Title:	Architecture/migration requirements for NTN

Document type: pCR

Document for:	Approval

# 1	Introduction

This document aims at providing requirements for NTN in clause 5.2 “Requirements for architecture and migration” and 5.3 “Operational requirements” of the TR 38.914.

# 2	Discussion

6G should be able to cover all people on Earth, and all areas, including oceans. With the seamless integration of the various 3GPP-compliant satellite networks into terrestrial networks, it shall be possible to deliver global coverage for basic mobile broadband, FWA, and IoT services.

While in the context of 5G, the support of NTN was added once the basic design principles of the New Radio (NR) access were already defined, it is strongly recommended to define and specify the enablers to support NTN as part of the 6G radio access definition right from the initial 6G specification in release 21. This approach will allow for:

- Achieving ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improving network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables.

By integrating the fundamental NTN enablers in the initial stages of 6G development, users will experience several benefits, including seamless connectivity regardless of geographic location. This will empower individuals, communities, and industries in the most isolated regions, ensuring that they are not left behind in the digital revolution. Moreover, enhanced network resiliency will provide robust and reliable communication channels during emergencies, significantly reducing downtime and maintaining critical infrastructure and services. This harmonized design approach for both TN and NTN components will pave the way for a resilient, flexible, and inclusive communication network for future generations.

A key differentiator for the 6G solution for NTN is to provide ubiquitous connectivity with increased resilience, e.g., by not relying on GNSS for access procedures, as the challenge with GNSS is that it is vulnerable to malfunction, interference, and attacks (i.e., jamming and spoofing). The introduction of Positioning, Navigation, and Timing in 6G enables services without depending on GNSS. More specifically, GNSS-free operation will result in improved resilience for critical services (e.g., public safety) and reduced complexity/battery consumption for demanding use cases (e.g., massive IoT). 6G should support NTN/TN integration from day 1 to provide efficient complimentary coverage of rural and remote areas currently underserved. Also, 6G will comprise a single RAT, covering various use cases including massive IoT and eMBB. Last for smartphones and other battery operated equipment, running the GNSS function drains the battery more quickly.

6G radio should be designed also to optimize the performances and network resilience compared to 5G in several folds:

- Physical layer operation independent form third-party component such as a GNSS system.

- Ability to extend coverage in adverse propagation conditions (e.g. outdoor and indoor environments)

- Reduced PAPR on the downlink to maximize the useful capacity with respect the available energy on board

- Reduced latency due to re transmission by operating at low error rate

Resilience aspects related to NTN are discussed in contribution RP-251946.

# 3 Text proposal for TR 38.914

Based on the above discussion we propose the following text proposal.

====================Start of the changes==================

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

…

The interworking (e.g. mobility, roaming) between 6G TN/NTN and 5G NTN shall be supported.

5G core network shall be able to support a 6G RAT.

6G core network shall be able to support 5G and 4G NTN RATs (NR-NTN and IoT-NTN).

## 5.3	Operational requirements

Editor note: Spectrum, co-existence, topology and other operational requirements

### 5.3.1	Spectrum

Editor note: e.g. spectrum and co-existence related

5.3.1.x NTN specific spectrum requirements

The service link of non-terrestrial networks, refers to the radio link between satellite(s) and User Equipment, and will use the 3GPP defined 6G radio interface. It operates in satellite service allocated frequency bands which can be paired (Downlink and Uplink in separate bands) or unpaired (Downlink and Uplink in the same band). While paired bands can apply to all orbits, unpaired bands are restricted to the Low Earth Orbits.

Satellite service links can operate in numerous frequency bands including

- FR1-NTN frequency range (below 14.5 GHz): L band (1-2GHz), S band (2-4GHz), C band (3.4-6.725 GHz), Ku band (10.7-14.5 GHz):

- FR2-NTN frequency range (above 10.7 GHz): Ku band (10.7-14.5 GHz), Ka band (17.3-21.2 GHz, 27.0-30.0 GHz) and Q/V bands (37.5-43.5 GHz, 47.2-50.2 GHz and 50.4-51.4 GHz).

Channel bandwidth scalability

The 6G Radio design shall be able to support channel bandwidth between [~1] MHz and 800 MHz (including native support of 5, 10, 20, 40, 50, 100, 200 and 400 MHz etc.)

Duplexing mode

The 6G Radio design shall be able to support

- FDD (Frequency Division Duplexing), and TDD (Time Division Duplexing) as well as Half Duplex Frequency Division Duplexing (HD-FDD) mode at UE level

- Full-duplex and Half-duplex FDD at Network side (Satellite payload).

TN/NTN spectrum sharing

TBD

### 5.3.2	Diverse device types

5.3.2.x NTN specific device types

The characteristics of the NTN capable UE are listed in Table 5.3.2.x

Table 5.3.2.x: Attributes for Non-Terrestrial Network capable UE

| UE types | Min antenna gain | Min Transmit Power | Max Noise figure | UE Ground speed | Coverage |
| --- | --- | --- | --- | --- | --- |
| IoT and smartphones (FR1-NTN) | -5 dB | 20 dBm | 7 dB | Up to 500 km/h (on board high speed trains) | Outdoor, Light indoor (NOTE1) |
| VSAT: Vehicle (NOTE2, NOTE3) and building mounted | Equivalent to the one of 20 cm aperture antenna (FR2-NTN), 0 to 3 dBic (FR1-NTN) | Equivalent to 26 dBm (FR1-NTN) and 2 Watts (FR2-NTN) | [4] dB (FR2-NTN), 7 dB (FR1-NTN) | Up to 1500 km/h (aircraft) Up to 250 km/h (cars) Up to 500 km/h (high speed train) Up to TBD km/h (NGSO satellite) | Outdoor only |
| NOTE1: Light indoor requires Maximum Coupling Loss comparable to 3GPP defined NB-IoT. It may lead to service rate degradation. NOTE2: Vehicle refer here to cars, buses, trains, drones, vessels, aircrafts and possibly satellites NOTE3: For cars, see RP-232733 and 5GAA “Maximising the benefit of future satellite communications for automotive” |  |  |  |  |  |

In the table above FR2-NTN refers to all equipment operating in bands above 10 GHz (e.g., it includes Ku band equipment using FR1-NTN or FR2-NTN numerology).

For drones/UAV, not only UE ground speed is an important attribute, but also the acceleration. Hence "UE speed acceleration" values TBD.

The 6G Radio access network shall support all of the above mentioned UE types.

### 5.3.3	RAN sharing

5.3.3.x NTN specific RAN sharing requirements

The 6G Radio access network shall be able to support the sharing between several network operators (mobile and satellite) of a multi orbit space segment.

### 5.3.4	Ease of operation and Self Organization

### 5.3.5	Network Resilience

5.3.5.x NTN specific resiliency requirements

Given that GNSS services may be unavailable or degraded, the operation over NTN access for communication service or for positioning, navigation and timing service should be possible independently from any service not under the control of the satellite network operator, e.g., external GNSS service.

The support of mobility and simultaneous multi connectivity between NTN and TN components or between two NTN components (e.g. GSO and NGSO) is beneficial for ubiquitous coverage, service continuity and/or resiliency purposes. Multi-connectivity can provide seamless continuity (no added data latency during handovers), robust mobility (connection failures minimized) and increased throughput.

Note: Given that the NTN may feature a different bandwidth compared to the one offered by TN, only part of the traffic (most critical one) may be duplicated via the NTN connectivity.

Radio level

The 6G Radio design shall be able to support a GNSS-independent physical layer operation for NTN, to provide resilience in case of service degradation or unavailability of third-party systems, such as Global Navigation Satellite Systems (GNSS).

Network level

The 6G RAN shall enable NTN to act as a backup component for the terrestrial network, with capacity that can be prioritized for the most critical services. Voice services and low data rate connection suitable for messaging shall be supported.

The 6G RAN shall enable GSO to act as a backup and/or capacity augmentation layer for a NGSO network component in a multi orbit NTN infrastructure.

### 5.3.6	[Service awareness in RAN]

## 5.x	Functional requirements

### 5.x.x	Unified 6G Radio and RAN Architecture for TN and NTN

The 6G Radio and RAN architecture shall be unified and shall be able to support the full range of targeted Terrestrial Network (TN) and Non-Terrestrial Network (NTN) use cases/deployment scenarios, including massive IoT deployments. The same design principles will be applied to both TN and NTN, as they are specifically defined to take into account the unique radio link and architectural characteristics of each network component.

### 5.x.x NTN and TN Interworking

The interworking between TN and NTN components of 6G is imperative and should be taken into account right from the start.

The 6G RAN architecture and protocols shall be able to support seamless idle and connected mobility within and between terrestrial and non-terrestrial components of the network with minimum service degradation.

6G Mobility procedures shall minimise UE battery life impact and interruption delay during HO

### 5.X.x Network energy efficiency

The 6G RAN shall be able to operate with reduced energy consumption compared to 5G by a factor of [TBD] at similar performance (e.g. throughput over a given area).

It shall offer a mode allowing to further reduce the energy consumption with reduced performances (e.g. Throughput) to provide service (e.g. during extended duration) when battery activated (BS in disaster relief situation or BS on board satellite).

### 5.x.x Enhanced MCL capability

The 6G Radio shall be designed to

- enable the increase of the Maximum Coupling Loss at least for messaging services in adverse propagation conditions (e.g. UE in vehicle/indoor),

- allow operation at low error rate

- and maximise the energy efficiency per bit (e.g., consider PAPR vs. throughput) at SAN level on the downlink.

### 5.x.x Multi connectivity

The 6G Radio Access Network architecture shall be able to support Dual Connectivity or Multi-connectivity between two NTN layers (e.g. GSO and NGSO) as well as between NTN and TN for QoS enhancement with simultaneous traffic flow distributed on both/the different access links.

### 5.x.8 Satellite payloads

The 6G Radio Access Network architecture shall be able to support Transparent (RF only or RU on board), Regenerative (Full base station or Functional BS split on board).

5.x.x Security related for NTN component

6G RAN shall be able to support measures to ensure the security between different Networks operators (Mobile and Satellite) sharing a NTN component, especially to address distributed intelligence on board satellites in motion interconnected via ISL (Inter Satellite Links) in a multi orbit network infrastructure and intermittently connected with ground network component.

The 6G radio shall provide means to enhance the security of the control plane (e.g. common signaling).

==================End of the changes========================

# 4	Conclusion

Proposal: Add the text proposal in clauses 5.2 and 5.3 of the TR 38.914.