3GPP TSG RAN Meeting #109					RP-252162

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	KT Corp., Thales, Iridium

Title:	NTN preferred/priority mode for 6G

Document for:	Discussion

# 1	Introduction

As described in the objective of SID: Study on 6G Scenarios and requirements [1], study aims to develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. Also, 6G aims to harmonize 6G Radio design for TN and NTN including their integration. As support of both 6G TN and NTN is expected to be supported as a Day-1 feature, 6G NTN will provide connectivity for the underserved areas, maritime coverage, and even TN coverage area suffering from disaster situations such as fire, earthquakes, etc. 6G NTN will enable achieving better resiliency and connecting the unconnected as described in ITU-R M.2160 [2]. This contribution proposes to introduce “NTN preferred/priority-mode” which can be applied to cases when TN infrastructure is suffering from disaster situations and UE is within the area where NTN can serve better coverage.

# 2	Discussion

6G will be able to cover all areas with help of 6G NTN which is to be supported as a Day-1 feature of 6G. This will be a great complementary to MNOs where deployment of terrestrial infrastructure is challenging such as oceans, aerials, or even with the terrestrial coverage considering the spectrum 6G will likely to be utilized. Moreover, 6G NTN will also allow providing connectivity in the disaster situations where terrestrial infrastructure is out-of-service due to fire, earthquake, etc.

For above reason we can consider introducing “6G preferred/priority radio access technology ” so that UEs in certain situations can camp onto 6G NTN cells instead of searching for TN cells where TN coverage is temporarily unavailable. This will allow to ensure a quasi seamless user experience to 6G subscribers and help UE to save battery life by not searching for the terrestrial radio access technology which is unavailable.

# 3 Text proposal

Based on the discussion above we propose the following text proposal.

## 5.4	Requirements of New and Existing Services

### 5.4.x	Change of preferred/priority radio access technology

Subject to operator’s policy, the 6G network shall be able to configure the UE to access the network via NTN as a preferred/priority radio access technology when TN coverage is not available due to disaster situations or TN temporary failure in a given area.

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914.

# 5	Reference

[1] RP-251395, SID: Study on 6G Scenarios and Requirements

[2] ITU-R M.2160, Framework and overall objectives of the future development of IMT for 2030 and beyond