3GPP TSG RAN #109	RP-252169

Beijing, China, 15th – 18th September, 2025

Agenda item:		8.2.1.2

Source:	MediaTek Inc.

Title:	6G coverage requirements

Document for:					Discussion and Decision

# Introduction

This contribution discusses the Coverage requirements for 6G and makes corresponding proposals.

# Discussion

## Status in 3GPP

The following objectives are captured within the RAN plenary SID [1] in relation to coverage:

- Enhanced overall coverage, focus on cell-edge performance and UL coverage.

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

However, there is no requirement currently captured in the TR 38.914 [2] for 6G coverage. Furthermore, at RAN1#122 there was some discussion about how to study coverage improvements, and the following was agreed:

Agreement

- On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

While we appreciate the proactivity of RAN1 to identify targets, RAN plenary has the responsibility to define 6G radio requirements. Therefore, any 6G coverage target values should be decided by RAN plenary. RAN plenary of course can task RAN WGs to provide further input/analysis where needed to help such a decision.

Proposal 1: RAN plenary to define targets for 6G coverage. RAN plenary can task RAN WGs to provide further input/analysis where needed to help RAN plenary make its decision.

## Considerations on internal 3GPP target requirements

General enhanced coverage

If 6G can provide basic connectivity in areas where basic service connectivity for 5G is not feasible from the same site/band, then we believe that can be a significant selling point for 6G.

From experience of NB-IoT/cat-M and 5G NR mechanisms to extend coverage, it can be observed that certain coverage extension techniques can also impact the achievable QoS and achievable spectrum efficiency and capacity of the cell. Therefore, spectral efficiency degradation from providing enhanced coverage in a frequency band in typical multi-band spectrum deployments should be taken into account, as well as the feasible QoS provided for the UE requiring the maximum level of enhanced coverage.

With regards to different types of devices with different constraints related to form-factor etc, we propose that coverage extension is targeted and applicable for all kinds of 6G device. However, the achievable QoS vs coverage will depend on the constraints of the device.

Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7GHz We assume that the goal of operators from the term “comparable coverage” part of the objective in [1] would be that, it means comparable coverage and corresponding QoS for the cell edge user. We also assume that handheld and maybe CPE/FWA devices are the key focus, rather than potential low-capability devices.

Functional considerations

While this contribution focuses on terrestrial coverage, there is also the following RAN SID objective:

- Aim at a harmonized 6G Radio design for TN and NTN, including their integration

Therefore, 3GPP should target harmonization (including simple extensibility) between TN and NTN of functionality for coverage enhancement.

## Recommended 6G coverage targets

Considering all of the above, and to make progress on this important area at RAN plenary, we propose the following framework and tentative targets as a starting point for defining requirements for 6GR terrestrial coverage:

- Target a [10] dB Maximum Coupling Loss improvement vs 5G NR Rel-16 for basic connectivity for all types of devices in sub-6GHz bands. The achievable QoS should be identified further by RAN1, with inputs also from RAN4.

- Re-use of existing 5G mid-band (~3.5GHz) site grid shall be targeted for 6G deployments in around 7 GHz and comparable coverage and corresponding cell-edge user QoS to 5G mid-band should be targeted. Handheld devices should be considered as the reference in both cases, with 5G NR based on Rel-16. The exact 5G mid-band site grid assumptions to be used as a reference are TBD.

For clarity, we are proposing these requirements only as internal targets for 3GPP. For IMT-2030 ITU-R TPRs, we believe link budget template is sufficient (same as IMT-2020 approach).

# 3	Proposal

Proposal 1: RAN plenary to define targets for 6G coverage. RAN plenary can task RAN WGs to provide further input/analysis where needed to help RAN plenary make its decision.

Proposal 2: Consider the following framework and tentative targets as a starting point for defining requirements for 6GR terrestrial 6GR coverage:

- Target a [10] dB Maximum Coupling Loss improvement vs 5G NR Rel-16 for basic connectivity for all types of devices in sub-6GHz bands. The achievable QoS should be identified further by RAN1, with inputs also from RAN4.

- Re-use of existing 5G mid-band (~3.5GHz) site grid shall be targeted for 6G deployments in around 7 GHz and comparable coverage and corresponding cell-edge user QoS to 5G mid-band should be targeted. Handheld devices should be considered as the reference in both cases, with 5G NR based on Rel-16. The exact 5G mid-band site grid assumptions to be used as a reference are TBD.

Proposal 3: If the above is agreeable, agree to the TP to TR 38.914 provided in the annex of this document.

# 4	References

[1]	RP-250810, “Revised SID: Study on 6G Scenarios and requirements”; CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.

[2]	3GPP TR 38.914 v0.1.0 “Study on 6G Scenarios and Requirements”.

# ANNEX: Text Proposal to TR38.914

### 5.1.xx			Coverage

Target a [10] dB Maximum Coupling Loss improvement vs 5G NR Rel-16 for basic connectivity for all types of devices in sub-6GHz bands. The achievable QoS should be identified further by RAN1, with inputs also from RAN4.

Re-use of existing 5G mid-band (~3.5GHz) site grid shall be targeted for 6G deployments in around 7 GHz and comparable coverage and corresponding cell-edge user QoS to 5G mid-band should be targeted. Handheld devices should be considered as the reference in both cases, with 5G NR based on Rel-16. The exact 5G mid-band site grid assumptions to be used as a reference are TBD.