3GPP TSG-RAN WG5 Meeting #108	R5-254268

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 1044 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test point analysis for the reference sensitivity test case for NR NTN with flexible Tx-Rx separation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test point analysis was updated for the new test case related to reference sensitivity tests for NR NTN with flexible Tx-Rx separation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates the table 4.4-2 with the test points for the new RF test case for NTN. The number of test points has been calculated based on 5 environment conditions, two 2 test frequencies and 2 channel bandwidths corresponding to the lowest and the highest Tx-Rx separation as specified in the core specifications TS 38.101-5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance of flexible Tx-Rx separation for NTN UEs cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.4	Test point analysis for satellite access test cases in TS 38.521-5

This section contains information on test point selection for NR NTN test cases in [10]. The general rules in this section apply to all the NR NTN test cases. Separate analysis is not provided for each single test case unless specific 	test requirement deviates from the general rules.

General rules of test point selection:

Considering that NR NTN operating bands are defined like FR1 FDD bands, test environment, frequencies, SCSs, channel bandwidths, waveforms, and modulations of a Tx or Rx test for an NR NTN band should be selected based on the same principles of the corresponding single carrier test for an FR1 FDD band.

Table 4.4-1: NR UE transmitter test point selection for NTN

| Subclause | Number of test points | Justification | Comments |
| --- | --- | --- | --- |
| 6.2.1 UE maximum output power | 540 | General rules of test point selection apply. Test points of TC 6.2.1 from TS 38.521-1 can be leveraged. | RAN5#98 |
| 6.2.2 Maximum Output Power Reduction (MPR) | contiguous allocation: 870 | General rules of test point selection apply. Test points of TC 6.2.2 from TS 38.521-1 can be leveraged except for the test point for almost contiguous allocation and the test points for 256QAM. Note: There is no test point for almost contiguous allocation because test channel bandwidth for almost contiguous resource allocation should be greater than 20MHz, which is not supported by NR NTN operating bands. | RAN5#107 |
| 6.2.3 UE additional maximum output power reduction | 126 for NS_100; 234 for NS_24 | General rules of test point selection apply. AMPR test points for NS_100 and NS_24 in TC 6.2.3 of TS 38.521-1 can be leveraged except for the test points for 256QAM. | RAN5#107 |
| 6.2.4 Configured transmitted power | 30 | General rules of test point selection apply. Test points of TC 6.2.4 from TS 38.521-1 can be levereage except for Test ID 3, which applies to TDD bands only. | RAN5#99 |
| 6.3.1 Minimum output power | 45 | General rules of test point selection apply. Test points of TC 6.3.1 from TS 38.521-1 can be leveraged. | RAN5#98 |
| 6.3.3 Transmit On/OFF Time Mask | 40 | General rules of test point selection apply. Test points of TC 6.3.3 from TS 38.521-1 can be leveraged. | RAN5#100 |
| 6.4.1 Frequency error | 15 | 38.521-5_TPanalysis_6.4.1_FreqErr_v1.zip | RAN5#101 |
| 6.4.2.1 Error Vector Magnitude | PUSCH: 216 PUCCH: 36 PRACH: 36 | General rules of test point selection apply. Test points of TC 6.4.2.1 from TS 38.521-1 can be leveraged except for the test points for 256QAM. | RAN5#107 |
| 6.4.2.1a Error Vector Magnitude including symbols with transient period | PUSCH: 6 | General rules of test point selection apply. Test points of TC 6.4.2.1a from TS 38.521-1 can be leveraged except for the test points for 256QAM. | RAN5#107 |
| 6.4.2.2 Carrier Leakage | 3 | General rules of test point selection apply. Test points of TC 6.4.2.2 from TS 38.521-1 can be leveraged. | RAN5#103 |
| 6.4.2.3 In-band emissions | 36 | General rules of test point selection apply. Test points of TC 6.4.2.3 from TS 38.521-1 can be leveraged. | RAN5#103 |
| 6.4.2.4 EVM equalizer spectrum flatness | 90 | General rules of test point selection apply. Test points of TC 6.4.2.4 from TS 38.521-1 can be leveraged. | RAN5#103 |
| 6.4.2.5 EVM equalizer spectrum flatness for Pi/2 BPSK | 9 | General rules of test point selection apply. Test points of TC 6.4.2.5 from TS 38.521-1 can be leveraged. | RAN5#103 |
| 6.5.1 Occupied bandwidth | 4 | General rules of test point selection apply. Test points of TC 6.5.1 from TS 38.521-1 can be leveraged noting only CBW up to 20 MHz can be tested. | RAN5#103 |
| 6.5.2.2 Spectrum emission mask | contiguous allocation: 144 | General rules of test point selection apply. Test points applicable to PC3 from TC 6.5.2.2 of TS 38.521-1 can be leveraged except for: -: Test IDs for almost contiguous allocation because test channel bandwidth for almost contiguous resource allocation should be greater than 20MHz, which is not supported by NR NTN operating bands. - Test IDs that are specified with NOTE 3 in Table 6.5.2.2.4.1-1 of TS 38.521-1, which applies to TDD bands only - [Test IDs listed as applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16] - Test IDs for 256QAM. | RAN5#107 |
| 6.5.2.4 Adjacent channel leakage ratio | contiguous allocation: 870 | General rules of test point selection apply. Test points of TC 6.5.2.4 which are derived from Table 6.2.2.4.1-1 of TS 38.521-1, applicable to PC3, can be leveraged except for: - Test IDs for almost contiguous allocation because test channel bandwidth for almost contiguous resource allocation should be greater than 20MHz, which is not supported by NR NTN operating bands. - Test IDs that are specified with NOTE 3 in Table 6.2.2.4.1-1 of TS 38.521-1, which applies to TDD bands only - Test IDs for 256QAM. | RAN5#107 |
| 6.5.3.1 General spurious emission | 27 | General rules of test point selection apply. Test points of TC 6.5.3.1 from TS 38.521-1 can be leveraged | RAN5#100 |
| 6.5.3.2 Spurious emissions for UE co-existence | 27 | General rules of test point selection apply. Test points of TC 6.5.3.2 from TS 38.521-1 can be leveraged | RAN5#100 |
| 6.5.4 Transmit intermodulation | 8 | General rules of test point selection apply. Test points of TC 6.5.4 from TS 38.521-1 can be leveraged | RAN5#100 |

Table 4.4-2: NR UE receiver test point selection for NTN

| Subclause | Number of test points | Justification | Comments |
| --- | --- | --- | --- |
| 7.3.2 Reference sensitivity power level | 45 | General rules of test point selection apply. Test points of TC 7.3.2 from TS 38.521-1 can be leveraged | RAN5#100 |
| 7.3.2_1 Reference sensitivity power level with flexible Tx-Rx separation | 20 | TS 38.521-5 limits the tests for variable Tx-Rx separation testing to lowest and largest channel bandwidth and Tx-Rx separation values. For other parameters, rules of test point selection for TC 7.3.2 from TS 38.521-1 can be leveraged. | RAN5#108 |
| 7.4 Maximum input level | 3 | General rules of test point selection apply. Test points of TC 7.4 from TS 38.521-1 can be leveraged limiting the modulation to 64QAM. | RAN5#99 |
| 7.5 Adjacent channel selectivity7.5 Adjacent channel selectivity | 3 | General rules of test point selection apply. Test points of TC 7.5 from TS 38.521-1 can be leveraged. | RAN5#101 |
| 7.6.2 In-band blocking | 3 | General rules of test point selection apply. Test points of TC 7.6.2 from TS 38.521-1 can be leveraged. | RAN5#101 |
| 7.6.3 Out of Band Blocking | 3 | General rules of test point selection apply. Test points of TC 7.6.3 from TS 38.521-1 can be leveraged. | RAN5#101 |
| 7.6.4 Narrow band blocking | 3 | General rules of test point selection apply. Test points of TC 7.6.4 from TS 38.521-1 can be leveraged. | RAN5#101 |
| 7.7 Spurious response | 3 | Same test points as TC 7.6.3 of TS 38.521-5. | RAN5#101 |
| 7.8.2 Wide band Intermodulation | 3 | General rules of test point selection apply. Test points of TC 7.8.2 from TS 38.521-1 can be leveraged. | RAN5#101 |
| 7.9 Spurious emissions | 3 | General rules of test point selection apply. Test points of TC 7.9 from TS 38.521-1 can be leveraged. | RAN5#101 |

Table 4.4-3: NS value specific test points for A-MPR, A-SEM and A-SE for NTN

| NS label | Number of test points for A-MPR | Number of test points A-SEM | Number of test points A-SE | Justification | Comments |
| --- | --- | --- | --- | --- | --- |
| NS_03N | 56 |  | 56 | “38.521-5_TPanalysis_6.2.3_AMPR_NS_03N_v2.zip” | RAN5#107 |
| NS_04N | 24 |  | 24 | “38.521-5_TPanalysis_6.2.3_AMPR_NS_04N and NS_05N_v2.zip” | RAN5#107 |
| NS_05N | 312 |  | 312 | “38.521-5_TPanalysis_6.2.3_AMPR_NS_04N and NS_05N_v2.zip” | RAN5#107 |

END OF CHANGES >>