3GPP TSG-RAN5 Meeting #108 	R5-254723

# Bengaluru, India, 25th August 2025 – 29th August  2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5550 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to LTE based 5G broadcast PBCH detection performance test cases. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_bands_part2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Update needed for LTE based 5G broadcast PBCH detection performance test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated some of the pending aspects for LTE based 5G broadcast PBCH detection performance test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - LTE based 5G broadcast PBCH detection performance test cases will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## 10.4	FDD with 5G terrestrial broadcast

### 10.4.0	Applicability of requirements

<< Skipped sections >>

#### 10.4.2.1		Minimum requirement for PBCH detection

Editor’s note: This test case is incomplete. Following aspects are pending

- Initial cal setup

- Test procedure

- Message contents

- MU/TT analysis

- Minimum test time

For the parameters specified in Table 8.6.1-1, the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified value in Table 10.4.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 10.4.2.1-1: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-bch (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x1 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.23-1 | N/A | AWGN | 1x1 low | 1 | -7.2 | ≥ 2 |
| 2 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x2 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.23-1 | N/A | EVA 162Hz | 1x2 low | 1 | -6.7 | ≥ 2 |

##### 10.4.2.1.1	Test purpose

This test verifies the PBCH performance with a given SNR for which the average probability of a mis-detected PBCH (Pm-bch) remains below a given reference value.

##### 10.4.2.1.2	Test applicability

This test applies to all types of E-UTRA FDD UE release 17 and forward and supporting 5G terrestrial broadcast and supporting CAS.

##### 10.4.2.1.3	Test description

10.4.2.1.3.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: As specified per test number in Table 10.4.2.1-1 as defined in TS 36.508 [7] clause 4.3.1.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connector (s) as shown in TS 36.508 [7] Annex A, [Figure A.9].

2.	The parameter settings for the cell are set up according to Table 10.4.2.1-1.

3.	The downlink signals are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B clause B.0.

5.	Ensure the UE is in [State 2A-RF] according to TS 36.508 [7] clause 5.2A.1A. Message contents are defined in clause 10.4.2.1.4.

6.	SS transmits MBSFNAreaConfiguration message. Message content is defined in clause 10.4.2.1.4.

7.	Wait for a period equal to the MCCH modification period to make sure the UE has received the MBSFNAreaConfiguration message.

8.	SS continues with the generic procedures described in TS 36.508 [7] clause 4.5.3A.3 and 4.5.4.3 and [ensures the UE is in State 4 according to TS 36.508 [7] clause 4.5.4 and the UE test loop Mode C is activated]. Message contents are defined in clause 10.4.2.1.4.

9.	SS is configured to include 10 MBMS packets in one TB.

10.4.2.1.3.2	Test procedure

FFS

##### 10.4.2.1.4	Message contents

Message contents are according to TS 36.508 [7] clauses 4.4, 4.6 and 4.7A with following exceptions:

Table 10.4.2.1.4-1: MasterInformationBlock-MBMS: Additional FDD 5G terrestrial broadcast PMCH performance (Fixed Reference Channel)

| Derivation Path: 36.508 table FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-MBMS-r14 ::= SEQUENCE { |  |  |  |
| dl-Bandwidth-MBMS-r14 | As specified per test number in Table 10.4.1.1-2 |  |  |
| systemFrameNumber-r14 | FFS |  |  |
| semiStaticCFI-MBMS-r16 | FFS |  |  |
| oneFrame | FFS |  |  |
| } |  |  |  |

Table 10.4.2.1.4-2: SystemInformation-MBMS : Additional FDD 5G terrestrial broadcast PMCH performance (Fixed Reference Channel)

| Derivation Path: 36.508 table FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation-MBMS-r14 ::= |  |  |  |
| SystemInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r8 | FFS |  |  |
| criticalExtensionsFuture-r15 CHOICE { |  |  |  |
| posSystemInformation-r15 | FFS |  |  |
| criticalExtensionsFuture | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.4.2.1.4-3: SystemInformationBlockType1-MBMS: Additional FDD 5G terrestrial broadcast PMCH performance (Fixed Reference Channel)

| Derivation Path: 36.508 table FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-MBMS-r14 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-r14 SEQUENCE { |  |  |  |
| plmn-IdentityList-r14 |  |  |  |
| trackingAreaCode-r14 |  |  |  |
| cellIdentity-r14 |  |  |  |
| } |  |  |  |
| freqBandIndicator-r14 | FFS |  |  |
| multiBandInfoList-r14 | FFS |  |  |
| schedulingInfoList-MBMS-r14 | FFS |  |  |
| si-WindowLength-r14 | FFS |  |  |
| systemInfoValueTag-r14 | FFS |  |  |
| nonMBSFN-SubframeConfig-r14 | FFS |  |  |
| pdsch-ConfigCommon-r14 | FFS |  |  |
| systemInformationBlockType13-r14 | FFS |  |  |
| cellAccessRelatedInfoList-r14 | FFS |  |  |
| } |  |  |  |

Table 10.4.2.1.4-4: SystemInformationBlockType2: Additional FDD 5G terrestrial broadcast PMCH performance (Fixed Reference Channel)

| Derivation Path: 36.508 table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| mbsfn-SubframeConfigList SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF SEQUENCE { |  |  |  |
| radioframeAllocationPeriod | n1 |  |  |
| radioframeAllocationOffset | 0 |  |  |
| subframeAllocation CHOICE{ |  |  |  |
| oneFrame | '111111' B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.4.2.1.4-5: SystemInformationBlockType13: Additional FDD 5G terrestrial broadcast PMCH performance (Fixed Reference Channel)

| Derivation Path: 36.508 table 4.4.3.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType13 ::= SEQUENCE { |  |  |  |
| mbsfn-AreaInfoList-r9 SEQUENCE (SIZE(1..maxMBSFN-Area)) OF SEQUENCE { |  |  |  |
| mcch-Config-r9 SEQUENCE { |  |  |  |
| mcch-RepetitionPeriod-r9 | rf32 |  |  |
| mcch-Offset-r9 | 1 |  |  |
| mcch-ModificationPeriod-r9 | rf512 |  |  |
| sf-AllocInfo-r9 | ‘100000’ B |  |  |
| signallingMCS-r9 | n7 | Test number 1 | QPSK |
|  | n13 | Test number 2 | 16QAM |
|  | n19 | Test number 3 | 64QAM |
| } |  |  |  |
| } |  |  |  |
| notificationConfig-r9 SEQUENCE { |  |  |  |
| notificationRepetitionCoeff-r9 | n4 |  |  |
| notificationOffset-r9 | 1 |  |  |
| notificationSF-Index-r9 | 1 | Subframe #1 |  |
| } |  |  |  |
| } |  |  |  |

##### 10.4.2.1.5	Test requirement

For the parameters specified in Table 8.6.1-1, Table 10.4.2.1-1, Annex A.3.18.1, and SNR in Table 10.4.2.1.5-1, the value for the average probability of mis-detected PBCH (Pm-bch) shall be below the test limit in Annex [TBD] for all subtests shown in Table 10.4.2.1.5-1.

Table 10.4.2.1.5-1: Test requirement

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x2 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.107 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 6 | R.107-1 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 7 | R.107-2 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |
|  | MBMS Dedicated Cell | 8 | R.107-3 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 + TT | ≥ 2 |

<< End of Changes >>