3GPP TSG-RAN WG4 Meeting #116	R4-2510842

Bangalore, India, 25th – 29rd August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0811 | rev |  | Current version: | 17.17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on PDSCH Requirements for 4Rx HST CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR1_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Table 5.2A.3.4-5, the table number 5.2A.3.14-3 is wrong, should be replaced by 5.2A.3.4-3. In Table 5.2A.3.5-8, the second test number should be 2-2 instead of 2-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of changes in this CR as below: - update Table 5.2A.3.4-5 - update Table 5.2A.3.5-8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of test number 3 in 5.2A.3.4 for minimum performance for multiple CA configurations is wrong, and the second test number should be updated, otherwise may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.3.4, 5.2A.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes 1>

5.2A.3.4	Minimum requirements for HST-SFN CA

For HST-SFN CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.3.4-5 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.3.4-3 ~ Table 5.2A.3.4-4, with the parameters in Table 5.2A.3.4-2, Table 5.2A-2, Table 5.2A-3, and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

The test purpose is specified in Table 5.2A.3.4-1.

Table 5.2A.3.4-1: Test purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions in the HST-SFN scenario defined in B.3.2 with CA | 1, 2, 3 |

Table 5.2A.3.4-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | FDD: 10 for CSI-RS resource 1,2,3,4. TDD: 20 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. |
|  | Frequency Occupation |  | Start PRB = 0 Number of PRB = 52 for TDD with BW 20MHz and higher BW Number of PRB = 52 for FDD with BW 15MHz and higher BW Number of PRB = ceil(BWP size/4)*4 for other case |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  | 15 kHz SCS: FR1.15-1 30 kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 |
| Number of PUCCH ResourceGroups |  |  | 1 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases with no more than 2 DL CCs PUCCH format 3 for cases with more than 2 DL CCs |

Table 5.2A.3.4-3: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-13.1 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 10.5 |
| 10 | R.PDSCH.1-8.3 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 10.7 |
| 15 | R.PDSCH.1-13.2 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.1 |
| 20 | R.PDSCH.1-13.3 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.5 |
| 25 | R.PDSCH.1-13.4 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.6 |
| 30 | R.PDSCH.1-13.5 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.8 |
| 35 | R.PDSCH.1-14.3 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.7 |
| 40 | R.PDSCH.1-14.1 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.9 |
| 45 | R.PDSCH.1-14.4 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.7 |
| 50 | R.PDSCH.1-14.2 FDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.9 |

Table 5.2A.3.4-4: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-19.1 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.0 |
| 10 | R.PDSCH.2-19.2 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.8 |
| 15 | R.PDSCH.2-19.3 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.1 |
| 20 | R.PDSCH.2-19.4 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.8 |
| 25 | R.PDSCH.2-19.5 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 11.9 |
| 30 | R.PDSCH.2-20.1 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.2 |
| 40 | R.PDSCH.2-10.4 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.4 |
| 50 | R.PDSCH.2-20.2 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.6 |
| 60 | R.PDSCH.2-20.3 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.5 |
| 80 | R.PDSCH.2-20.4 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.7 |
| 90 | R.PDSCH.2-20.5 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.7 |
| 100 | R.PDSCH.2-21.1 TDD | 16QAM, 0.48 | HST-SFN | 2x4 | 70 | 12.7 |

Table 5.2A.3.4-5: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.4-3 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.4-4 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.4-3 and Table 5.2A.3.4-4 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in Section 5.1.1.7. |  |  |

<End of changes 1>

<Start of changes 2>

#### 5.2A.3.5	Minimum requirements for PDSCH HST-DPS CA

For HST-DPS CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.3.5-7 and Table 5.2A.3.5-8 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.3.5-3 - Table 5.2A.3.5-6, with the parameters in Table 5.2A.3.5-2, Table 5.2A-2 and Table 5.2A-3 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

The test purpose is specified in Table 5.2A.3.5-1.

Table 5.2A.3.5-1: Test purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance of UE under 4 receive antenna conditions in the HST-DPS scenario defined in B.3.3 with CA with 1 active TCI state | 1-1, 1-2, 1-3 |
| Verify PDSCH performance of UE under 4 receive antenna conditions in the HST-DPS scenario defined in B.3.3 with CA with 2 active TCI states | 2-1, 2-2, 2-3 |

Table 5.2A.3.5-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD and TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | FDD: 12 TDD: Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB maping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 15kHz SCS: 10 for CSI-RS resource 1,2,3,4. 30kHz SCS: 20 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #2 |
|  |  | Frequency Occupation |  | Start PRB 0 Number of PRB = min(52, ceil(BWP size/4)*4) |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 15kHz SCS: 10 for CSI-RS resource 5,6,7,8. 30kHz SCS: 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #3 |
|  |  | Frequency Occupation |  | Start PRB 0 Number of PRB = min(52, ceil(BWP size/4)*4) |
| NZP CSI-RS for CSI acquisition | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 15kHz SCS:20 30kHz SCS: 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 15kHz SCS:20 30kHz SCS: 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  |  | 15kHz SCS: FR1.15-1 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | As defined in Table 5.2A-3 |
| Number of PUCCH ResourceGroups |  |  |  | 1 |
| PUCCH format for HARQ-ACK feedback |  |  |  | PUCCH format 1 for cases with no more chan 2 DL CCs PUCCH format 3 for cases with more than 2 DL CCs |
| Note 1: SSB # (k mod 2), CSI-RS (for tracking) resource set # ((k mod 2) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) are transmitted by kth RRH. For Test 1-1, TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc+TfirstTRS+TTRS proc, 0] to slot# 2k+1n PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc, 0] to slot# 2k+1n PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. Where k=0, 1, 2… is the RRH number, n = 2520 is half of the number of slots between two RRH, THARQ = 2 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 3 is the number of slots for MAC CE processing, TfirstTRS = 6 is the number of slots to first TRS transmission occasion after MAC CE command is decoded by the UE, TTRS proc = 2 is the number of slots for TRS processing. |  |  |  |  |

Table 5.2A.3.5-3: Single carrier performance for FDD 15 kHz SCS for HST-DPS CA configurations with 1 active PDSCH TCI states

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-15.1 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.5 |
| 10 | R.PDSCH.1-8.4 FDD | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.8 |
| 15 | R.PDSCH.1-15.2 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.7 |
| 20 | R.PDSCH.1-15.3 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.5 |
| 25 | R.PDSCH.1-15.4 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.8 |
| 30 | R.PDSCH.1-15.5 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.9 |
| 35 | R.PDSCH.1-16.3 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.6 |
| 40 | R.PDSCH.1-16.1 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.7 |
| 45 | R.PDSCH.1-16.4 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.6 |
| 50 | R.PDSCH.1-16.2 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 11.0 |

Table 5.2A.3.5-4: Single carrier performance for FDD 15 kHz SCS for HST-DPS CA configurations with 2 active PDSCH TCI states

| Bandwidth (MHz) | Reference channel | Modulation format and code rate |  | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-15.1 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 10.5 |
| 10 | R.PDSCH.1-8.4 FDD | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 10.8 |
| 15 | R.PDSCH.1-15.2 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 10.7 |
| 20 | R.PDSCH.1-15.3 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 10.5 |
| 25 | R.PDSCH.1-15.4 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 10.8 |
| 30 | R.PDSCH.1-15.5 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 10.9 |
| 35 | R.PDSCH.1-16.3 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 10.6 |
| 40 | R.PDSCH.1-16.1 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 10.7 |
| 45 | R.PDSCH.1-16.4 |  | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.6 |
| 50 | R.PDSCH.1-16.2 | 64QAM, 0.43 |  | HST-DPS | 2 | 2x4 | 70 | 11.0 |

Table 5.2A.3.5-5 Single carrier performance for TDD 30 kHz SCS for HST-DPS CA configurations with 1 active PDSCH TCI states

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-22.1 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.4 |
| 10 | R.PDSCH.2-22.2 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.5 |
| 15 | R.PDSCH.2-22.3 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.5 |
| 20 | R.PDSCH.2-22.4 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.5 |
| 25 | R.PDSCH.2-22.5 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.6 |
| 30 | R.PDSCH.2-23.1 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.5 |
| 40 | R.PDSCH.2-10.5 TDD | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.5 |
| 50 | R.PDSCH.2-23.2 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.7 |
| 60 | R.PDSCH.2-23.3 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.7 |
| 80 | R.PDSCH.2-23.4 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.5 |
| 90 | R.PDSCH.2-23.5 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.7 |
| 100 | R.PDSCH.2-24.1 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.7 |

Table 5.2A.3.5-6 Single carrier performance for TDD 30 kHz SCS for HST-DPS CA configurations with 2 active PDSCH TCI states

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-22.1 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.4 |
| 10 | R.PDSCH.2-22.2 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.5 |
| 15 | R.PDSCH.2-22.3 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.5 |
| 20 | R.PDSCH.2-22.4 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.5 |
| 25 | R.PDSCH.2-22.5 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.6 |
| 30 | R.PDSCH.2-23.1 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.5 |
| 40 | R.PDSCH.2-10.5 TDD | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.5 |
| 50 | R.PDSCH.2-23.2 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.7 |
| 60 | R.PDSCH.2-23.3 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.7 |
| 80 | R.PDSCH.2-23.4 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.5 |
| 90 | R.PDSCH.2-23.5 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.7 |
| 100 | R.PDSCH.2-24.1 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.7 |

Table 5.2A.3.5-7: Minimum performance for multiple CA configurations for HST-DPS with 1 active TCI state

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1-1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.5-3 |
| 1-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.5-5 |
| 1-3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.5-3 and Table 5.2A.3.5-5 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7.4. |  |  |

Table 5.2A.3.5-8: Minimum performance for multiple CA configurations for HST-DPS with 2 active TCI states

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 2-1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.5-4 |
| 2-2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.5-6 |
| 2-3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.5-4 and Table 5.2A.3.5-6 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7.4. |  |  |

<End of changes 2>