3GPP TSG RAN #109	                                                                                      RP-252671

Beijing, China, September 15-18, 2025

Agenda item:	8.2.2

Source:	CSCN

Title:	Views on 6G Radio

Document for:	Discussion/Decision

- Introduction

The evolution toward 6G provides an opportunity to fundamentally improve the performance, efficiency, and scalability of the radio access network. In order to meet the increasing demands of diverse services and deployment, it is critical that the 6G Radio air interface design prioritizes simplicity, energy efficiency, and forward compatibility, while maintaining strong performance across all relevant spectrum bands. There were some agreements achieved in RAN1#122, and the details are as follows [1].

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. Conclusion - Numerologies for sensing is up to sensing agenda discussion. Agreement - 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR. - FFS potential need for other CP

In this contribution, we propose some details of numerology, SCS per band and frame structure for 6G Radio.

- Discussions

## 2.1 Numerology

For the design of 5G New Radio (5G-NR), significant efforts were invested, with full consideration of factors such as deployment convenience, Doppler robustness, Delay Spread (DS) robustness, latency performance, and implementation efficiency. In the design of 6GR, if applicable, we should leverage the 5G-NR solutions as much as possible to avoid redundant work. Considering that 6GR needs to support Multi-Radio Access System Sharing (MRSS) to address the lack of available new spectrum, we believe that the SCS for 6GR should be the same as the SCS for 5G-NR deployments in the same carrier, to avoid complicated mixed numerology operation.

Proposal 1: For existing 5G-NR bands, the SCS for 6GR should be the same as the SCS for 5G-NR deployments in the same carrier for MRSS purpose.

Furthermore, in 5G-NR, the SCS design is highly flexible with multiple SCS options in a band, Not only different Bandwidth Parts (BWPs) , but the Synchronization Signal Block (SSB) and the initial Downlink Bandwidth Part (initial DL BWP) also support different SCSs. However, in actual 5G-NR deployments, most of the aforementioned flexibilities are not realized. For example, 60kHz SCS for FR1 is rarely used and the SSB and initial DL BWP usually adopt the same SCS. Therefore, 6GR can eliminate these redundant and unused flexibility to reduce unnecessary implementation and testing complexity for both gNB and UE.

Proposal 2: For new 6GR bands, supports a basic set of SCS with relatively fixed options.

## 2.2	Subcarrier spacing per band

Further, from the agreements achieved in the last meeting, we observe that for FR1 (sub 6GHz) band, it supports 15kHz SCS for FDD and 30kHz SCS for TDD. Meanwhile, for FR2 band (24.25GHz-52.6GHz), 120kHz SCS is supported. Based on these existing conclusions, we conduct further detailed discussions on some FFS items, such as whether 30kHz SCS should bed supported for FDD mode in FR1.

For 6GR, we propose that the FDD mode below 6GHz needs to support 30kHz SCS. From a technical perspective, 30kHz SCS demonstrates several obvious benefits over 15kHz SCS in FDD bands, as detailed below:

- Low Latency Performance

A slot/symbol duration for 30kHz SCS is half of that for 15kHz SCS. Consequently, various time parameters such as the Transmission Time Interval (TTI), Hybrid Automatic Repeat Request (HARQ) latency, and HARQ Round-Trip Time (RTT) are reduced by half, resulting in a 50% decrease in overall latency. This is highly beneficial for latency sensitive scenarios, e.g. Non-Terrestrial Networks (NTN).

- Better Frequency Offset Resistance Performance

Compared with the 15kHz SCS, the 30kHz SCS features a wider subcarrier bandwidth and a higher proportion of cyclic prefix (CP) to the total symbol duration which means a higher tolerance for Doppler frequency shift. In high-dynamic scenarios, such as NTNand high-speed rail, the rapid movement between base station and UE causes large Doppler frequency shift, which leads to severe inter-subcarrier interference (ICI) and inter-symbol interference (ISI). In such cases, 30kHz SCS demonstrates better performance.

- Improved UE Baseband Area Efficiency

Compared with 15kHz SCS, the short-slot characteristic of 30kHz SCS reduces the requirement for baseband buffers, thereby improving the area efficiency of baseband chips.

- Simplified Tight Integration of Multi-Carrier Operations (e.g., TDD+FDD Hybrid Scenarios)

Adopting a unified SCS for all carriers in FR1 can greatly simplify multi-carrier operations and resolve various complex issues caused by mixed SCS (i.e., FDD 15kHz drags down the processing timeline of 30kHz).

- Facilitation of TN-NTN Integration

Both Terrestrial Network (TN) devices and NTN FDD devices can use a unified 30kHz SCS, which solves various complications of mixed SCSs during TN-NTN handover.

Proposal 3: For sub-6GHz (FR1), 30kHz SCS should be supported for FDD mode.

## 2.3  Frame structure and duplex mode

To meet the strong demand for Multi-Radio Access System Sharing (MRSS) deployment, it is essential for 6GR to keep the same frame and slot structure as 5G-NR.

Proposal 4: For 6GR, keep the same structure of 10ms radio frame and 14 OFDM symbols per slot as 5G-NR.

TDD mode does not require paired spectrum. It can avoid interference between downlink and uplink transmissions by introducing a Guard Period (GP) longer than the Round-Trip Time (RTT). Base stations and UE operating in TDD mode can adopt a higher transmission power spectral density compared to that in FDD mode. In addition, UE in TDD mode can more easily achieve high output power because no duplexer is needed in the front-end. Based on the above advantages, by configuring a reasonable GP to ensure time domain efficiency, TDD is feasible for non-Geostationary Orbit (NGSO) satellite systems with a regenerative architecture. Further, when applying TDD to NGSO satellite communication systems, it is meaningful to study technical approaches that can obviously improve air interface resource utilization ratio without introducing unacceptable degradation of other performances.

Proposal 5: For 6GR, the study on TDD for NGSO satellite systems should be considered.

- Conclusion

For 6GR discussion, we provide the following proposals:

Proposal 1: For existing 5G-NR bands, the SCS for 6GR should be the same as the SCS for 5G-NR deployments in the same carrier for MRSS purpose.

Proposal 2: For new 6GR bands, supports a basic set of SCS with relatively fixed options.

Proposal 3: For sub-6GHz (FR1), 30kHz SCS should be supported for FDD mode.

Proposal 4: For 6GR, keep the same structure of 10ms radio frame and 14 OFDM symbols per slot as 5G-NR.

Proposal 5: For 6GR, the study on TDD for NGSO satellite systems should be considered.

- Reference

[1] Chair notes RAN1#122