3GPP TSG RAN Meeting #109					RP-251949

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	Ericsson, CHTTL, CMCC, ESA, FirstNet, KT Corp., Nokia, T-Mobile USA, Telstra

Title:	3GPP specific requirements on NTN

Document for:	Approval

# 1	Introduction

Satellites are key to provide wireless communication services wherever they are needed even in areas not covered by terrestrial networks. By leveraging the diversity of satellite systems (e.g. LEO, MEO, GEO) 6G can provide connectivity across the globe offering a harmonized solution adapted to different needs even in remote regions.

Since digital services are key to enable education, healthcare and reduce isolation for populations, and since terrestrial infrastructures are vulnerable to war and natural disasters, non-terrestrial network infrastructure offers an opportunity to offer seamless connectivity almost everywhere.

In 5G, NTN capabilities were introduced as a complementary solution after the core terrestrial network standards were already established. In contrast, 6G should strive for an integrated approach in which NTNs are an integrated part of the whole infrastructure.

# 2	Discussion

6G should be able to cover virtually all people on Earth, and virtually all areas, including oceans. With the seamless integration of 3GPP-compliant satellites into terrestrial networks, it shall be possible to deliver global coverage for basic mobile broadband, FWA, and IoT services.

5G NTN relies on GNSS for access procedures. The challenge with GNSS is that it is vulnerable to malfunction, interference and attacks (i.e., jamming and spoofing), as well as being a bottleneck on the link budget of the NTN. Moreover, currently the 3GPP NTN technologies are fragmented depending on the use case (i.e., NR NTN and IoT NTN, which comprises NB-IoT and LTE-M).

A key differentiator for the 6G solution for NTN is to provide ubiquitous connectivity without depending on GNSS. More specifically, GNSS-free operation will result in improved resilience for critical services (e.g., public safety),reduced complexity/battery consumption for demanding use cases (e.g., massive IoT), and improved NTN coverage. 6G should support NTN/TN integration from day 1 to provide efficient complimentary coverage of rural and remote areas currently underserved. Also, 6G will comprise a single RAT, covering various use cases including massive IoT and eMBB.

Resilience aspects related to NTN are discussed in our contribution RP-251946.

# 3 Text proposal

Based on the discussion above we propose the following text proposal.

## 5.x	Functional requirements for a 6G Network with satellite access

### 5.x.x Seamless operation under mobility

The 6G RAN architecture shall support the seamless operation under mobility within and between terrestrial and non-terrestrial components of the network with minimum service disruption. The non-terrestrial components include at least GSO, MEO, LEO, multi-orbit and HAPS.

Mobility procedures shall be optimised to minimise UE battery life impact and time spent scanning for networks.

### 5.x.x GNSS-free operations

The 6G Radio design shall support GNSS-free operation for the non-terrestrial network.

### 5.x.x Harmonized radio design

The 6G Radio and 6G RAN architecture design for the TN and NTN RAT should be harmonized, and the design should be able to cover various use cases including e.g. eMBB and massive IoT.

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914.