3GPP TSG RAN Meeting #109	RP-251959

# Beijing, China, September 15-18, 2025

Agenda item:	8.2.1.4

Source:	Samsung

Title:	Requirements for Supporting Diverse Services

Document for:	Discussion and Decision

1	Introduction

6G Radio (6GR) will need to support diverse device types ranging from high-performance FWA, smartphones, and XR devices to wearables and IoT devices. Many of these emerging use cases, particularly in the IoT domain, require connectivity solutions that prioritize low cost, low power consumption, and extended coverage rather than high data throughput. Designing 6GR with a single RAT that supports different use cases offers several critical advantages:

- Simplified network deployment: A unified RAT reduces the need for deploying and maintaining multiple parallel networks (e.g., separate infrastructures for IoT and broadband services), lowering both capital expenditures (CAPEX) and operational expenditures (OPEX).

- Enhanced mobility and roaming support: A single RAT enables unified mobility management across all device types, simplifying handovers and ensuring continuous connectivity for devices regardless of their complexity or service profile.

- Simplified device ecosystem: Chipset and device manufacturers can design universal modules compatible with a single RAT, thereby reducing development costs and promoting faster market adoption across different industries.

- Future-proof evolution: A unified RAT supports future technological advancements without the need for fragmented upgrades, thereby enabling the network to evolve gracefully as new device categories and use cases emerge.

This contribution proposes requirements for a single RAT to support diverse device types such as smartphones, FWA, wearables (watch, glasses), IoT devices.

2	Discussion

Device type and UE capability signalling

To avoid unnecessary fragmentation in device implementations while allowing implementation of different functionalities depending on the market need, consider defining

- a set of device types with a corresponding set of capabilities (e.g., smartphone, FWA, wearable, IoT); and

- UE capability signalling for each device type.

To realize such hybrid approach, it would be necessary to define a set of functions that must be implemented by each device type.

UE bandwidth

Various 3GPP technologies used for different IoT services support different UE RF bandwidths. NB-IoT and eMTC support 200 kHz and 1.4 MHz, respectively. Cat-1/1bis, Cat-4, RedCap (FR1), and eRedCap support 20 MHz.

For support of high data rate devices (i.e., smartphones and FWA), we expect max 400 MHz channel bandwidth to be specified in 6GR.

To design a single RAT supporting the large range of capabilities for devices as listed above while avoiding the performance loss of fundamental channels and signals (e.g., PBCH, synchronization signals, PDCCH), it is not desirable to choose 200 kHz or 1.4 MHz as the minimum UE bandwidth.

In case of IoT devices, the implementation complexity is the primary consideration. Based on the eRedCap study, reducing RF/BB bandwidth from 100 MHz to 20 MHz leads to significant reduction of complexity and cost (e.g. 63.4% of eMBB UE as reference for HD-FDD with 1 Rx – Table 7.8.2-1 of TR 38.875). However, further reduction from 20 MHz to 5 MHz provides only limited additional benefit (e.g. 14.1% of Rel-17 RedCap as reference for HD-FDD with 1 Rx - Table 7.2.2-7 of TR 38.865) comparing to restricting PDSCH bandwidth to 5MHz  (e.g. 8.9% of Rel-17 RedCap as reference).

Based on the above observations and in order to guarantee smooth migration of IoT services from 4G/5G to 6G, we propose 20 MHz as the UE maximum RF channel bandwidth and 5 MHz as the UE minimum BB bandwidth (at least for downlink PDSCH) for IoT devices. For network energy saving, long SSB periodicities such as 80 msec or 160 msec need to be enabled. The associated UE search effort can be simplified and the SSB acquisition time can be reduced by making the sync raster sparser, which can be achieved with an SSB BW of 3 MHz for FR1 and scalable to SCS depending on the FR.

Minimum number of UE RX antennas

6GR should be designed assuming the existence of UEs with one RX antenna for cost reduction of IoT devices. Means for minimizing the impact on the overall system performance has to be considered.

3	Proposal

We propose to agree the following text proposal for TR 38.914.

=== Start Text Proposal ===

5.4	Requirements of New and Existing Services

< Proposed new sections >

5.4.x Device type and UE capability signalling

To avoid unnecessary fragmentation in device implementations while allowing implementation of different functionalities depending on the market need, 6G Radio shall define

- a set of device types with a corresponding set of capabilities (e.g., smartphone, FWA, wearable, IoT); and

- UE capability signalling for each device type.

5.4.y UE bandwidth

6G Radio shall define 5, 20, and 50 MHz as the UE minimum channel bandwidth for FR1, the frequency range between FR1 and FR2-1, and FR2-1, respectively and 3, 6, 12, and 24 MHz as the SSB bandwidth for 15, 30, 60, and 120 kHz SCS.

For 6G IoT device, the UE maximum RF channel bandwidth shall be 20 MHz and the UE minimum BB bandwidth shall be 5 MHz.

5.4.z Minimum number of UE RX antennas

6G Radio shall be designed assuming the existence of UEs with one RX antenna for cost reduction of IoT devices. Means for minimizing the impact on the overall system performance has to be considered.

< End new sections >

=== End Text Proposal ===