3GPP TSG-RAN Meeting #109			RP-251994

Beijing, China, September 15-18, 2025

Agenda item:	8.2.1.2

Title:	Views on 6G key performance indicators

Source:	NTT DOCOMO, INC.

Document for:	Discussion

# 1	Introduction

In RAN#108, the target values of minimum technical performance requirements (TPRs) for IMT-2030 radio interface(s) were discussed and some of the target values were agreed [1], while several TPR items have not been discussed yet. Based on the agreements in RAN#108, this contribution provides our updated views on target values for IMT-2030 TPRs.

# 2	Discussions

## 2.1	Target values for legacy TPRs from IMT-2020

In our previous contribution for RAN#108 [2], our initial views on target values for IMT-2030 minimum TPRs inherited from IMT-2020 derived by carefully reviewing the IMT-2020 self-evaluation results [3] with reasonable assumptions. Table 1 summarizes our updated views (highlighted in cyan) on target values for legacy TPRs from IMT-2020 based on agreements in RAN#108.

Table 1: Proposed target values for IMT-2030 minimum TPRs inherited from IMT-2020

| TPR | Ref: IMT-2020 requirement | Proposed target value for IMT-2030 | Comments |
| --- | --- | --- | --- |
| Peak data rate | DL: 20 Gbps UL: 10 Gbps | 1.5x of IMT-2020 | DCM view: It is preferable to define a requirement with reasonable target value. 1.5x is achieved assuming 1024QAM / 256QAM, 8 layers for DL / UL and bandwidth of 500 MHz. |
| Peak spectral efficiency | DL: 30 bps/Hz UL: 15 bps/Hz | 2x of IMT-2020 | DCM view: Assuming 1024QAM / 256QAM and 8 layers for DL / UL |
| User experienced data rate - 5th percentile user data rate | DL: 100 Mbps UL: 50 Mbps | DL: 2.5x of IMT-2020 UL: TBD | 3GPP RAN agreed to use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. Due to the different evaluation methodology, a new title can be considered to avoid the confusion with previous IMT-2020 values. DCM view: In the latest WP 5D meeting, it was agreed to change the title as “5th percentile user data rate.” 2.5x for DL is calculated based on the target value of 5th percentile user spectral efficiency (i.e., 3x of IMT-2020) assuming bandwidth of 400 MHz.. |
| 5th percentile user spectral efficiency | Indoor Hotspot – eMBB DL: 0.3 bps/Hz UL: 0.21 bps/Hz Dense Urban – eMBB DL: 0.225 bps/Hz UL: 0.15 bps/Hz Rural – eMBB DL: 0.12 bps/Hz UL: 0.045 bps/Hz | Indoor Hotspot-IC/Dense Urban-IC: DL: 3x of IMT-2020 UL: 3x of IMT-2020 Rural-IC: DL: 3x of IMT-2020 UL: 3x of IMT-2020 | Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx Form factor and other operational considerations may limit the number of receiver antennas of UE |
| Average spectral efficiency | Indoor Hotspot – eMBB DL: 9 bps/Hz/TRxP UL: 6.75 bps/Hz/TRxP Dense Urban – eMBB DL: 7.8 bps/Hz/TRxP UL: 5.4 bps/Hz/TRxP Rural – eMBB DL: 3.3 bps/Hz/TRxP UL: 1.6 bps/Hz/TRxP | Indoor Hotspot-IC/Dense Urban-IC: DL: 3x of IMT-2020 UL: 3x of IMT-2020 Rural-IC: DL: 3x of IMT-2020 UL: 3x of IMT-2020 | Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx Form factor and other operational considerations may limit the number of receiver antennas of UE |
| Area traffic capacity | 10 Mbps/m2 @ Indoor Hotspot – eMBB | 2-5x of IMT-2020 @ Indoor Hotspot – IC 0.8 Mbps/m2 @ Dense Urban – IC | DCM view: For Indoor Hotspot – IC, 12-36 TRxPs and bandwidth of 400 MHz are assumed For IMT-2030, Area traffic capacity should also be evaluated in Dense Urban – IC test environment. 0.8 Mbps/m2 is calculated based on the target value of average spectral efficiency (i.e., 3x of IMT-2020) assuming bandwidth of 400 MHz and ISD = 200 m. |
| Latency | User plane: eMBB: 4 ms URLLC: 1 ms Control plane: 20 ms | User plane: IC: 4 ms HRLLC: 1 ms Control plane: 20 ms |  |
| Connection density | 106 devices/m2 | 106 devices/km2 for massive communication |  |
| Energy efficiency | - | TBD |  |
| Reliability | 1-10-5 for 32 bytes in 1 ms | 1-10-5 | with same IMT-2020 assumption |
| Mobility | Indoor Hotspot – eMBB 1.5 bps/Hz @ 10 km/h Dense Urban – eMBB 1.12 bps/Hz @ 30 km/h Rural – eMBB 0.8 bps/Hz @ 120 km/h 0.45 bps/Hz @ 500 km/h | Indoor Hotspot-IC: 2.25 bps/Hz @ 10 km/h Dense Urban-IC: 1.68 bps/Hz @ 30 km/h Rural-IC: [0.8~1.2] bps/Hz @ 120 km/h [0.45~0.675] bps/Hz @ 500 km/h | NOTE1: Maximum velocity is up to 1000/1200km/h. Do not set spectrum efficiency target for this velocity for ITU TPRs. NOTE2: For Rural-IC, TBD on whether to include other velocities, e.g. 30km/h, 60km/h |
| Mobility interruption time | 0 ms | Same as IMT-2020 |  |
| Bandwidth | At least 100 MHz | 400 MHz | same definition as IMT-2020 |

Proposal 1: To adopt the target values in Table 1 for legacy TPRs from IMT-2020.

For energy efficiency, the evaluation in IMT-2020 was conducted based on the sleep ratio and sleep duration for network and Device. Network and Device can achieve high sleep ratio and long sleep duration in unloaded case.

At the latest ITU-R WP 5D meeting, the definition of energy efficiency for Network and Device was conducted as follows [4]:

The requirement is defined as the relative energy [savings/consumption] (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

According to [4], fully loaded IMT-2030 case is assumed as the reference case to evaluate the relative energy savings. However, in that case, the energy efficiency improvements in IMT-2030 from IMT-2020 may not be clear. Therefore, for the evaluation of energy efficiency in 3GPP, it should be discussed how to clarify the improvement from IMT-2020 to IMT-2030.

Proposal 2: For the evaluation of energy efficiency in 3GPP, it should be discussed how to clarify the improvement from IMT-2020 to IMT-2030.

## 2.2	New TPRs of IMT-2030

### 2.2.1	Positioning

ITU-R WP 5D is discussing positioning accuracy as the positioning-related TPR, which is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of a UE/device. The latest draft of the definition for positioning accuracy is provided in ITU-R WP 5D working document as follows:

The positioning accuracy is the [90%] point of the CDF of the device positioning error for [both] horizontal [and vertical] direction[s]. Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). [Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude)].

In [5], the following requirement for positioning accuracy is defined in NR Release 16:

-	Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios

-	Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios

According to the evaluation results in [5], it is possible to achieve higher horizontal accuracy compared to the above requirement. Therefore, for IMT-2030, target values of the positioning accuracy can be considered based on the evaluation results of horizontal accuracy in [5] with reasonable level of enhancement, and it should be discussed whether the evaluation of vertical accuracy is necessary or not. The target values for positioning related TPRs are proposed as shown in Table 2.

Table 2: Target values for positioning accuracy

| Test environment | Horizontal accuracy | Vertical accuracy |
| --- | --- | --- |
| Indoor | < 2 m for 90% of UEs/devices | [meter-level] |
| Outdoor | < 6 m for 90% of UEs/devices | [meter-level] |

Proposal 3: To adopt the target values in Table 2 for positioning of IMT-2030 and discuss whether the evaluation of vertical accuracy is necessary or not.

### 2.2.2	Sensing-related capabilities

For sensing-related capabilities, in latest ITU-R WP 5D meeting, the definitions of performance metrics/requirements were updated as follows [4]:

□	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present.

□	Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error.

□	Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

□	[Sensing Resolution: TBD]

For consideration of sensing-related capabilities, sensing task, test environment, and target values of the above requirements should be discussed based on [6] as starting point. It should also be discussed whether to include sensing resolution as a performance requirement. Table 4 summarizes the requirements indicated in [6].

Table 4: Target values of sensing-related capabilities

| Sensing task | Test environment | Detection probability (%) | False alarm probability (%) | Localization accuracy (m) | Velocity accuracy (m/s) | [Sensing resolution (m)] |
| --- | --- | --- | --- | --- | --- | --- |
| Object detection and tracking | Indoor Outdoor | 95~99.9 | 2~5 | 0.5~10 | 0.1~15 | [0.5~10] |
| Environment monitoring | TBD | 95~99.9 | 3 | Horizontal: 10 Vertical: 0.2 | - | [-] |
| Motion monitoring | TBD | 95 | 5 | 0.2 | 0.1 | [0.375] |

Proposal 4: For sensing-related capabilities, sensing task, test environment, and target values of the requirements should be discussed based on TS 22.137 as starting point and should also be discussed whether to include sensing resolution as a performance requirement.

### 2.2.3	AI-related capabilities

In RAN, it was agreed to define AI-related capabilities as qualitative TPR evaluated by inspection [1]. The definition of AI-related capabilities was updated as follows in the latest ITU-R WP 5D meeting [4]:

AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications, including. • RIT/SRIT facilitating AI applications i.e. “Network for AI”. • AI to improve RIT/SRIT performance, i.e. “AI for network”. The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signalling/ related to AI functionalities. This should include support for one or more of the following: – Data collection – Distributed processing – Distributed learning – AI model training – AI computing – AI model execution – AI model inference – or other AI capabilities of the RITs/SRITs (to be reported by the proponent).

For AI-related capabilities as qualitative TPR, its definition should be discussed based on the above draft as starting point.

Proposal 5: For AI-related capabilities as qualitative TPR, its definition should be discussed based on the latest draft in the ITU-R WP 5D working document as starting point.

### 2.2.4	Composite requirement

In the latest ITU-R WP 5D meeting, it was agreed that the composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users [4]. For consideration of its target value in Immersive Communication (IC) usage scenario, the evaluation results for XR application in [7] can be used as starting point of discussion. Our views on target values for composite requirement are summarized in Table 6 and 7.

Table 6: DL target values for composite requirement for IC usage scenario

| Test environment | Data rate (Mbps) | Latency (ms) | Packet success probability | Number of satisfied users (/TRxP) |
| --- | --- | --- | --- | --- |
| Dense Urban-IC | 30 | 10 | 99% | 4~11 |
|  |  | 15 |  | 5~15 |
|  | 45 | 10 |  | 2~7 |
|  |  | 15 |  | 6 |
| Indoor Hotspot-IC | 30 | 10 | 99% | 4~9 |
|  |  | 15 |  | 8~11 |
|  | 45 | 10 |  | 4~6 |
|  | 60 | 10 |  | 2~4 |

Table 7: UL target values for composite requirement for IC usage scenario

| Test environment | Data rate (Mbps) | Latency (ms) | Packet success probability | Number of satisfied users (/TRxP) |
| --- | --- | --- | --- | --- |
| Dense Urban-IC | 0.2 | 10 | 99% | 20~240 |
|  | 10 | 15 |  | 5 |
|  |  | 30 |  | 2~9 |
|  | 20 | 30 |  | 3 |
| Indoor Hotspot-IC | 0.2 | 10 | 99% | 20~240 |
|  | 10 | 30 |  | 4~9 |

Proposal 6: To adopt the target values in Table 6 and 7 for composite requirement in IC usage scenario.

### 2.2.5	Other items

In RAN#108, for the security, interoperability, sustainability and resilience, the following agreements are also reached.

- Do not define security and interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030. - Sustainability is covered by energy efficiency, do not define separate TPR for sustainability. - For resilience, whether to define a qualitative TPR or to use the submission template for IMT-2030 is TBD.

For resilience, it is also under on-going discussion in ITU-R WP5D as one of the important requirements for 6G. Having the resilience as part of TPRs of IMT-2030 would be a good message that our industry will develop such reliable and tolerant 6G system. So, we should also discuss how to capture resilience, as qualitative TPR similar to AI related capabilities or some other way.

Proposal 7: RAN should discuss how to capture resilience, as a qualitative TPR similar to AI-related capabilities or some other way.

# 3	Conclusion

In this contribution, the following proposals are provided.

Proposal 1: To adopt the target values in Table 1 for legacy TPRs from IMT-2020.

Proposal 2: For the evaluation of energy efficiency in 3GPP, it should be discussed how to clarify the improvement from IMT-2020 to IMT-2030.

Proposal 3: To adopt the target values in Table 2 for positioning of IMT-2030 and discuss whether the evaluation of vertical accuracy is necessary or not.

Proposal 4: For sensing-related capabilities, sensing task, test environment, and target values of the requirements should be discussed based on TS 22.137 as starting point and should also be discussed whether to include sensing resolution as a performance requirement.

Proposal 5: For AI-related capabilities as qualitative TPR, its definition should be discussed based on the latest draft in the ITU-R WP 5D working document as starting point.

Proposal 6: To adopt the target values in Table 6 and 7 for composite requirement in IC usage scenario.

Proposal 7: RAN should discuss how to capture resilience, as a qualitative TPR similar to AI-related capabilities or some other way.

# 4	References

[1] RP-251825, Reply LS to ITU-R WP5D =RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN

[2] RP-251770, Discussions on minimum technical performance requirements for IMT-2030 radio interface(s), NTT DOCOMO

[3] TR 37.910

[4] Annex 5.14 to Doc. 5D/792, Working document towards a preliminary draft new Report ITU-R M.[IMT-2030.TECH PERF REQ] – Minimum requirements related to technical performance for IMT-2030 radio interface(s), ITU-R WP 5D

[5] TR 38.855

[6] TS 22.137

[7] TR 38.838