3GPP TSG RAN Meeting #109	                                 RP-252088

Beijing, China, September 15-18, 2025

Source: 	CMCC

Title:	Discussion on 3GPP KPIs on sensing

Agenda item:	8.2.1.2

Document for:	Discussion & Decision

- Introduction

In RAN#108 meeting, 3GPP requirements for 6G Radio for new services are approved to be developed in SID for 6G[1]. This contribution gives proposals to define 3GPP key performance indicators (KPIs) on sensing.

- Use cases

Aim to provide sensing services for low-altitude economy, traffic surveillance, intrusion detection while also assisting communication services, detection and tracking of UAV, automotive, human are still key use cases. New cases of sensing assisting communication are covered by 6G SID and should be taken as a landmark for 6G ISAC[2].

Proposal 1: KPIs on 6G sensing should consider at least the following use cases:

- UAV

- Automotive

- Human

- Environment awareness/reconstruction to assist communication

Use cases and requirements of sensing assisting communication differ from object detections or tracking. The communication services and functions which can be assisted by sensing should be firstly identified in 6G studies. Since sensing acquires information about the physic world, those techniques relate to UE’s location and environment should be taken into consideration for sensing assisting communication.

Proposal 2: RAN should identify the communication services or functions which can be assisted by sensing, including as least the following：

- Beam management

- Channel estimation

- Power saving

- Metrics for sensing

ITU defines three TPRs for sensing-related capabilities in TPR working documents as follows[3].

4.13 Sensing-related capabilities Sensing-related capabilities are measured in terms of the following technical performance requirements: - Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. - Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. - Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error. - [Sensing Resolution: TBD]

And in TS 22.137, SA1 also defines several key metrics[4]. The definitions of the metrics are copied as follows.

5G wireless Sensing service is required to fulfil different performance requirement (e.g., accuracy, resolution, latency, etc.) based on the characteristics of one or multiple target object(s) and/or the environment to be sensed in a target sensing service area. The following set of key performance requirements is used and shown for each service scenario in Table 6.2-1. - Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e., position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude. - Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e., velocity) of the target object to its true velocity. - Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy. - Sensing Resolution describes the minimum difference in the measured magnitude of target objects (e.g., range, velocity) to be allowed to detect objects in different magnitude. - Missed detection probability describes the conditional probability of not detecting the presence of target object/environment when the target object/environment is present. This probability is denoted by the ratio of the number of events falsely identified as negative, over the total number of events with a positive state. It applies only to binary sensing results. NOTE 1: An event with a positive state refers to the presence of the characteristics of a target object or environment, including the event falsely identified as being negative and truly identified as being positive. - False alarm probability describes the conditional probability of falsely detecting the presence of target object/environment when the target object/environment is not present. This probability is denoted by the ratio of the number of events falsely identified as being positive, over the total number of events with a negative state. It applies only to binary sensing results. NOTE 2: An event with a negative state refers to the non-presence of the characteristics of a target object or environment, including the event falsely identified as being positive and truly identified as being negative. - Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface. - Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results. 5G wireless sensing performance requirements are applied to 3GPP sensing data and sensing results.

These metrics are quite aligned. However, according to the ongoing discussion in R20 ISAC for NR, the definitions of detection probability, false alarm probability, positioning accuracy and velocity accuracy are not clear for cases with multiple targets. As mentioned in the SID, they should be further translated into technical metrics that can be evaluated from a RAN perspective.

To clarify these sensing metrics, definitions which can be calculated explicitly for cases with multiple targets are needed before discussing the related requirements. And to meet the requirements from deployment, new metrics beyond ITU and SA1 should not be precluded.

For most of the use cases, there are usually more than one target, as shown in Figure 1. Since the number and the position of the targets are unknown, sensing algorithms cannot output a binary sensing result for each target. It prevents to directly apply the definitions of detection probability and false alarm probability from both ITU and SA1. The sensing algorithms can only report a set of detected targets with optional attributes including position and velocity. To evaluate the detection probability and false alarm probability, association between the set of true targets and the set of detected targets is required to decide whether a true target is correctly detected or not.

Figure 1 A case with 2 dropped target and 3 detected target output by sensing algorithm

This issue has been studied in RAN1#122 for 5G ISAC evaluation. The agreement is as follows.

Agreement For the purpose of performance metric calculation, association of the detected object(s) and the true target(s) should fulfil at least the following conditions: - One true target is associated with at most one detected object - One detected object is associated with at most one true target - The same association applies to miss detection, false alarm probability Type 2 (if defined) and positioning/velocity accuracy - Companies to report the method used for association of the detected object(s) and the true target(s) Agreement - Missed detection probability is agreed as performance metric for NR ISAC. - It is defined as the conditional probability of not detecting the presence of a target when the target is actually present in the simulation area. Pmd=n=0N-1DnMnN Where, - Dn is the number of missed targets in the drop n, i.e., the true target not associated with any detected object - Mn is the number of true targets in the drop n. - N is total number of drops with at least one target per drop Agreement - False alarm probability is agreed as performance metric for NR ISAC. - For cases without true target dropped in simulation area, False alarm probability Type 1 is computed and reported - For cases with true targets dropped in simulation area, False alarm probability Type 2 is computed and reported - Note: both False alarm probability Types are mandatory - KPI values for False alarm probability Type 1 and 2 can be discussed separately. Definitions: - False alarm probability Type 1 (no target dropped in simulation area): An object is detected when there is no target present in simulation area is considered a false alarm. Pf1=n=0N-1QnN Where, - Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0. - N is the total number of drops without targets in the simulation area. - False alarm probability Type 2 (targets dropped in simulation area): An object is detected but not associated with any true targets in the simulation area is considered as a false alarm. Pf2=0≤n<NMn'≠0Dn'Mn'K Where, - Dn' is the number of detected objects but not associated with any true targets in the drop n. - Mn' is the total number of detected objects in the drop n. - FFS: - Option 1: K is number of drops (N) - Option 2: K is number of drops with at least one detected object Note: the number of targets should be reported by companies when providing False alarm probability Type 2 Agreement - Velocity accuracy is agreed as performance metric for NR ISAC. - Velocity accuracy is defined as the absolute value of the difference between the estimated velocity and the corresponding true velocity of a sensing target. - For single TRP monostatic sensing, - The radial velocity accuracy can be estimated - The true radial velocity is the projection of true velocity on the direction from TRP to target for TRP monostatic. - The true velocity accuracy can be estimated.

According to the agreement, both miss detection probability and false alarm probability are defined based on the association.

Meanwhile, in practice, sensing performance is not only determined by system design and algorithm, but the number of targets as well. For example, detection algorithms with adaptive threshold suffers from nearby target. Estimation of the threshold is significantly influenced by nearby targets or their sidelobes. The assumption of additive white Gaussian noise is broken when power from other targets falls into the region for threshold estimation. It indicates that even if the requirements of performance can be achieved when observing a small number of targets, the real performance in deployment can be much worse, which is unacceptable from operator’s position.

To address the issue, sensing target density is defined in TR 22.870 as follows[5]:

Sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing area.

From RAN perspective, this definition should also be translated into technical metrics which can be evaluated explicitly.

We recommend defining sensing capacity as a metric for sensing, which evaluates the ability to sense multiple targets with acceptable performance. And it is defined as the maximum number of the targets per sector when sensing results of all targets in observation zone fulfil QoS requirements with 95% probability. The QoS requirements are defined by other KPIs (except sensing capacity itself) on sensing. Note that, according to this definition, non-compliance of sensing KPIs with a given low probability is allowed.

For better understanding, a simplified drop-based detection capacity simulation of 1-D CA-CFAR detection is presented in Figure 4. Detection probability of 100% is selected as the only KPI for simplicity. And the sensing capacity is about 4 per km according to the curve.

| (a) Range profile of a single drop with 3 targets | (b) Probability of detecting all targets |
| --- | --- |

Figure 1. A simplified drop-based simulation to figure out sensing capacity

Therefore, we suggest the following metrics for sensing.

Table 1 Metrics for sensing

| Metric | Definition |
| --- | --- |
| Miss detection probability | Miss detection probability is the probability that a true target is not associated with a detected target. From the aspect of evaluation, it is defined as Pmd=n=0N-1DnMnN Where, - Dn is the number of missed targets in the drop n, i.e., the true target not associated with any detected object - Mn is the number of true targets in the drop n. - N is total number of drops with at least one target per drop |
| False alarm probability | Definition Type 1 (no target dropped in simulation area): False alarm probability is defined as the probability that an object is detected when there is no target present in simulation area is considered a false alarm. From the aspect of evaluation, it is defined as Pf1=n=0N-1QnN Where, - Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0. - N is the total number of drops without targets in the simulation area. Definition Type 2 (targets dropped in simulation area): False alarm probability is defined as the probability that an object is detected but not associated with any true targets in the simulation area is considered as a false alarm. From the aspect of evaluation, it is defined as Pf2=0≤n<NMn'≠0Dn'Mn'K Where, - Dn' is the number of detected objects but not associated with any true targets in the drop n. - Mn' is the total number of detected objects in the drop n. - K is number of drops (N) |
| Horizontal/Vertical Positioning Accuracy | Horizontal/vertical positioning accuracy is defined as the 95th percentile point of the cumulative distribution function (CDF) of the horizontal/vertical position estimation error. From the aspect of evaluation, the horizontal/vertical position estimation error is the norm of the difference between the estimated horizontal/vertical position and the corresponding true position of a sensing target. |
| Velocity Accuracy | Velocity accuracy is defined as the 95th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. From the aspect of evaluation, the velocity estimation error is the norm of the difference between the estimated velocity and the corresponding true velocity of a sensing target. |
| Sensing Capacity Note 1 | Sensing capacity is defined as the maximum number of the targets per sector when sensing results of all targets in observation zone fulfil QoS requirements with 95% probability. The QoS requirements are defined by other KPIs (except sensing capacity itself) on sensing. |
| Max sensing service latency | Max sensing service latency is the time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface. |
| Refreshing rate | Refreshing rate is the rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results. |
| Note 1: An intuitive evaluation methodology for sensing capacity can be found in Annex A. |  |

Since the metrics are all defined by statistic with a given threshold, confidence level is not needed. If a different confidence level is needed for some specific use cases, adjusting the requirement of accuracy / capacity can achieve a same result.

Proposal 3: Support the metrics listed in Table 1 for 6G sensing.

For the cases of sensing assisting communication, which are promising in 6G, all the metrics discussed above are incompatible. Thus, new metrics for sensing assisting communication need to be further studied in 6G.

Proposal 4: Support to further study the metrics for sensing assisting communication in 6G.

- KPIs on sensing

In TS 22.137, KPIs related to different use cases are defined for 5G, which can be taken as a starting point for 6G KPIs on sensing[4]. Meanwhile, requirements related to UAV supervision, safe & economic UAV transportation and road digitalization are also studied in TR 22.870 for 6G sensing[5]. These requirements can also be found in Annex B for convenience.

Taking TR 22.870 as a reference while considering our deployment requirements, we suggest the KPIs on sensing as follows.

Table 2 KPIs on sensing

| Use case | Positioning Accuracy |  | Velocity Accuracy [m/s] | Max sensing service latency [ms] | Refreshing rate [Hz] | Sensing Capacity [per sector] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal [m] | Vertical [m] |  |  |  |  |  |  |
| UAV detection | 10 | 10 | 1 | 1000 | 1-10 | 100 | 1 | 1 |
| UAV tracking | 1 | 1 | 1 | 100-1000 | 1-10 | 100 | 0.1-1 | 1 |
| Automotive detection and tracking | 1 | N/A | 1 | 1000-5000 | 10 | 100 | 1 | 3 |
| Human detection | 1 | N/A | N/A | 1000 -5000 | 10 | 100 | 5 | 2 |
| Note 1: Metrics for tracking can be different with detection. And metrics for tracking should be evaluated independently. |  |  |  |  |  |  |  |  |

Proposal 5: Support the KPIs listed in Table 2 for 6G sensing.

- Conclusion

In this contribution, we provide our view on 6G sensing use cases and related KPIs.

Proposal 1: KPIs on 6G sensing should consider at least the following use cases:

- UAV

- Automotive

- Human

- Environment awareness/reconstruction to assist communication

Proposal 2: RAN should identify the communication services or functions which can be assisted by sensing, including as least the following：

- Beam management

- Channel estimation

- Power saving

Proposal 3: Support the metrics listed in Table 1 for 3GPP KPI on 6G sensing.

Proposal 4: Support to further study the metrics for sensing assisting communication in 6G.

Proposal 5: Support the KPIs listed in Table 2 for 6G sensing.

- Reference

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, 3GPP TSG RAN Meeting #108Prague, Czech Republic, June 9-13, 2025.

- RP-251881, New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108Prague, Czech Republic, June 9-13, 2025.

- Minimum requirements related to technical performance for IMT-2030 radio interface(s), 2 July 2025

- 3GPP TR 22.137, 2024

- 3GPP TR 22.870, 2025

- Annex A (informative)

Evaluation methodology for sensing capacity

| Step 1 | Initialize evaluation parameters. Decide the number of targets dropped. |
| --- | --- |
| Step 2 | Implement Mont-Carlo system-level simulations with the given number of targets. Generate sensing transmit signal. Obtain sensing received signal and perform signal processing to estimate the interested parameters of all targets. |
| Step 3 | Count the cases that all targets fulfil QoS requirements (depending on use case). Calculate the probability that all targets fulfil QoS requirements. |
| Step 4 | If necessary, update the number of targets dropped and repeat Step.1 to Step.3, until the probability that all targets fulfil QoS requirements equalling to 95%. |

- Annex B (informative)

Requirements from TR 22.870

Table 7.4.6-1:	Performance requirements of sensing results for low-altitude UAV supervision

| Scenario |  | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate (Hz) | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s] |  |  |  |  |
| Low-altitude UAV supervision | UAV intrusion detection | Outdoor | ≥95 | ≤10 | ≤10 | N/A | N/A | [10] | [≥5] | [≤1000] | [≤1] | [≤5] | [≤5] |
|  | UAV trajectory tracking | Outdoor | ≥90 | 1-2 | 1-2 | 3-5 | 3-5 | N/A | N/A | 100~1000 | ≤1 | ≤5 | ≤5 |

Table 7.6.6-1:  Performance requirements for road digitalization

| Scenario | Service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Sensing target density （note 3） | Sensing target Velocity | Max sensing service latency [s] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |
| Road digitalization | Outdoor | 95 | 0.5 (vehicle) 1（pedestrian） | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | ≤ 750 vehicles Per 1000 m x 24 m ≤100 pedestrian per 1000 m x 24 m (note 1) | Static/moving, up to 5 km/h | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
|  | Outdoor | 95 | 3 | N/A | 3 | N/A | 1 | 1 m/s | ≤85 vehicles per 1000 m x 24 m (note 2) | Moving, up to 120 km/h | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
| NOTE 1: One crossroad area is approximated by 1000m dual three-lane carriageway (24 m), by assuming 250 m per direction at the crossroad. NOTE 2: Based on an assumption of dual three-lane carriageway (1000 m x 24 m). NOTE 3: The definition of sensing target density is described in Clause 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.17.6-1:	Performance requirements for Safe and Economic UAV transport

| Scenario | Sensing service availability [%] | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing Resolution |  | Max sensing service latency [ms] | Refreshing Rate [s] | Missed Detection [%] | False Alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Safe and economic UAV transport | 99.999 | 99.99 | ≤ 50 | 50 | 5 | 5 | 10 | 0.5 | ≤ 100 | 0.1 - 1 | 0.1% (notes 2, 3) | 1% (note 2) |
| NOTE 1: The listed positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the UAV and validate the reported location of the UAV itself. NOTE 2: Safety impact of missed detection is substantially higher compared to the false alarm, here fore the Missed Detection is assigned a more stringent value compared to the False Alarm. NOTE 3: To enable safe operation of BVLOS UAV flights in mixed airspace (manned & unmanned) and in high risk areas (e.g. above cities) the value for missed detection of 0.1% corresponds to the lower value of the missed detection range specified in Table 6.2-1 of TS 22.137 [6] for sensing service category 2. |  |  |  |  |  |  |  |  |  |  |  |  |