3GPP TSG-RAN WG Meeting #109       	                      			RP-252092

Beijing, China, 15th-18th Sep 2025

Agenda item:	8.2.1.3

Source:	CMCC

Title:	Views on architecture of 6G RAN for AI

WID/SID:	FS_6G_RAN_Scen_Req

Document for:	Discussion

Background

In the 6G Radio SID [1], the following objectives about AI are captured, and it can be observed that AI/ML is a crucial component of 6G:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] (6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

Based on the 6GR SID, NW for AI is assumed to be covered by new services. In this contribution, we will provide our views on the 6G RAN architecture to support RAN for AI.

Potential 6G RAN architecture to support RAN for AI

According to the discussion in [2], some requirements of 6G RAN for AI new services could be observed as following:

- High computing demand to guarantee the quality of AI services

- Low-latency requirement to support the real-time communication

- High security requirement on the exposed AI data and RAN related information

To fulfill the above requirements, following two solutions can be considered:

- Solution 1: Application server provides AI function, e.g. computing capability

- Solution 2: RAN provides AI function, e.g. computing capability

Figure 2-1: AI function in application server or in RAN

- For solution 1: The computing capability can be controlled by the application server, but application server cannot directly control network connections. Also, the transmission latency may be relatively high, which is difficult to meet the demands of real-time AI services. Moreover, the computing and communication are optimized independently, making it challenging to achieve collaborative optimization.

- For solution 2: The computing capability is provided by RAN AI function, and the transmission latency will be lower than to application server. As RAN can provide directly and real-time control to the radio link connections and computing resources, the collaboration optimization of communication, computing and AI capabilities can be achieved, which also guarantee the quality of AI services.

Observation 1: Compared with computing capability provided by application server, RAN AI function can meet the low latency requirement of real-time AI services, and provides more benefits in jointly optimization of communication, computing and AI capabilities.

Based on the above analysis, RAN AI function is more beneficial to provide high computing capability to real-time AI services. Then, the deployment options for RAN AI function can be further discussed:

- Option 1: RAN AI function deployed in BS

- Option 2: RAN AI function deployed independently

We provide the analysis for these two options from different aspects in the following table:

Table 2-1: The comparison between Option 1 and Option 2

|  | Option 1: RAN AI function deployed in BS | Option 2: RAN AI function deployed independently |
| --- | --- | --- |
| Multi-vendor interworking | Base stations from different vendors may use distinct AI architectures or hardware platforms (e.g. CPU/GPU), which forces UE to adapt to multiple AI architectures from different vendors. This can lead to the compatibility issue when UE handover between different vendors’ BSs. | A unified AI architecture will be supported. This means that UE only needs to adapt a single AI architecture, and no need to adapt to multiple AI architectures from different base station vendors. There is no compatibility issue during handover. |
| Cost of computing | Each BS is equipped with AI computing resource, which is costly, such as n * the price of a single computing board (where n is the number of BS in RAN) | The computing resources of a centralized AI function can be shared among multiple BSs, which is cost-effective. For example: 8 base states share a single computing board |
| Efficiency of collaboration | The design of inter-BS collaborative algorithms is complex, as it needs to consider Cnk (where n is the number of BS in RAN, and k is the number of collaborating BS) different data or model collaboration methods. | The centralized AI management and control algorithms are simple, only requiring unified global data collection, analysis, and decision-making. |
| Overhead of data transmission | Inter-BS collaboration would lead to repeated data collection and signaling overhead for different use cases, wherein the total data volume = a single-link data volume * Cn2. | Centralized AI management and control significantly reduce the transmission overhead of signaling/data, wherein the total data volume = a single-link data volume * n. |
| Decoupling AI module from legacy communication module | The AI module cannot be decoupled with legacy communication module, thus cannot be updated independently, which is not conducive for future evolution. | AI module is decoupled with legacy communication module, the AI module can be flexibly and independently upgraded with the development of AI technology. |
| Computing capability | Restricted by the space, power consumption, heat dissipation and cost, the computing capability of a single BS is limited. Only lightweight or simplified versions of AI models can be deployed. | High-performance server clusters (CPU/GPU/TPU/dedicated AI accelerators) can be utilized to provide computing capability far exceeding that of single BS. This enables the operation of more complex and high-precision AI models. |
| Security guarantee | If BS directly exposes RAN-related information or AI data to 3rd parties, it may face security risks such as being attacked or network deployments being disclosed. | The security risks such as being attacked or network deployments being disclosed can be avoided by centralized RAN AI function. Exposure of AI-related information (e.g. RAN related information, AI data) can be supported. |

Observation 2: Compared with RAN AI function deployed in BS, RAN AI function deployed independently is beneficial in the following aspects:

- Unified AI architecture to avoid compatibility issue from different BS vendors

- Computing resources can be shared among multiple BSs, which is cost-effective

- Unified data collection and management can be achieved

- Signaling/data transmission overhead is reduced

- Decoupling with communication module, AI module can be upgraded independently and flexibly

- Supporting more complex and high-precision AI models, as the computing capability is far exceeding that of single BS

- Security guaranteed by RAN AI function independently

Given the benefits identified in observation 2 for deploying AI function in RAN independently, it seems reasonable to introduce a centralized AI node to support AI function. The centralized AI node can be utilized to support real-time control to the radio link connections and computing resources, for example, the computing of AI applications/services can be offloaded to RAN efficiently to meet the requirements of real-time AI services.

Proposal 1: It is proposed to introduce a centralized AI node to support 6G RAN for AI new service, e.g. computing of AI applications/services can be offloaded to RAN to meet the requirements of real-time AI services.

Figure 2-2. Overall architecture of centralized AI node in 6G RAN

From our perspective, the centralized AI node will support AI functions including AI computing resource management (e.g. allocation, sharing), online model training, model inference, and service exposure (e.g. expose AI-related information to the third party). Also, the centralized AI node will be able to serve multiple base stations, which can significantly improve collaboration efficiency, reduce complexity and latency, as well as reduce deployment costs compared to AI function distributed deployed in each BS.

Proposal 2: The centralized AI node, which serves multiple BSs, will support various AI functions, including AI computing resource management, online model training, model inference, and service exposure.

Furthermore, due to the introduction of a centralized AI node in 6G RAN, a new interface between 6G BS and the centralized AI node will be needed to transfer the AI-related information, such as AI data, AI/ML model, etc.

Proposal 3: Introduce a new interface between centralized AI node and 6G BS to transfer AI-related information, e.g. AI data, AI/ML model.

Conclusions

In this contribution, the following proposals are provided.

Observation 1: Compared with computing capability provided by application server, RAN AI function can meet the low latency requirement of real-time AI services, and provides more benefits in jointly optimization of communication, computing and AI capabilities.

Observation 2: Compared with RAN AI function deployed in BS, RAN AI function deployed independently is beneficial in the following aspects:

- Unified AI architecture to avoid compatibility issue from different BS vendors

- Computing resources can be shared among multiple BSs, which is cost-effective

- Unified data collection and management can be achieved

- Signaling/data transmission overhead is reduced

- Decoupling with communication module, AI module can be upgraded independently and flexibly

- Supporting more complex and high-precision AI models, as the computing capability is far exceeding that of single BS

- Security guaranteed by RAN AI function independently

Proposal 1: It is proposed to introduce a centralized AI node to support 6G RAN for AI new service, e.g. computing of AI applications/services can be offloaded to RAN to meet the requirements of real-time AI services.

Proposal 2: The centralized AI node, which serves multiple BSs, will support various AI functions, including AI computing resource management, online model training, model inference, and service exposure.

Proposal 3: Introduce a new interface between centralized AI node and 6G BS to transfer AI-related information, e.g. AI data, AI/ML model.

References

- RP-251881, “New SID: Study on 6G Radio”, RAN#108, June 2025.

- RP-252099, Requirements of 6G RAN for AI Services, CMCC

Annex: TP to 38.914

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

#### 5.2.X	AI/ML architecture

To fulfill the high computing, low-latency and high security requirements of 6G RAN for AI new service, a centralized AI node, which serves multiple BSs, can be introduced to support various AI functions, including AI computing resource management (e.g. allocation, sharing), online model training, model inference, and service exposure (e.g. expose the AI data to the third party).

With the centralized node, the collaboration efficiency would be significantly improved, as well as the complexity and deployment costs would be significantly reduced.

Figure X. Overall architecture of centralized AI node in 6G RAN