3GPP TSG RAN Plenary Meeting #109						   RP-252118

Beijing, China, September 15-18, 2025

Source:	    Tejas Networks, CEWiT, Indian Institute of Technology Madras (IITM)

Title:		    Discussion on 6G Deployment Scenarios

Agenda item:	    8.2.1.1 – Deployment Scenarios

Document for:   Discussion and Decision

Introduction

The RAN technical specification group started a study “Study on 6G Scenarios and requirements” in RAN#107 with an objective to provide guidance to technical work in RAN WGs and an input for ITU-R to develop minimum technical performance requirements for IMT-2030. The study involves identifying the practical 6G deployment scenarios along with their attributes and the performance requirements. The study details are being updated in the running technical report TR 38.914 of Release 20 [1]. In this contribution, we present our proposals to add new deployment scenarios to 6GR evaluation studies.

## Background

In the current version of TR 38.914, the rural deployment scenario for multi cell deployments, has the following agreed configurations.

| Carrier Frequency | Aggr system Bandwidth | Layout | ISD | UE distribution and UE speed |
| --- | --- | --- | --- | --- |
| 700 MHz | Up to [60] MHz | Single layer: Hex Grid | 1732m, 5000m and [7500m] | [15% outdoor vehicles (120km/h), 15% outdoor (3 km/h) and 70% indoor (3 km/h), or 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP |
| 2 GHz | Up to [120] MHz |  | 5000m |  |
| 4 GHz | Up to [200] MHz |  | 1732m |  |
| 7 GHz | Up to [400] MHz |  |  |  |

In this contribution, for multi cell rural deployments, we suggest to agree to the ISD of 7500m with 700 MHz carrier frequency.

For the extreme rural scenarios, an Isolated Macro cell was considered as a deployment scenario with a cell range of 100 Km [4]. However, the deployment of this scenario is not practical in the Indian context. To this end, we would like to propose an isolated macro cell deployment with cell radius in the range of 3Km to 8Km.

## Population distribution in Rural India

The population and geographical distribution of rural villages in India, also true of most of the Asian countries, is not uniform as shown in Figure 1. In India, more than 50% of the population lives in rural parts of the country. A group of villages with an approximate population of 1K to 2K are administered from a village with higher population, approximately 5K to 10K. These villages are not located continuously, but instead, are separated by a distance of approximately 2KM to 5KM. To provide telecom services to the people living in these geographical scenarios, it is important for standardization bodies such as 3GPP and ITU to consider this geographical distribution of rural population as one of the deployment scenarios for evaluation of 6G Radio Interface Technology(ies) RIT/SRIT.

Figure 1: A general representation of cellular communication in rural parts of India. A moderately populated (5K to 10K) geographical area is surrounded by a few less populated (1K to 2K) areas.

Deployment Scenarios

In this contribution, we provide our recommendations on adding new deployment scenarios for 6G studies. The two new deployment scenarios are 1) Single Cell with Large Coverage (as an alternate to extreme rural deployment scenario with low population density) and 2) Indoor Factory.

## Single cell with large coverage

This deployment scenario is intended for single macro cell deployment in a very low density rural areas where the primary mode of communication is through terrestrial.  The following figure shows a possible layout for the single cell deployment with 3 sectors.

Cell radius

Cell Radius = 3km to 8km

Some of the attributes of this deployment scenario are in the following table, Table 2.1.

Table 2.1: Attributes for eMBB Extreme Rural (Single Cell with Large Coverage)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 600 MHz |
| System Bandwidth | 20 MHz (DL+UL) |
| Layout | Single cell layout [Isolated Macro cell] |
| Cell radius | 3km to 8km |
| User density and UE speed | 10 users per cell (distributed along cell edge) 40% Outdoor pedestrian: 3km/h 20% Outdoor in cars: 30km/h or 60km/h 40% Indoor: 3km/h |
| UE height | 1.5m |
| Traffic model | Full buffer |
| KPI | DL Coverage, UL Coverage, 5th percentile spectral efficiency, Average spectral efficiency |

Table 2.2: Attributes for FWA Extreme Rural (Single Cell with Large Coverage)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 7GHz |
| System Bandwidth | 50 MHz (TDD) |
| Layout | Single cell layout [Isolated Macro cell] |
| Cell radius | 3km to 8km |
| User density and UE speed | 10 users per cell (distributed along cell edge) 40% Outdoor pedestrian: 3km/h 20% Outdoor in cars: 30km/h or 60km/h 40% Indoor: 3km/h |
| UE height | 5m |
| Traffic model | Full buffer |
| KPI | DL Coverage, UL Coverage, 5th percentile spectral efficiency, Average spectral efficiency |

The performance metric Coverage is defined as the cell edge distance of a single cell through link budget analysis. Coverage is one of the performance requirements for this deployment scenario.

Single Cell Simulation Results

We performed link level simulations for 5G NR PUSCH with the following parameters.

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 600 MHz |
| Bandwidth | 20 MHz |
| Sub carrier spacing | 30 kHz |
| Number of concurrent users | 5 |
| Number of resource blocks per user | 10 |
| Code Rate | 379/1024 |
| Modulation | QPSK |
| Speeds | 0 kph and 3kph |

From the link level simulations, we obtained the SNR at which a target BLER of 10% was achieved. We also calculated the corresponding throughput. The BLER vs SNR and Throughput vs SNR curves for 0kph and 3kph speeds are as shown below.

Figure 1: BLER vs SNR and  Throughput vs SNR for 0 kmph speed

Figure 2: BLER vs SNR and  Throughput vs SNR for 3 kmph speed

The target SNR values are used in the link budget calculations to obtain the Coverage (cell edge distance). The link budget sheet is attached to this contribution.

The following are the Coverage results for a target throughput of ~ 2 Mbps and 10% BLER as shown below:

| UE Type | Maximum Link Distance (m) |
| --- | --- |
| Handheld UE (0 kmph) | 3966 |
| CPE (0 kmph) | 13571 |
| Handheld UE (3 kmph) | 2926 |

## Indoor Factory (InF)

In 5G, Indoor Factory (InF) deployment scenario is extensively used for evaluation of various functionalities. Going ahead, the InF deployment scenario is equally important for certain usage scenarios e.g. Immersive Communication, HRLLC and Integrated Sensing and Communication (ISAC). The InF scenario focuses on factory halls of varying sizes and with varying levels of density of "clutter", e.g. machinery, assembly lines, storage shelves, etc. [2]. The key characteristics of this deployment scenario are clutter type, Clutter density, height of Tx and Rx and LOS/NLOS. The InF scenario is classified further into InF-SL, InF-DL, InF-SH, InF-DH and InF-HH based on a) density of the clutter (S-sparse and D-dense) and b) the height of Tx and Rx (L- low and H-high).

Figure 2.1: Indoor factory deployment scenario with 18 base stations, with a floor dimension of 300m by 150m.

Table 2.2 provides the possible attributes of InF-SL, InF-DL, InF-SH, InF-DH and InF-HH scenarios.

Table 2.2: Attributes for Indoor Factory

| Parameters |  | InF-SL (sparse clutter, low BS) | InF-DL (dense clutter, low BS) | InF-SH (sparse clutter, high BS) | InF-DH (dense clutter, high BS) | InF-HH (high Tx, high Rx) |
| --- | --- | --- | --- | --- | --- | --- |
| Layout | Room size | Rectangular: 45000 m2 i.e. 300 x 150 m |  |  |  |  |
|  | Ceiling height | 5-25 m | 5-15 m | 5-25 m | 5-15 m | 5-25 m |
| ISD |  | 50m |  |  |  |  |
|  | Effective clutter height hc | <10m |  |  |  |  |
|  | External wall and ceiling type | Concrete or metal walls and ceiling with metal-coated windows |  |  |  |  |
| Clutter type |  | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Any |
| Typical clutter size, dclutter |  | 10 m | 2 m | 10 m | 2 m | Any |
| Clutter density r (percentage of surface area occupied by clutter) |  | Low clutter density (<40%) | High clutter density (≥40%) | Low clutter density (<40%) | High clutter density (≥40%) | Any |
| BS antenna height |  | Clutter-embedded, i.e. the BS antenna height is below the average clutter height |  | Above clutter |  | Above clutter |
| UT location | LOS/NLOS | LOS and NLOS |  |  |  | 100% LOS |
|  | Height | Clutter-embedded |  |  |  | Above clutter |
| User distribution and UE speed |  | 100% Indoor, 3km/h |  |  |  |  |
| Service profile |  | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |  |  |  |
| KPI |  | UE positioning, Sensing related capability, Reliability |  |  |  |  |

Conclusion

Proposal 1: Consider the ISD of at least 7500 m as one of the configurations for the multi cell rural deployment scenario with 700 MHz carrier frequency.

Proposal 2: Consider 600 MHz as a candidate carrier frequency for extreme rural deployment scenario for long distance coverage with low density areas.

Proposal 3: Define a single cell layout with a cell radius of 3km-8km with low density and moderate mobility (less than 100 Kmph) as deployment scenario under extreme rural.

Proposal 4: Add Indoor Factory (InF) as one of the deployment scenarios for 6G.

References

1. TR 38.914, Study on 6G Scenarios and Requirements, Release 20.

2. TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

3. Report ITU-R M.2142-0, Guidelines for evaluation of radio interface technologies for IMT-2020.

4. TR 38.913, Study on scenarios and requirements for next generation access technologies, 2017.