3GPP TSG-RAN Meeting #109								       RP-252123

Beijing, China, 15 - 18 September, 2025

Agenda Item:		8.2.1.1

Source:		Deutsche Telekom, Vodafone, Orange, Telecom Italia, Turkcell, Spark NZ, Odido, BT, Bouygues Telecom, Telefonica, KPN, Rakuten Mobile, CK Hutchison, Telenor

Title:		pCR on Deployment Scenarios for TR 38.914

Document for:		Discussion and decision

## 1	Introduction

The RAN P SI on “Study on 6G scenarios and requirements” has been approved in December 2024 [1].

At the RAN#108 6G deployment scenarios where discussed and agreeable text captured into the newly created TR 38.914 [2]. A multiple operator contribution in [3] defined key deployment scenarios and the relevant configuration in terms of bands deployed, multi-band support deployed, ISD, BS and UE antenna configurations, User Distribution and speed etc.

With this contribution the cosigning operators defined their key parameters for their key deployment scenarios for 6G for inclusion into the TR 38.914.

## 2	Text Proposal

*** START OF CHANGES ***

# 4.	Deployment scenarios

Editor note: Sensing related aspects will be included in section 4

High-level descriptions on deployment scenarios including carrier frequency, aggregated system bandwidth, network layout / ISD/ Orbit, BS / UE antenna elements, UE distribution / speed and service profile are proposed in this TR. It is assumed that more detailed attributes and simulation parameters, for example, the channel model, BS / UE Tx power, number of antenna ports, etc. should be defined in the new RAT study item.

## 4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 4 GHz Around 7 GHz Around 4GHz+Around 7GHz Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 4 GHz: Up to [200] MHz (DL+UL) NOTE3 Around 7 GHz: Up to [400] MHz (DL+UL) NOTE3 Around 30 GHz: [1]GHz (DL+UL) NOTE3 |
| Layout | Single layer: - Indoor floor (Open office) |
| ISD | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements | TBD |
| UE antenna elements | TBD |
| User distribution and UE speed | 100% Indoor, 3km/h, [10] users per TRxP NOTE4 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

## 4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 MHz Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100]m All micro TRxPs are all outdoor |
| BS antenna elements | TBD |
| UE antenna elements | TBD |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP [10] users per TRxP with single-layer only 80% indoor (3km/h), 20% outdoor (30km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

## 4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting stationary and nomadic devices as well as high speed vehicles (cars, trains). This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.3.

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7GHz (ISD 1) Around 700MHz + Around 7GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7GHz: Up to [400] MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| BS antenna elements | Around 700 MHz: 4T4R with passive antenna Around 2 GHz: 4T4R with passive antenna |
| UE antenna elements | Around 700 MHz: 1T2R Around 2 GHz: 2T4R |
| User distribution and UE speed | 10% outdoor vehicles (120km/h), 20% outdoor (3 km/h) and 70% indoor (3 km/h) 10/2 users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450MHz – 960MHz identified for WRC-15 are currently being considered and around 700MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For UL User Experienced Data rate it is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes.

NOTE4: 	10 users per TRxP is the baseline with full buffer traffic. For evaluation of UL user experienced data rate,  the number of users per TRxP can be reduced to 2

## 4.4	Urban macro

The urban macro deployment scenario focuses on large cells providing continuous and ubiquitous coverage; there is no requirement that the continuous coverage is provided on all bands of a multi-band BS. The key characteristics of this scenario are continuous and ubiquitous coverage for stationary and nomadic devices in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements | Around 700 MHz: 4T4R Around 2 GHz: 4T4R with passive (baseline) and 32T32R with up to 128 active antenna elements (optional) Around 4 GHz: 32T32R with up to 192 active antenna elements Around 7 GHz: up to 256T256R with up to 1024 active antenna elements |
| UE antenna elements | Around 700 MHz: 1T2R Around 2 GHz: 2T4R Around 4 GHz: 2T4R Around 7 GHz: up to 4T8R |
| User distribution and UE speed | 10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10/2 users per TRxP NOTE4 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 24.25 GHz – 52.6 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range. A range of bands from 1427 – 2690MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For UL User Experienced Data rate it is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes.

NOTE4:	10 users per TRxP is the baseline with full buffer traffic. 20 users per TRxP with full buffer traffic is not precluded. For evaluation of UL user experienced data rate  the number of users per TRxP can be reduced to 2.

## 4.5	Sub-Urban macro

The sub urban deployment scenario focuses on providing large and continuous coverage; there is no requirement that the continuous coverage is provided on all bands of a multi-band BS. The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 700 MHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| System bandwidth | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) |
| Layout | Single layer - Hex. Grid [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| TRP antenna elements | Around 700 MHz: 4T4R antenna Around 2 GHz: 4T4R (for all 4 bands) or 32T32R with up to 128 active antenna elements (for only the “around 700 MHz and around 2 GHz” scenario) Around 4 GHz: 32T32R with up to 192 active antenna elements Around 7 GHz: Up to 256T256R with up to 1024 active antenna elements |
| UE antenna elements | Around 700 MHz: 1T2R Around 2 GHz: 2T4R antenna Around 4 GHz: 2T4R antenna Around 7 GHz: Up to 4T8R antenna |
| User distribution and UE speed | 10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h 10/2 users per TRxP NOTE4 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 1427 – 2690MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of ...

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For UL User Experienced Data rate it is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes.

NOTE4:	10 users per TRxP is the baseline with full buffer traffic. 20 users per TRxP with full buffer traffic is not precluded. For evaluation of UL user experienced data rate the number of users per TRxP can be reduced to 2.

## 4.6	High speed

Editor note: Highway scenario will be considered and whether highway scenario is merged to section 4.6 can be further discussed

Editor note: airborne will not be captured in section 4.6.

## 4.7	Extreme long distance coverage in low density area

The extreme Long Range deployment scenario is defined to allow for the provision of services for very large areas with low density of users whether they are humans and machines (e.g. wilderness, river or water, remote village or island areas, etc). The key characteristics of this scenario are Macro cells with very large area coverage supporting [at least] basic data speeds and voice services, with low to moderate user throughput and low user density.

Table 4.7: Attributes for extreme rural

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz |
| System Bandwidth | [20] MHz (DL+UL) |
| Layout |  |
| Cell range |  |
| User density and UE speed |  |
| Traffic model |  |

## 4.8	Urban coverage for massive connection

Editor note: focusing on MTC

## 4.9	Air-to-Ground Scenario

The commercial Air to Ground deployment scenario is defined to allow for the provision of services for commercial aircraft to enable both humans and machines aboard the aircraft to initiate and receive mobile services. It is not for the establishment of airborne based base stations.

The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput that are optimized for high altitude users that are travelling at very high speeds. The commercial airlines aircrafts are likely equipped with an aggregation point (e.g. Relay).

Some of the characteristics of this deployment scenario are listed in Table 4.9.

Table 4.9: Attributes for commercial Air to Ground Scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro + relay: for BS to relay: [4] GHz |
| System Bandwidth | [100] MHz (DL+UL) |
| Layout |  |
| Cell range |  |
| User density and UE speed |  |
| Traffic model |  |

## 4.10	Non-Terrestrial Network

Editor note: orbit type and payload type will be captured in section 4.10

## 4.11	[Urban grid]

## 4.12	[Highway Scenario]

*** END CHANGES ***

## 3	Proposal

It is proposed by the cosigning operators to agree the above pCR to update the relevant sections in the new TR 38.914 v0.1.0

## References

[1] 	RP-243327 “Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, RAN#107

[2]	RP-251873 “3GPP TR 38.914 V0.1.0”, 3GPP RAN#108

[3] 	RP-251684 “Operator deployment scenarios for 6G”, Deutsche Telekom, Vodafone, Orange, Telecom Italia, Telefonica, Spark NZ, Odido, Turkcell, Telstra, Telenor, Telus, Jio Platforms, KPN, Boost Mobile Network, 3GPP RAN#108