3GPP TSG RAN Meeting #109								RP-252167

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.2

Source:	Xiaomi

Title:	Discussion on 6G Key performance indicators

Document for:	Discussion

- Introduction

In RAN#108, most of the legacy TPR have been discussed and the values have been agreed. In addition, the agreements have been captured in TR 38.914[1]. However, there are still some remaining issues on the value of legacy TPR. In addition, the detailed definition and values for performance indicator such as energy efficiency and sensing are still missing.

In this contribution, the remaining issues of legacy TPRs are discussed. We also give our views on performance indicators of energy efficiency and sensing.

- Remaining issues of legacy TPR

  - Peak spectral efficiency & Peak data rate

The peak spectral efficiency is calculated according to parameters including maximum number of layers, maximum modulation order, the system overhead, etc. In 3GPP NR evaluation [2], the DL peak spectrum efficiency is evaluated assuming 8 layers and 256QAM, while the UL peak spectrum efficiency is evaluated assuming 4 layers and 256QAM.

In RAN1#122 meeting, the following agreements were made:

Agreement

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

Therefore, for 6GR DL, 1024 QAM can be assumed for DL peak spectral efficiency evaluation; while 256QAM shall still be assumed for 6GR UL peak spectral efficiency calculation.

Observation 1: 1024QAM/256QAM shall be assumed for 6GR DL/UL peak spectral efficiency calculation.

In NR, maximum 8 layers are supported for DL/UL CPE devices. Whether higher number of layers shall be supported for 6GR is still open for discussion. Although higher layers can provide higher theoretical throughput, the channel fading corresponding to different UE antennas of the same UE are coherent, which will limit the number of spatially multiplexable layers, i.e., rank. Therefore, it is preferred to assume maximum 8 layers for DL/UL peak spectral efficiency calculation.

Observation 2: 8 layers can be assumed for 6GR DL/UL peak spectral efficiency calculation.

Comparing 1024QAM with 256QAM, 1.25 times of peak spectrum efficiency can be expected for DL peak spectral efficiency. Considering that around 1.5 times of peak spectrum efficiency has already been achieved in [2], and potential more system overhead efficiency improvement for 6GR, a total 1.5-2 times of DL peak spectrum efficiency can be assumed, i.e. 45-60 bits/s/Hz.

Comparing 8 layers with 4 layers, 2 times of peak spectrum efficiency can be expected for UL peak spectral efficiency.

In 6GR, diverse device types would be supported, e.g. IOT device, handheld UEs, FWA devices, etc. For handheld UE, the number of UE antennas will be limited due to the cost/size restrictions, e.g. 2T4R for 7GHz spectrum. Therefore, 8 layers will not be possible for handheld UEs. It shall be clarified that the DL/UL peak spectrum efficiency requirement may not be achieved by the handheld UEs.

Therefore, we propose:

Proposal 1: For 6GR, the minimum requirement of DL peak spectral efficiency can be set as 45-60 bits/s/Hz, and the UL peak spectral efficiency requirement can be set as 30 bits/s/Hz.

- the DL/UL peak spectrum efficiency may not be supported by handheld UEs.

The value of peak data rate is calculated based on the value of peak spectral efficiency and the channel bandwidth aggregated across multiple bands. It is noted that the required bandwidth to meet the DL/UL peak data rate for IMT-2020 has already been relatively high, e.g. >700MHz in TDD spectrum. Therefore, it is suggested that a more reasonable increase of peak data rate is considered, e.g. 1.5-2 times for DL, and 2 times for UL. Otherwise the value can be left for each company to provide.

Proposal 2: For 6GR, the minimum requirement of DL peak data rate can be set as 30-40Gbps, and the UL peak data rate can be set as 20Gbps.

- the DL/UL peak data rate may not be supported by handheld UEs.

  - 5th percentile user/Avg. spectral efficiency

In RAN#108 meeting, 3 times of DL 5th percentile user/Avg. spectral efficiency has been agreed for Indoor Hotspot and Dense Urban scenario.  It was noted that for 7GHz band up to 1024 antenna elements are assumed for BS and 4/6/8 RX antennas are assumed for UE. However, the values of UL 5th percentile user/Avg. spectral efficiency are still TBD.

For UL spectral efficiency evaluation, the same assumption on BS antenna elements can be considered, i.e. up to 1024 elements. However, the assumption on the number of TX antennas of UE needs to be carefully studied.

Different from requirement of peak spectral efficiency and peak data rate, which are theoretical calculated under ideal conditions, the minimum requirement of 5th percentile user/Avg. spectral efficiency shall be more reasonable values which can be satisfied by the typical 6GR UEs. Therefore, the typical antenna configuration of handheld 6GR UEs shall be considered as the assumption for evaluation of 5th percentile user/Avg. spectral efficiency.

Observation 3: The number of TX antennas of typical 6GR handheld UEs is considered as the assumption for 5th percentile user/Avg. spectral efficiency evaluations.

Although it is still open for discussion, we expect that 2 TX antennas will be the typical TX antennal configuration for 6G handheld UE in 7GHz band, with the following reasons:

- Till now, handheld UEs with 2 TX antennas can only be supported for some new TDD bands, e.g. n41 and n78. Although FDD bands has been utilized for many years, 2 TX antennas are still not  supported for FDD bands.

- Scaling up the number of TX antennas requires additional PA and RF chains, and will introduce issues such as mutual coupling interference, thermal management, power efficiency degradation, and hardware costs.

- More spectrum bands need to be supported by 6GR handheld UEs, which will inevitably increase the difficulty of RF design considering constraints of size, cost,  and antenna isolation, etc.

Observation 4: 2 TX antennas shall be the typical antenna assumptions for 6G handheld UEs in 7 GHz spectrum.

Preliminary evaluations have been performed assuming 2TX antenna UEs and 1024 BS antenna elements, for indoor hotspot and Dense Urban scenario. the results of which are shown in Figure 1.

Figure 1. UL Average SE and 5% percentile user SE with different deployment scenarios (Left: Dense Urban – Macro Layer; Right: Indoor Hotspot)

It can be observed that 2.5 times of UL 5th percentile user SE can be expected, and 1.5 times of average SE can be expected compared with IMT-2020. Therefore, we propose:

Proposal 3: For 6GR, the minimum requirement of

- UL 5th percentile user spectral efficiency is 2.5 times of that of IMT-2020

- UL average spectral efficiency is 1.5 times of that for IMT-2020.

  - Area Traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2), which can be expressed as

Carea = ρ × W × SEavg

where W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). Assuming that the same TRxP density as 5G, 2 times of improvements on average spectral efficiency, and 2 times increase on channel bandwidth, totally 4 times improvements on area traffic capacity can be expected.

  - Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

For IMT-2030, the minimum requirement for mobility interruption time shall be 0 ms.

- Energy efficiency

Better energy efficiency compared with 5G NR is the key to a sustainable and successful generation for 6GR. Energy efficiency can be denoted as the power consumption per transmitted bit, in terms of Joule.

The following EE definition can be used to calculate energy efficiency for UE and base station respectively:

EE=vE；E=i=0nEiTtotal=（Eon+Eoff+Etransition）Ttotal

v is the data rate, and v=R(Ttotal)/Ttotal , R(Ttotal) denotes the number of correctly transmitted or received bits during the total observation/evaluation time, i.e., Ttotal. E denotes the average energy consumption during the Ttotal . Ei  is the energy computation state, and three states, i.e., on, off, and transition, can be considered as starting point.  Eon is the energy consumption for on state when there is transmission and/or reception.  Eoff is the energy consumption for off state when there is no transmission and/or reception. Etransition is the energy consumption during transition between on state and off state.

For unloaded cases, as it does not involve the transmitting and receiving of user data, energy efficiency analysis can be reflected through average energy consumption, e.g., E.

Furthermore, it is fundamental for 6GR to guarantee base station and UE can benefit from high energy efficiency simultaneously, which is one of the targets for 6GR and captured in [3]:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device.

The joint metric to evaluate overall energy efficiency between base station and UE should take both sides into consideration. For example, a scaling factor can be introduced for base station and UE respectively on top of aforementioned energy efficiency. The detailed value can be further discussed and determined according to different scenarios or traffic loads. Another example is to analyze the impacts on the other side when evaluating the energy efficiency for one side. No matter which way to go, a comprehensive assessment on energy efficiency for both base station and UE should be achieved.

Proposal 4:  Energy efficiency can be evaluated in terms of power consumption per transmitted bit, which can be defined by the following formula for base station and UE respectively:

EE=vE；E=i=0nEiTtotal=（Eon+Eoff+Etransition）Ttotal

- Sensing

Various use cases are identified in SA1 TR 22.837 and TR 22.870. As described in a companion paper [4], from this wide range of use cases a careful selection should be done with focus on the most urgent commercial needs and taking into account what RAN already has developed during Rel-19, i.e., the ISAC channel model for object detection and tracking. Therefore, it is proposed in [2] to prioritize the use cases for UAV, human and automotive.

According to the TPR discussions, the following performance metrics can be considered for sensing.

- Missed detection probability and False alarm probability

Missed detection probability is defined as the probability of not detecting the presence of a target object when the target object is present, and False alarm probability is defined as the probability of falsely detecting the presence of a target object when the target object is not present.

- Position accuracy

The accuracy on position is defined as the difference of the sensed position of the target object to its true position value. Sensing accuracy on position can be further derived into a horizontal accuracy and a vertical accuracy.

- Velocity accuracy

The accuracy on velocity is defined as the difference of the sensed velocity of the target object to its true velocity value. Sensing accuracy on velocity may be further derived into a horizontal accuracy and a vertical accuracy too.

- Sensing resolution

Sensing resolution can be defined as the minimum difference in the measured magnitude of target objects to be allowed to detect objects in different magnitude, e.g., range resolution (m), velocity resolution (m/s).

The requirements on the performance metrics are dependent on the use cases of sensing. Even for the same target type, the performance requirements also vary a lot in different use cases. In the first release for an extensive study on ISAC, it is preferred to starting with relaxed requirements. Taking TS 22.137 as reference, we propose the KPI values for sensing use cases in Table 1.

Table 1:	Performance requirements for 6G sensing

|  | KPI value set 1 |
| --- | --- |
| Position accuracy | Horizontal: [2]m Vertical: [5]m |
| Velocity accuracy | Horizontal: [1]m/s Vertical: [1]m/s |
| Resolution | Range: [1]m Velocity: [1]m/s |
| Missed detection probability / False alarm probability | [5]% / [5]% |

Proposal 5:  Adopt the following TP for section 5.17 in TR 38.914

5.1.17	Sensing

The following performance metrics can be considered for sensing:

- Missed detection probability and False alarm probability

Missed detection probability is defined as the probability of not detecting the presence of a target object when the target object is present, and False alarm probability is defined as the probability of falsely detecting the presence of a target object when the target object is not present.

- Position accuracy

The accuracy on position is defined as the difference of the sensed position of the target object to its true position value. Sensing accuracy on position can be further derived into a horizontal accuracy and a vertical accuracy.

- Velocity accuracy

The accuracy on velocity is defined as the difference of the sensed velocity of the target object to its true velocity value. Sensing accuracy on velocity may be further derived into a horizontal accuracy and a vertical accuracy too.

- Sensing resolution

Sensing resolution can be defined as the minimum difference in the measured magnitude of target objects to be allowed to detect objects in different magnitude, e.g., range resolution (m), velocity resolution (m/s).

The performance requirements for 6G sensing are shown as in Table 5.1.17-1

Table 5.1.17-1:	Performance requirements for 6G sensing

|  | KPI value set 1 |
| --- | --- |
| Position accuracy | Horizontal: [2]m Vertical: [5]m |
| Velocity accuracy | Horizontal: [1]m/s Vertical: [1]m/s |
| Resolution | Range: [1]m Velocity: [1]m/s |
| Missed detection probability / False alarm probability | [5]% / [5]% |

- Conclusion

In this contribution, the remaining details on legacy TPR values, performance indicators for energy efficiency and sensing are discussed, and the following observations and proposals are made:

Observation 1: 1024QAM/256QAM shall be assumed for 6GR DL/UL peak spectral efficiency calculation.

Observation 2: 8 layers can be assumed for 6GR DL/UL peak spectral efficiency calculation.

Observation 3: The number of TX antennas of typical 6GR handheld UEs is considered as the assumption for 5th percentile user/Avg. spectral efficiency evaluations.

Observation 4: 2 TX antennas shall be the typical antenna assumptions for 6G handheld UEs in 7 GHz spectrum.

Proposal 1: For 6GR, the minimum requirement of DL peak spectral efficiency can be set as 45-60 bits/s/Hz, and the UL peak spectral efficiency requirement can be set as 30 bits/s/Hz.

- the DL/UL peak spectrum efficiency may not be supported by handheld UEs.

Proposal 2: For 6GR, the minimum requirement of DL peak data rate can be set as 30-40Gbps, and the UL peak data rate can be set as 20Gbps.

- the DL/UL peak data rate may not be supported by handheld UEs.

Proposal 3: For 6GR, the minimum requirement of

- UL 5th percentile user spectral efficiency is 2.5 times of that of IMT-2020

- UL average spectral efficiency is 1.5 times of that for IMT-2020.

Proposal 4:  Energy efficiency can be evaluated in terms of power consumption per transmitted bit, which can be defined by the following formula for base station and UE respectively:

EE=vE；E=i=0nEiTtotal=（Eon+Eoff+Etransition）Ttotal

Proposal 5:  Adopt the following TP for section 5.17 in TR 38.914

5.1.17	Sensing

The following performance metrics can be considered for sensing:

- Missed detection probability and False alarm probability

Missed detection probability is defined as the probability of not detecting the presence of a target object when the target object is present, and False alarm probability is defined as the probability of falsely detecting the presence of a target object when the target object is not present.

- Position accuracy

The accuracy on position is defined as the difference of the sensed position of the target object to its true position value. Sensing accuracy on position can be further derived into a horizontal accuracy and a vertical accuracy.

- Velocity accuracy

The accuracy on velocity is defined as the difference of the sensed velocity of the target object to its true velocity value. Sensing accuracy on velocity may be further derived into a horizontal accuracy and a vertical accuracy too.

- Sensing resolution

Sensing resolution can be defined as the minimum difference in the measured magnitude of target objects to be allowed to detect objects in different magnitude, e.g., range resolution (m), velocity resolution (m/s).

The performance requirements for 6G sensing are shown as in Table 5.1.17-1

Table 5.1.17-1:	Performance requirements for 6G sensing

|  | KPI value set 1 |
| --- | --- |
| Position accuracy | Horizontal: [2]m Vertical: [5]m |
| Velocity accuracy | Horizontal: [1]m/s Vertical: [1]m/s |
| Resolution | Range: [1]m Velocity: [1]m/s |
| Missed detection probability / False alarm probability | [5]% / [5]% |

- References

- 3GPP TR 38.914 v 0.1.0, Study on 6G Scenarios and requirements, Jun. 2025

- 3GPP TR 37.910, Study on self evaluation towards IMT-2020 submission