3GPP TSG RAN #109	RP-252168

Beijing, China, 15th – 18th September, 2025

Agenda item:		8.2.1.1

Source:	MediaTek Inc., T-Mobile USA

Title:	FWA evaluation scenarios for TR 38.914

Document for:		Discussion and Decision

# Introduction

At RAN#108, a contribution was provided in [1] proposing to cater specific needs within the 6G RAN Scenarios and Requirements TR 38.914 [2]. This contribution proposes to add the FWA scenario specifics to section 4 of the TR.

# Discussion & Proposal

The FWA deployment scenario has the following specific attributes that need to be catered for when evaluating such:

- FWA device capabilities, i.e. antennas and Tx power may be typically higher than for other types of device.

- FWA may have certain characteristics more typical to such a device (i.e. stationary, typical antenna height may be higher in Urban deployments especially).

We assume attributes such as Tx power, associated power control modelling and any differences in antenna heights will be further defined by the WGs. Therefore, the attributes proposed (see TP below) in this document reflect those normally captured at RAN plenary level.

Also, it was highlighted at RAN#108 that the 120MHz aggregated bandwidth for evaluations does not reflect the maximum aggregate spectrum available for deployment by a single operator within certain bands close to 2GHz such as band 41. Therefore, this is addressed in a NOTE when adding the evaluation specific aspects for FWA.

Proposal: Agree the TP to 38.914 section 4 (below) incorporating FWA-specific evaluation aspects.

# 3	Text Proposal to TR38.914

----------------------------- Start of changes -----------------------------------

## 4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4-1.

Table 4.4-1: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth (see NOTE 1) | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) Around 30 GHz: [1]GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements | TBD |
| UE antenna elements | FWA: 4Tx and 8 Rx (Around 2GHz / 4GHz / 7GHz) |
| User distribution and UE speed | FWA (Around 2GHz / 4GHz / 7GHz): 100% indoor (0.3km/h), or 100% outdoor (0.3km/h) or [ TBD % outdoor (0.3km/h and TBD % indoor (0.3km/h) [10] users per TRxP |
| Service profile | See NOTE 2: |
| NOTE 1: The aggregated system bandwidth for each frequency range is for evaluation purposes only. This does not preclude the aggregation of more bandwidth within those ranges by the 6GR system. NOTE 2: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |

## 4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.] This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5-1.

Table 4.5-1: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 700 MHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth (see NOTE 1) | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400] MHz (DL+UL) |
| Layout | Single layer - Hex. Grid [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| TRP antenna elements | TBD |
| UE antenna elements | FWA: 4Tx and 8 Rx (Around 2GHz / 4GHz / 7GHz) |
| User distribution and UE speed | General: [10% Outdoor pedestrian (3km/h), 10% Outdoor in cars (40km/h), 80% Indoor in houses (3km/h)] FWA (around 2GHz / 4GHz / 7GHz): 100% indoor (0.3km/h), or 100% outdoor (0.3km/h), or [TBD % outdoor (0.3km/h and TBD % indoor (0.3km/h) [10] users per TRxP |
| Service profile | See NOTE 2 |
| NOTE 1: The aggregated system bandwidth for each frequency range is for evaluation purposes only. This does not preclude the aggregation of more bandwidth within those ranges by the 6GR system. NOTE 2: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |

----------------------------end of changes---------------------------------

# 4		References

[1]	RP-251134, Requirements for support of FWA in 6G; T-Mobile USA, AT&T, Ericsson, Nokia, Telstra, Verizon

[2]	3GPP TR 38.914 v0.1.0 “Study on 6G Scenarios and Requirements”.