3GPP TSG RAN Meeting #109												RP-252177

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.1.   Deployment scenarios

Source:	Robert Bosch, Volkswagen AG, BMW AG, Toyota ITC, Continental Automotive, Fraunhofer IIS

Title:	Text proposal for TR 38.914 for deployment scenarios for Vehicular Use Cases

Document for:	Discussion/Decision

# Introduction

This contribution presents the Text proposal for TR 38.914 on deployment scenarios.

In RAN1#122, it was agreed that deployment scenarios from TR38.914 should be considered as evaluation assumptions in the RAN1 6G Rel-20 SI [1]. Among these evaluation assumptions, which will be taken from TR38.917, are antenna modeling and general system-level simulation assumptions (including the carrier frequency, bandwidth, and subcarrier spacing used for link-level simulation).

Agreement - The deployment scenarios in TR38.914 should be considered for evaluation assumption - The common evaluation assumptions including the antenna modelling, general system-level simulation assumptions (including the carrier frequency, bandwidth and subcarrier spacing used for link-level simulation) for the deployment scenarios in TR38.914, link budget and traffic models will be discussed in AI 11.2 - Other assumptions including for link-level simulation specific to each technical topic will be separately discussed under each individual agenda. - Note: Subcarrier spacing decision is up to AI 11.3.2. …

# Text Proposal

======================Start of the changes========================

# 4.	Deployment scenarios

## 4.11	Urban grid for connected vehicles

The urban macro deployment scenario focuses on scenario of highly densely deployed vehicle UE moving in urban area. It could cover a scenario where freeways lead through an urban grid. The main KPI evaluated under this scenario are availability, reliability, latency and data rate in high network load and high UE density scenarios.

Some of its attributes are listed in Table 4.11-1.

Table 4.11-1: Attributes of urban grid for connected vehicles

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700MHz Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 7 GHz] [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) |
| Layout | Option 1: Single layer: - Uban Grid/Roof-top Option 2: Two layers for Around 4 or 2 GHz + Around 7 GHz - Macro layer: Hex. Grid, 4 or 2 GHz (Uban Grid/Roof-top) - Micro layer: drop at Intersections, 7 GHz |
| ISD | Macro cell : 200m Micro layer: (at each intersection), values (50m and 100m) should also be considered for option 2 |
| BS configuration | TBD |
| UE configuration | Around 700 MHz: 1T2R Around 2 GHz: 1T2R and 2T4R Around 4 GHz: 1T2R and 2T4R Around 7 GHz: 1T2R and 2T4R |
| User distribution and UE speed | Urban grid model (car lanes and pedestrian/bicycle sidewalks are placed around a road block. 2 lanes in each direction, 4 lanes in total, 1 sidewalk, one block size: 433m x 250m) Average inter-vehicle distance (between two vehicles’ center) in the same lane is 1sec * average vehicle speed (average speed 30, 60, 100km/h) Pedestrian/bicycle dropping: average distance between UEs is 20m |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT2030 values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options.

[NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.]

Table 4.11-2: Details of vehicle UE drop and mobility model

| Parameter | Urban grid case | Freeway case |
| --- | --- | --- |
| Number of lanes | 2 in each direction (4 lanes in total in each street) | 3 in each direction (6 lanes in total in the freeway) |
| Lane width | 3.5 m | 4 m |
| Road grid size by the distance between intersections | 433 m * 250 m. NOTE1 | N/A |
| Simulation area size | Minimum 1299 m * 750 m | Freeway length >= 2000 m. Wrap around should be applied to the simulation area. |
| Vehicle density | Average inter-vehicle distance in the same lane is 2.5 sec * absolute vehicle speed. Baseline: The same density/speed in all the lanes in one simulation. |  |
| Absolute vehicle speed | 15 km/h, 30, 60 km/h | 60 km/h, 120 km/h |

Illustrative diagram of urban grid.

Figure 6.1.9-1: Road configuration for urban grid

## 4.12	Highway Scenario

The highway deployment scenario focuses on vehicle UEs moving on highways with high speeds. The main KPIs evaluated under this scenario are availability, reliability, latency and data rate under high speeds/mobility (and thus frequent handover operations).

Some of its attributes are listed in the following Table.

Table 4.12-1: Attributes of Highway

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 7GHz +Around 4GHz Around 2GHz+ Around 700 MHz [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz] |
| System bandwidth NOTE2 | Around 700 MHz: Up to [60] MHz (DL+UL) Around 2GHz: Up to [120] MHz (DL+UL) Around 4 GHz: Up to [200] MHz (DL+UL) Around 7 GHz: Up to [400]MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid (only around the highway sides) |
| ISD | ISD 1: 1732m ISD 2: 5000m |
| BS configuration | TBD |
| UE configuration | Around 700 MHz: 1T2R Around 2 GHz: 1T2R and 2T4R Around 4 GHz: 1T2R and 2T4R Around 7 GHz: 1T2R and 2T4R |
| User distribution and UE speed | 85% outdoor vehicles (120km/h), 15% outdoor vehicles (200 km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT2030 values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options.

[NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For UL User Experienced Data rate it is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.]

======================End of the changes========================

Proposal 1: Capture the Text Proposal in Section 2 of this TDoc to 3GPP TSG RAN level SI Technical Report TR 38.914, Section 4 “Deployment Scenarios”.

# Conclusion

The following proposal is presented:

Proposal 1: Capture the Text Proposal in Section 2 of this TDoc to 3GPP TSG RAN level SI Technical Report TR 38.914, Section 4 “Deployment Scenarios”.

# References

[1] R1-25xxxx	RAN1 Chair, Draft Agenda and Chair Notes for RAN1#122, Bengaluru, India, August 2025, Chair notes RAN1#122 v11.zip