3GPP TSG RAN Meeting #109					RP-252342

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	Google

Title:	Requirements of New and Existing Services for 6G

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some discussions on requirements of new and existing services for 6G.

# Discussion

The AI traffic (e.g., Generative AI) is a new type of traffic which is an important service to be provided by 6G. The AI traffic is fundamentally different from the traditional traffics, e.g., web browsing or video streaming traffics. The AI traffic is usually token based, in which a token is a piece of a word that an AI model understands and a single word might be one token or broken into multiple tokens. As shown in Figure 1, one AI model, e.g., large language model (LLM), can generate inference result sequentially which is consist of a lot of tokens according to instructions (prompt) received from the user. The server would start to communicate with users after generating a few tokens in a Packet instead of after generating all the tokens. The process repeats, with the next batch of tokens going into the next packet, creating a steady stream of data packets for the AI traffic.

Figure 1: AI traffic communication

Nowadays there are the following types of AI traffics:

- AI based summary. The AI writes a summary for a paper. The number of output tokens is smaller than the number of input tokens. The LLM may generate one token within 1-5ms.

- AI based text generation. The AI generates a text based on some instructions. Usually, the number of output tokens is larger than the number of input tokens. In one example, to generate a text with 800 words, 1000 output tokens may be required. The LLM may generate one token within 1-5ms.

- AI based image/video generation. The AI generates an image or a video according to some instructions. Compared to a text generation, the video generation would require more output tokens. For example, to generate a 5s video (e.g., 81 frames) with 720p, the number of output tokens could be 40K-80K or more. The LLM may generate one video within 90s.

- Interactive AI. The AI inference can be interrupted and updated according to the updated instructions from the user. This type of AI would have higher delay requirement and certain package loss would lead to incorrect inference results. The LLM may generate one token within 1ms.

For AI traffic, if one packet with output tokens is lost, the whole inference result will be broken. For example, for AI based video generation, if one output token is missing, the user cannot get the video. Further, the economic cost to generate the result for an AI traffic is 5-10x as the cost to generate the results for other types of traffic. Therefore, the reliability requirement for the AI traffic should be higher than the traditional eMBB traffic. As a starting point, the reliability for AI traffic should be 10x as the reliability of eMBB traffic. Thus, the package loss rate for an AI traffic should be 0.01. Further, for interactive AI, the package loss rate requirement could be higher, since it is more sensitive to the token loss. As a starting point, similar requirement as HRLLC can be considered.

Moreover, since the inference result is delivered by multiple packets, i.e., a burst of packets, it is also necessary to define the burst of packet loss rate as one of the requirements. As a starting point, the burst of packet loss rate as 0.01 can be considered.

The delay requirement for AI traffic depends on the types of AI. For AI based text/image/video generation, the delay requirement depends on the token generation speed and the size of tokens. As a starting point, the delay requirement of 10ms can be considered for such types of traffic. However, the interactive AI would be more sensitive to the delay, since the inference result would be changed based on the updated input tokens. Therefore, as a starting point, 1ms delay requirement can be considered.

Proposal 1: Support the packet loss rate requirement as 0.01~0.00001 for AI traffic.

Proposal 2: Support the burst packets loss rate requirement as 0.01 for AI traffic.

Proposal 3: Support the delay requirement of 1ms~10ms for AI traffic.

# Conclusion

In this contribution, we provided discussion on the requirement for new and existing services for 6G. Based on the discussion, the following proposals are provided.

Proposal 1: Support the packet loss rate requirement as 0.01~0.00001 for AI traffic.

Proposal 2: Support the burst packets loss rate requirement as 0.01 for AI traffic.

Proposal 3: Support the delay requirement of 1ms~10ms for AI traffic.