3GPP TSG RAN Meeting #109						          RP-252371

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.3

Source:	CEWiT

Title:	Requirements for 6G Architecture and MRSS

Document for:	Discussion/Decision

# Introduction

In this contribution, we propose requirements for 6G RAN architecture and 5G-to-6G RAN migration

# Discussion

## Migration plan from 5G to 6G

One of the key driving requirements to consider in executing the 6G SI in 3GPP WGs is the commercially viable migration plan for the operators and network vendors to move from 5G to 6G.  Defining a proper architecture for 5G-6G integration from network and UE perspective is very important along with the migration plan as 6G is expected to be deployed re-using the existing 5G mid-band (~3.5GHz) and site grid using spectrum refarming and the around 7 GHz band targeting comparable coverage to 5G mid-band, using enhanced beam-management features.  Based on the inputs received during RAN#108 and also the SID scope of FS_6G_RAN_Scen_Req , we propose possible 6G design aspects to be considered for smooth migration from 5G network deployment.

### Multi-RAT Spectrum Sharing

MRSS was discussed by the majority of companies as a simple and efficient way of deploying 6G in the current 5G (3.5GHz) bands. The system architecture for MRSS in a co-located site of 5G and 6G using the same band can be depicted as shown below

Fig 1. MRSS Architecture with optional co-ordination of resources (static / dynamic)

Based on the deployment requirements a static configuration of 5G and 6G TDD configurations can be assumed, or an interface from the 5G RAN node to the 6G RAN node can be facilitated, reusing the existing TDD configurations of 5G. This interface can be assumed to be used in a semi-static or dynamic manner to share TDD configurations as per the network requirements. This guarantees a simple standalone 6G system co-existing with the 5G deployments preserving legacy 5G UEs and allowing 6G UEs into the network.

### 5G-6G NW architecture based on UE stack options

Though MRSS provides a simple solution for the co-existence of 5G and 6G, this is achieved at the cost of dividing the spectrum between 5G and 6G using TDM/FDM. This result in reduced data rates for both 5G and 6G UEs. Though 6G UEs may benefit from the new 6G bands around 7GHz using CA, the 6G UE may not be utilized for it’s full potential in the anchor cell under the 3.5GHz band. To counter this, 6G NW can be architected

Fig 2. NW configurations for single stack UE

considering the UE stack options.

If we consider 6G UE to be capable of having only a single active stack at the time, still supporting both 5G and 6G, the following options are considered for the NW configurations .Now as shown in Fig.2, the UE supports both 5G and 6G stacks with only one active at a time. Wherein the option on left, the UE is connected to either 5G or 6G with support for inter-RAT mobility and in the option on right, the UE is connected to both 5G and 6G at a time with a dynamic TDM switching between 5G and 6G connectivity.

Fig 3a. NW configurations for dual stack UE with single CORE

Similarly, if the UE supports dual connectivity, the UE can be connected to both 5G and 6G at the same time with the options of RAN level aggregation, CORE level aggregation with dual connectivity and dual steering based architectures, as depicted in Fig.3a. considering a single 5GC or 6GC or using separate 5GC and 6GC as in Fig.3b.

Fig 3b. NW configurations for dual stack UE with 5GC and 6GC

Though all these schemes have their own merits and demerits, having a RAN-level resource aggregation like EN-DC is very limiting on the Stand-alone operation of 6G and so, we need to consider CORE-level resource aggregation for 5G-6G dual connectivity, where the concept of MN and SN need not be restrictive and leads to a stand-alone operation of 6G, still utilizing the 5G spectrum fully.

Proposal 1: 6G Study should aim to define a simple architecture for the 5G-6G migration using MRSS and inter-RAT co-ordination with CN-aggregation.

### Future proof enhanced network architecture

Though migration from 5G to 6G is the first step, we strongly believe that the study should also focus on future proofing the architecture for advance scenarios, which will be required from day1 targeting the current 5G pain points. One of the major pain points in 5G identified by many operators and NW vendors was the usability of DC beyond EN-DC. Though targeted towards capacity improvement, NR-DC proved to be of little use except for HetNet deployments in certain countries. We believe studying better Intra-RAT connectivity options for capacity improvement and spectrum utilization of the 6G NW also plays important role in making the 6G network future proof. So, we propose the following guideline to be studied during Rel-20 considering both the migration from 5G-6G and future proofing the network architecture.

Proposal 2: Consider the following bullets as guidelines for defining proper NW architecture, interfaces between the RAN node and the CORE NW of 6G by studying various options of network architecture, radio interface protocol architecture and procedures [in co-operation with SA2]

- Study the feasibility of different options of different RAN node connectivity and MRSS for the co-existence of 5G and 6G in the same band

    - Study and outline the RAN-Core Network interface options based on the UE stack and functional split

### Inter-gNB resource aggregation

In 5G, intra-gNB resource aggregation across multiple frequency is achieved through carrier aggregation (CA) and inter-gNB resource aggregation through dual connectivity (DC). In CA, multiple carriers are coordinated by a single gNB, whereas in DC, the connection is maintained with two gNBs. In 6G, inter-gNB resource aggregation will be one crucial feature to consider in the 6G study, to provide multiple connections with different gNBs across various bands simultaneously, enhancing reliability, data rates, and exploring the deployment restrictions.  In the current NR specification of CA, only intra gNB-DU CA (within the same DU) is possible, and CA between newly deployed carriers on different gNBs is not supported. In 6G, inter-gNB resource aggregation using CA framework and/or multi-TRP framework will improve performance, coverage, and capacity.

#### Inter-gNB CA/MTRP

Inter-gNB carrier aggregation or Multi-TRP can be utilised when the traffic distribution is non-uniform between adjacent cells and the surplus resources at a cell can be used for a UE in the adjacent cell. The target design should lead to simpler network configuration and procedures compared to NR-DC. Instead of using the Xn interface to transfer RLC SDUs between the two gNBs as in NR-DC that increases the latency, schemes for DL data flow where the data is either duplicated or split from the core network among two gNBs (viz. gNB1 and gNB2) are proposed as depicted in the figure below. The information defining the data split can be one or more of a pattern of PDCP sequence numbers, a pattern of packet numbers, parameter to an equation generating the data packet numbers, list of packet numbers, index referring to a pre-defined set of splits etc.

UE is already connected to sPCell from gNB1 and has established the PDU sessions. To enable inter-gNB CA/MTRP for a given PDU session, the PDCP and RLC entities for the radio bearers associated with the PDU session should be running using the same configuration which can be communicated through Xn interface. UE maintains a single protocol stack but with two security keys at the PDCP for processing downlink packets from these two gNBs. The RLC sequence numbers need to be assigned to all packets in UM mode irrespective of segmentation and RLC sequence numbers can be made same as that of PDCP sequence number to avoid the overlap of sequence number in the two gNBs. For retransmission purpose in RLC AM mode, the status PDUs may be shared between MN and SN through Xn.

Similarly, schemes for UL data flow shall be considered to ensure that the data reaches the application properly despite using carriers from two different cells. UL data flow should ensure that all the segments of an RLC SDU should be transmitted through the set of serving cells from a given gNB. This can be achieved by defining a carrier group which can contain the group of serving cells under a single gNB. Only a single PDCP either in gNB1 or gNB2 is used for re-ordering. The RLC SDUs received in the non-reordering gNB are to be passed to the other gNB using the Xn interface. If re-ordering is not configured (out-of-order delivery), then the receiving gNB can send UL packets directly to the Core.

Proposal 3: 6G Study should explore 6G-6G intra-RAT co-ordination options at CORE level, as an extension of the inter-RAT migration solution based on CN-aggregation

## Text proposal for TR 38.914

Following is the text proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>> ## 5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration - The RAN architecture shall support tight interworking between the new RAT and 5G NR. - Considering MRSS with sufficient inter-RAT co-ordination - Considering high performing inter-RAT mobility and aggregation of data flows supported at CORE level. - The RAN architecture shall support connectivity through multiple transmission points, either collocated or non-collocated. - The RAN architecture shall support interfaces supporting effective inter-site coordination at CORE-level <<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Conclusion

In this contribution, we make a text proposal to Chapter 5.2 in TR 38.914.

# Reference

[1] 3GPP TR 38.913: “Study on scenarios and requirements for next generation access technologies”