3GPP TSG RAN Meeting #109	RP-252374
Beijing, China, September 15th -18th, 2025

Agenda Item:	9.3.3.1

Source:	Ericsson

Title:	Rel-19 leftover for Rel-20 Data Collection for SON/MDT

Document for:	Discussion, Decision

# 1		Introduction

One of the main objectives of the Release 19 Data Collection for SON/MDT in NR standalone and MR-DC work item was to develop SON/MDT enhancements for Network Slicing and NTN. In the context of this objective, a considerable number of companies proposed the support of Conditional MDT as an important MDT enhancement for both Network Slicing and NTN [2], [3].

In this contribution we highlight the importance of considering Conditional MDT as a Rel-19 leftover for Rel-20 Data Collection for SON/MDT.

# 2	Discussion

## 2.1	Conditional MDT for Network Slicing

In the current framework for both signalling-based and management-based MDT, once a UE has been selected for MDT measurement collection by the gNB (in case of management-based MDT) or by the OAM/Core Network (in case of signalling-based MDT), the measurements are collected by the selected UEs regardless of the slice being actively used by the UE. In the context of improving network slice specific observability, is very important to ensure that measurements at the selected UE(s) are collected only when the UE(s) is actively using the slice(s) under observation.

## 2.2	Conditional MDT for NTN

In the context of management-based immediate MDT, the existing MDT framework allows the OAM to specify the Area Scope as a UE selection criterion. While this enables physically identifying and selecting UEs in certain cells, TAs, etc., the selection criteria does not determine when the measurements are done, or which of the collected measurements are signalled to the OAM. In the current MDT framework, measurements are always collected and transmitted to the OAM after a UE has been selected.

The above described, existing, MDT functionality is especially expensive when considering NTN and other deployment scenarios where the cost associated with transmitting additional information to/from the network is high. Indeed, in case of NTN all the measurements collected by the gNB and reported to the OAM are sent over the satellite to ground link which makes the process very expensive. If not all the collected measurements are relevant (no filtering is done at gNB level before reporting the measurements to the OAM), a good amount of the satellite to ground link bandwidth is wasted in transferring irrelevant information. Currently the only trade-off available is to either configure such MDT measurements, although not all of the collected information may be relevant to the analysis that may be planned with the collected data; or not configure such measurements at all. In this context, conditionally triggered MDT measurements can significantly decrease the demands on the UE and the network and facilitate collection of MDT measurements in situations where the additional collected information can be put to good use.

# 3	Conclusion

The points made in this paper identify and motivate for measurement/reporting criteria, where for a UE that has previously been selected (in accordance with a selection criteria), the measurements are collected/reported to the OAM only upon fulfilment of the measurement/reporting criteria, which may be defined in terms of thresholds, on slice-level utilisation values.

# 4	Proposal

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include Conditional MDT as a Rel 19 leftover for Rel-20 Data Collection for SON/MDT.

# 5	References

RP-234038, New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4, CMCC, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, Dec 11-15, 2023

R3-252051, MDT Enhancements for slice-focused measurements collection, Ericsson, Jio Platforms (JPL), Deutsche Telekom, AT&T, InterDigital, FiberCop, Orange, 3GPP TSG-RAN WG3 Meeting #127-bis, Wuhan, China, 7th – 11th April, 2025

R3-253193, MDT Enhancements for slice-focused measurement collection, Ericsson, Orange, InterDigital, Deutsche Telekom, Jio Platforms (JPL), FiberCop, 3GPP TSG-RAN WG3 Meeting #128, St Julian, Malta, 19th – 23rd May, 2025