3GPP TSG RAN Meeting #110						                 RP-253040

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.0

Source:	Xiaomi

Title:	Discussion on 6G NTN deployment scenario

Document for:	Discussion and Decision

- Introduction

In RAN#109 meeting, the moderator’s summary for deployment scenario in [1] provides the following guidelines:

- The not agreed parts of the pCR in RP-252887 are captured in a pCR in RP-252915, which should be used as a starting point for further discussion in RAN#110 to avoid starting off from scratch

For NTN subsection, the high-level use cases, target services, as well as table of attributes for NTN deployment scenarios are captured in RP-252915.

Moreover, in RAN1#123 meeting, the following agreement regarding to Ku band carrier frequency was made.

Agreement For 6GR NTN evaluations, the carrier frequency for Ku-band is 14GHz for UL and 11GHz for DL.

In this contribution, our views on NTN deployment scenarios are elaborated.

- Discussion

Per outcome of the current ongoing RAN plenary study on 6G scenarios and requirements, TR 38.914 lacks input on NTN deployment scenario. As per footnote under section 4, subsection 4.10 covering NTN deployment scenario requires further input including at least carrier frequency, aggregated system bandwidth and orbit/cell type information.

Carrier Frequency

In RAN1#122bis and RAN1#123 meeting, RAN WG-level SI on 6GR study was kicked off. The following agreements were made on NTN regarding to carrier frequency in evaluation assumption agenda.

Agreement At least the following carrier frequencies could be considered (from RAN1 perspective) for 6GR NTN evaluations: - L-band (i.e., 1.5GHz) - S-band (i.e. 2 GHz) - Ku-band (FFS detailed frequency range) - Ka-band (i.e. 30 GHz for UL, 20GHz for DL)

Agreement For 6GR NTN evaluations, the carrier frequency for Ku-band is 14GHz for UL and 11GHz for DL.

From carrier frequency perspectives, those agreed for RAN1 evaluation can be taken as starting point for carrier frequencies in the attributes of NTN scenarios.

Proposal 1: The carrier frequencies as agreed in RAN1 evaluation can be taken as starting point.

Aggregated system bandwidth

Carrier frequencies around 2GHz for both DL and UL as well as around 20 GHz DL and 30GHz UL have been widely evaluated since 5G and deployed worldwide for handheld UE and VSAT UE respectively. From carrier frequency perspective, these values are likely to be re-used for 6GR. The Duplex mode is FDD for these bands per the legacy duplex type setting for these bands.

For system bandwidth, throughout the RAN1 evaluations in both system level calibration and link budget analysis [2], the bandwidth of up to 2*30MHz has been widely used in L band (i.e. 1.5GHz) /S band (i.e. 2GHz) and 2*400MHz has been widely used in Ka-band (30GHz UL, 20GHz DL). In summary, we propose the followings for system bandwidth

Proposal 2: The following system bandwidth can be used for different carrier frequencies.

- 2*30MHz L band and S band

- 2*400MHz in Ka band

Orbit/cell type information

For satellite orbit and cell type information, we emphasize the following points taking the settings from TR 38.913 as starting point.

- vLEO-300 km can be additionally considered as candidate satellite orbit for 6GR NTN

- Multiple satellite layout needs to be considered as fundamental topology for cell type

For vLEO, given the commercial deployment interest for satellite service on this orbit, we propose to take the vLEO-300 km as a candidate orbit type. Moreover, in order to support high data rate service with 6G NTN, it’s envisioned that multiple satellite layout should be considered as a typical cell type setting.

Proposal 3: Capture the following in the table of attributes for NTN deployment scenarios

- vLEO-300 km can be additionally considered as candidate satellite orbit for 6GR NTN

- Multiple satellite layout needs to be considered as fundamental topology for cell type

From our perspective, the texts in RP-252915 can be used as starting point to discuss the high-level use cases and target services.

- TP for TR 38.914

Summing up the proposals and discussions in previous section, we propose the following TP on table of attributes to section 4.10 in TR 38.914.

Proposal 4: Adopt the following table of attributes to section 4.10 in TR 38.914

4.10 Non-Terrestrial Network ***Unchanged Omitted*** Table 4.10: Attributes for Non-Terrestrial Network Deployment Scenarios Attributes; Values or assumptions. Carrier Frequency; Around 1.5 GHz Around 2 GHz Around 11/14GHz Around 20/30 GHz. Aggregated system Bandwidth; 2*30MHz (1.5 GHz, 2 GHz) 2*400MHz (20/30 GHz). Orbit types; Any orbit from VLEO (300 to 600 km altitude) to GSO (~35 786 km altitude), including LEO (600 to 2000 km altitude) and MEO (2000 to ~35000 km altitude),. Architecture; Transparent payload: RAN on ground (at feederlink gateway) Regenerative payload: RAN on board or RAN split across feeder link. Radio Cell type; Earth fixed (GSO), Quasi Earth fixed (NGSO) or Earth moving (NGSO) Cell served by Multiple Satellites. ***Unchanged Omitted***

- Conclusion

In this contribution, the deployment scenarios for 6GR NTN are discussed, and the following proposals are made:

Proposal 1: The carrier frequencies as agreed in RAN1 evaluation can be taken as starting point.

Proposal 2: The following system bandwidth can be used for different carrier frequencies.

- 2*30MHz L band and S band

- 2*400MHz Ku band and Ka band

Proposal 3: Capture the following in the table of attributes for NTN deployment scenarios

- vLEO-300 km can be additionally considered as candidate satellite orbit for 6GR NTN

- Multiple satellite layout needs to be considered as fundamental topology for cell type

Proposal 4: Adopt the following table of attributes to section 4.10 in TR 38.914

4.10 Non-Terrestrial Network ***Unchanged Omitted*** Table 4.10: Attributes for Non-Terrestrial Network Deployment Scenarios Attributes; Values or assumptions. Carrier Frequency; Around 1.5 GHz Around 2 GHz Around 11/14GHz Around 20/30 GHz. Aggregated system Bandwidth; 2*30MHz (1.5 GHz, 2 GHz) 2*400MHz (20/30 GHz). Orbit types; Any orbit from VLEO (300 to 600 km altitude) to GSO (~35 786 km altitude), including LEO (600 to 2000 km altitude) and MEO (2000 to ~35000 km altitude),. Architecture; Transparent payload: RAN on ground (at feederlink gateway) Regenerative payload: RAN on board or RAN split across feeder link. Radio Cell type; Earth fixed (GSO), Quasi Earth fixed (NGSO) or Earth moving (NGSO) Cell served by Multiple Satellites. ***Unchanged Omitted***

- References

- RP-252914, Moderator's summary AI 8.2.1.1 “Deployment Scenarios”, Deutsche Telekom AG, Sept. 15th – 18th, 2025

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)