3GPP TSG RAN Meeting #110 								                   RP-253045

Baltimore, Maryland, USA, December 8-11, 2025

Agenda Item:	8.2.3.1.2

Source:	OPPO

Title:	Motivation and definition of 6G device types

Document for:	Discussion

Introduction

UE types and related features/capabilities has always been a profound topic ever since 3G times and this is not only a technical issue, but also in many cases is directly related to the UE availability, time-to-market, etc. and eventually affects the market success of relevant services. This observation reflects the complex nature of real-world usage scenarios and the strong demands on suitable UE formfactors and radio features/capabilities. This document discusses the pros and cons of different options and the benefits of UE-type based feature sets and capability definition.

Motivation of defining device types for 6G

In RAN#109, the device type topic was discussed with the following agreed:

Agreement

To investigate further:

- Motivations/justifications behind the proposed diverse device types, which should be a limited set

- Whether/how to have one or more device types for eMBB or 6G IoT

- Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

- Whether/how to explicitly standardize device types

- Ensuring forward compatibility

- Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

Motivations of defining device types

A glance back into the history reveals that when LTE specifications were drafted, UE categories (including DL-category and UL-category) were defined primarily focusing on peak data rate of both DL and UL utilizing parameters like maximum number of DL/UL-SCH transport block bits received within a TTI and maximum number of supported layers for spatial multiplexing [1]. Additionally, features not directly related to data rate are reported using UE Capability report. Later on, the “feature group indicator (FGI)” mechanism was introduced to indicate mandatory features together with their implementation and IoDT status [3, Clause B1].

This arrangement seemed reasonable at the time since most LTE UEs were smartphones and the types of expected services are mainly mobile Internet content delivery, so the peak data rate, which in many cases could be more or less proportional to the achievable user throughput, to some extent reflects the level of services end users could enjoy e.g., the time needed to load a complex web page and whether the user could watch an online HD video smoothly on their smartphones. Thus, the UE category concept was able to provide some guidance to the end users when they were choosing their next smartphone.

In the 5G era, as the number of target usage scenarios vastly increased (eMBB, mMTC and URLLC; various “vertical” scenarios etc.) and number of features surged, it became increasingly difficult and less meaningful to continue using peak data rate-oriented UE-categories. Thus, 5G NR uses a universal method for the UE to report all capabilities using RRC signalling. However, new problems emerged e.g., with the introduction of new UE-capabilities every release, the size of RRC messages also increased. And in cases e.g., when CA is configured and handover is happening, the RRC message size may exceed the limit of a PDCP SDU/PDCP Control PDU i.e., 9K Bytes. Thus, segmentation of RRC messages will be necessary, which makes the operations of both gNB and UE more complex.

From implementation and deployment perspectives, too many UE capabilities without meaningful categorization have brought complexity and additional costs to the whole industry, at least on the following aspects:

- For both chipset and infra-vendors, it is hard to decide which feature(s) should be developed, IoDTed and finally commercialized

- For UE vendors and operators, it is hard to decide on mandatory /optional capability categorization.

- Only “globally mandatory for all device types” kinds of features are actually deployed in the market. Optional features are very much not likely deployed, even if they are “truly mandatory” for certain usage scenarios and device types. And this practice seriously limits the room of performance and cost optimization for many scenario-oriented applications e.g., NES and FWA, while introduces unnecessary redundancies across various devices targeting different usage scenarios since many “globally mandatory” features are actually NOT required in these scenarios but have to be implemented

For 6G, the motivation of introducing device types include but may not be limited to:

- To meet market demands, i.e., to enable devices optimized for various market segments and diverse life-cycles; to achieve unified RAT and economy of scale with common mandatory functionalities; and, o efficiently support services and devices emerged after “Day One” i.e., the first Release of 6G specification

- To resolve the drawbacks of 4G/5G solutions e.g., to reduce UE capability/feature set reporting overhead

- To meet R&D and operational requirements e.g., to ensure scalability and forward compatibility; to ease IoDT between infra-vendors and UE vendors, and reduce time-to-market; and, to allow sufficient room for implementation-based innovation

As a summary, we may deduce the following observation for the handling of device features in 4G and 5G:

Observation 1: the handling of device features/capabilities in 4G and 5G has serious limitations and cannot meet the requirement of performance and cost optimization for various application scenarios.

Proposal 1: the definition of device types in 6G should fulfil the following requirements:

- To enable devices optimized for various market segments and diverse life-cycles

- To achieve unified RAT and economy of scale with common mandatory functionalities

- To efficiently support services and devices emerged after “Day One”

- To ensure scalability and forward compatibility

- To ease IoDT between infra-vendors and UE vendors, as well as device certification and thus reduce development cost and time-to-market

- To allow sufficient room for implementation-based innovation

- To reduce UE capability/feature set reporting overhead

Designing a scalable 6G for diverse device types

A modular design can be applied to 6GR air interface, as shown in Figure 1. A low-cost and energy-efficient “Basic function set” can be designed for 6GR air interface, which serves as a "common basis" for various usage scenarios. For a specific usage scenario, a full functionality set can be designed which “grows up” from Basic functionality set.

Figure 1. Scalable 6GR with Mandatory baseline functionality set

From our perspective, 6G MBB (i.e., Immersive Communication) and 6G IoT (i.e., Massive Communication) are the most important usage scenarios for the 1st release of 6G system. And each 6G MBB device are naturally a 6G IoT device in “low-data rate” mode. For example, when a smartphone is in a “screen off” status or communicates with IoT devices, only the low-rate “background data” are received and transmitted, only “IoT-like” traffic is conveyed for the smartphone. Therefore, the 6G IoT provides a proper design benchmark for 6G Basic functionality set. And the Basic functionality set should be designed to fully support 6G IoT operation. Meanwhile, the Basic functionality set is also designed to support low-data rate energy-efficient mode of 6G MBB. With this, the Basic functionality set is the common basis shared by 6G IoT and 6G MBB. Beyond it, a MBB full functionality set can be designed to support high-data rate mode of 6G MBB operation. This a unified 6GR air interface can be designed, scalable from 6G IoT to 6G MBB.

The design principle for modular 6GR air interface can be:

- Unified and scalable, by designing a Mandatory baseline functionality set for all device types (at least from 6G IoT to eMBB).

- The Mandatory baseline functionality set is designed to support low-data rate operation with basic coverage enhancement and basic energy-efficient operation. Growing up from the Mandatory baseline functionality set, device type-specific mandatory/optional functionality set can be defined (e.g., for eMBB, 6G IoT, etc.).

- A device type-specific mandatory functionality set is only mandatory for the corresponding device type (e.g., mandatory functionalities in Sensing functionality set are only mandatory for 6G Sensing UEs).

The purpose of defining device types is to provide a guidance to design a 6GR with intra- and inter-device-type scalability. So, RAN only needs to select a typical device class in each device type as a representative of the device type, and use it to design the scalable 6GR. It is the UE/chipset vendors’ task, not RAN’s task, to define the commercial device classes within each 6G device type, e.g., for categorize different ends of products (e.g., flagship smartphone, low-end smart phone), marketing purpose, etc. Thus, the number of device types defined in 3GPP 6GR study can be minimized. Meanwhile the device types identified in RAN should support designing a 6GR to guarantee the forward compatibility to the evolvement of 6G devices. From this perspective, minimizing the number of device types in RAN is also helpful for the forward compatibility.

Proposal 2: the definition of device types in 6G should enable a scalable 6GR

- The number of device types defined in 3GPP should be minimized, e.g., defining eMBB, IoT and FWA with high priority.

- The scalable 6GR should support forward compatibility to emerging devices in future.

Defining device types for 6G

Potential UE-type based feature sets definition in 6G and relevant capability categorization

The 6G targets at even wider range of usage scenarios - “anticipated diverse service classes ranging from immersive multimedia and massive IoT to integrated communications and sensing” [4], thus the definition of device types and relevant capability reporting are becoming imminently important for economically viable network roll-out and large-scale device penetration.

The 6G WG SID [4] demands the study to “target scalable and forward compatible design for diverse device types”, while RAN1 #122 started discussion on this topic and reached the following agreements as a first step [5]:

---------------------------------------------

Agreement

Study a scalable 6GR design for diverse device types, considering aspects:

- What should be commonly applicable to all 6G device types

- FFS: add-on features dedicated to specific device types, if any

Agreement

- Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN

- ---------------------------------------------

We believe one of the best possible ways to achieve optimized performance for specific application scenarios while at the same time to enjoy the economy of scale from a single-origin “unified” ecosystem, is to define different UE-types optimized for different scenarios, and at the same time keeping a common technology base. Here the different UE-types may own different feature sets, and the feature set of one UE-type is not necessarily the super-set or sub-set of the feature set of another UE-type. And the contents of this common technology base are up to further study, where some possible components could be listed like unified initial access scheme including SSB design.

Use highest-tier class within a device type to define the device type

In 6GR, both intra-device-type scalability and inter-device-type scalability should be supported, but via different approaches:

- Intra-device-type scalability is supported with the scalable design over device-type-specific mandatory and device-type-specific optional functionalities and capabilities.

- Inter-device-type scalability is supported with the scalable design from “mandatory baseline functionality set” to different device-type-specific functionality sets.

Proposal 3: In 6GR, both intra-device-type scalability and inter-device-type scalability should be supported, but via different approaches:

- Intra-device-type scalability is supported with the scalable design over device-type-specific mandatory and device-type-specific optional functionalities and capabilities (similar to 5G NR).

- Inter-device-type scalability is supported with the scalable design based on “mandatory baseline functionality set” together with various device-type-specific functionality sets.

For designing the scalable 6GR air interface, what really matters is the upper limit and lower limit the 6GR scalability should cover. For each device type, the highest-tier device class is used to identify the upper bound of the intra-device-type scalability, and the lowest-tier device class is used to identify the lower bound of the intra-device-type scalability, i.e., the mandatory features. However, it is too early to identify which features are mandatory at this moment. Hence, the highest-tier device class within a device type can be used to define the device type.

Figure 2: 3GPP-defined device type based on a typical device class

Observation 2: For each device type (e.g., eMBB, IoT, FWA, etc),

- The highest-tier device class is used to identify the upper bound of the intra-device-type scalability.

- The lowest-tier device class is used to identify the lower bound of the intra-device-type scalability, i.e., the mandatory features.

Proposal 4: For each device type (e.g., eMBB, IoT, FWA, etc.), the highest-tier device class can be used to define the device type.

Parameters/factors to define the device types

In RAN#109, the device types were also discussed with the followings agreed.

Agreement

In terms of diverse device types, study further:

  - Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

Note: some of the above parameters/factors may be related with form factor

Note: aim to have a focused/limited set of parameters/factors for a device type

  - The value(s) for the identified parameters for a device type

We think a limited number of key parameters/factors should be used to define the device types, which could be further divided into a minimized “mandatory” common functionality set (i.e., applicable to all device types) and device type specific parameter/features. Key parameters/factors may include maximum bandwidth, peak data rate, maximum modulation order etc. Features like UE processing capability, Sensing and AI may only be applicable to specific device type and are thus not listed.

The key parameters of three examples of device types (6G IoT, eMBB and FWA) are listed in Table 1.

- Table 1: Example of device types and Mandatory baseline functionality set

|  | Mandatory baseline functionality set | Examples of device types |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6G IoT (Lowest-tier) | eMBB (Handheld UE) | FWA (e.g., CPE) |
| Max. UE bandwidth | [5~20MHz] | [5~20MHz] | [200MHz] | [400MHz] |
| Peak data rate | [DL: 5Mbps/ UL: 1Mbps] | [DL: 5Mbps/ UL: 1Mbps] | [DL: 5Gbps/ UL: 2Gbps] | [DL: 5Gbps/ UL: 2Gbps] |
| Max. MIMO layers | 1T1R | [1T1R] | [3T6R] | [4T8R] |
| UE power class | N.A. | [20dBm] | [29dBm] | [32dBm] |
| Duplex mode | FDD/TDD | FDD/TDD/HD-FDD | FDD/TDD/SBFD | FDD/TDD/SBFD |
| Max. DL modulation order | 64QAM | 64QAM | [1024QAM] | [4096QAM] |
| Max. UL modulation order | [QPSK] | [QPSK] | [256QAM] | [256QAM] |
| UE energy efficiency | Basic power saving | Enhanced power saving | Basic power saving | Basic power saving |
| Coverage | Basic coverage enhancement | Extreme coverage enhancement | Basic coverage enhancement | Basic coverage enhancement |
| Mobility/speed | N.A. | [120]km/h | 120km/h | 3km/h |
| … |  |  |  |  |

Proposal 5: The parameters/factors for defining a device type may include:

- Maximum UE bandwidth (DL/UL)

- Peak data rate (DL/UL)

- Maximum MIMO layers (DL/UL)

- Duplex mode

- Max modulation order (DL/UL)

- Coverage

- Energy efficiency

- Mobility/speed

The values in Table 1 are used for starting point for the parameters.

Mandatory baseline functionality set

Regarding what features should be listed in the Mandatory baseline functionality set, it should be considered that the set includes mandatory baseline functionalities for all device types. Hence, It should be a “lean” functionality set, in which each functionality shall be well justified to be truly mandatory for all device types. For example, different device types have different requirements on mobility/speed. eMBB UEs and low-end IoT UEs are typically required to support 120km/h moving speed. But FWA CPE may only need to support 3km/h moving speed.

Proposal 6: Define a “lean” Mandatory baseline functionality set, in which each functionality shall be well justified to be truly mandatory for all device types.

Low-end IoT is the lowest end device type in 6G on most of aspects, which is very suitable to provide the template for identifying the Mandatory baseline functionality set. From our perspective, eMBB and 6G IoT are most important device types for the 1st release of 6G system. And each 6G MBB device in “low-data rate” mode is naturally a 6G IoT device. For example, when a smartphone is in a “screen off” status or communicates with IoT devices, only the low-rate “background data” are received and transmitted, and only “IoT-like” traffic is conveyed for the smartphone. Therefore, the low-tier 6G IoT provides a proper design template for 6GR Mandatory baseline functionality set. Starting from the template, some functionalities specially optimized for low-end IoT devices and not fit to other device types (e.g., extreme coverage enhancement for MTC sensors) can be cut off.

Therefore, a proper working approach is to firstly define the mandatory functionality set for low-tier 6G IoT, then use the low-tier 6G IoT functionality set as a template to form the Mandatory baseline functionality set, where existing functionalities unjustified for other device types should be taken out, and necessary “outside” functionalities could be added in.

Figure 3: Low-end IoT provides a good template for defining Mandatory baseline functionality set

Proposal 7: Use the lowest-tier 6G IoT functionality set as a template/starting point for definition of the Mandatory baseline functionality set for 6GR.

- Functionalities specially optimized for low-tier 6G IoT (e.g., extreme coverage enhancement) does not have to be included.

Proposal 8: The Mandatory baseline functionality set for all 6G device types include:

- Basic PHY features, such as waveform, modulation, coding, frame structure, numerology.

- Basic duplex modes.

- Basic initial access and Idle mode procedures.

- Basic DL/UL control channels and scheduling/HARQ procedures.

- Basic MIMO, e.g., 1 TRX chain.

- Basic power saving mechanism.

- Smallest maximum supported RF and BB UE BW.

- Meeting basic coverage target.

Minimum spectrum allocation for 6GR

The minimum spectrum allocation in which 6G can operate depends on the minimum spectrum operators need to operate 6G network and minimum bandwidth 6GR initial access procedure can complete.

From the 6G spectrum perspective, most of 6G networks will be deployed in a spectrum with at least 5MHz bandwidth. But some operators also require to deploy 6G IoT networks in 3MHz spectrum to re-farm their low-frequency fragmented spectrums being used by e.g., GSM-R systems. For success of 6G, Massive IoT is an essential usage scenarios. If the low-frequency 3MHz spectrum can be re-farmed, it will provide valuable frequency resource for 6G IoT services.

From initial access signal/channel design perspective, both 5MHz bandwidth and 3MHz are feasible. As agreed in RAN1#123, the two options listed can both support the 3MHz spectrum allocation with 15kHz SCS. The common signals/channels will be designed with a down-selection between the two options via further study in RAN1.

Agreement

If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR,

- Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

- Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations

Therefore, operators’ requirements on deploying 6G network in 3MHz spectrum should be supported. 6GR common signals/channels for initial access should be applicable to any spectrum allocations such as to avoid the significant performance loss caused by puncturing used in 5G.

Proposal 9: Support 3MHz as minimum spectrum allocation (with 15kHz SCS) of 6GR for FR1 FDD. Consider to design the 6GR common signals/channels applicable to the 3MHz channel BW.

Smallest maximum UE channel bandwidth

In RAN#122 and #123meetings, the following agreement was reached:

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

  - FFS: whether RF and BB UE BW are same or different

Agreement

- For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least

  - Overall device complexity

  - Overall system performance impact

  - Energy efficiency for both BS and UE

  - Aim at a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority

On this aspect, the service requirement and spectrum resource should be considered.

As discussed in Section 2, the design principle of 6GR is to support a unified air interface scalable from 6G IoT to 6G MBB. The smallest maximum UE BW is obviously more related to a proper setting of 6G IoT capability, rather than 6G MBB capability. If the smallest maximum BW is set to a too high value, the cost and power consumption of 6G IoT devices cannot be well controlled. If the smallest maximum UE BW is set to a too low value, it is difficult to design a unified 6GR air interface for 6G IoT and 6G MBB.

In principle, reducing the bandwidth would reduce the cost and power consumption of the UE. However, one may argue that a mature and scaled 20MHz chipsets and UE components supply chain has been formed in 4G and 5G era. Reusing the ecosystem is the major driver for maximizing the cost efficiency. At this moment, 5MHz and 20MHz should both be studied as two options for the maximum UE bandwidth of the lowest-tier 6G IoT devices.

And the typical bandwidth of wearable devices is 20MHz or larger. Hence, if taking the 5MHz option, the wearable devices (i.e., 6G RedCap) can be treated as a separate device type (e.g., with maximum UE bandwidth of at least 20MHz). If taking the 20MHz option, the wearable device (i.e., 6G RedCap) is combined into the lowest-tier 6G IoT device type.

Proposal 10: Study the two options for smallest maximum UE bandwidth for lowest-tier 6G IoT devices.

- Option 1: 5MHz

  - If taking this option, the wearable devices (i.e., 6G RedCap) can be treated as a separate device type (e.g., with maximum UE bandwidth of at least 20MHz).

- Option 2: 20MHz

  - If taking this option, the wearable device (i.e., 6G RedCap) is combined into the lowest-tier 6G IoT device type.

Conclusion

In this contribution, the motivation and methodology of defining device types are discussed, as well as the key parameters to form a device type.

The following observations and proposals are made:

Observation 1: the handling of device features/capabilities in 4G and 5G has serious limitations and cannot meet the requirement of performance and cost optimization for various application scenarios.

Observation 2: For each device type (e.g., eMBB, IoT, FWA, etc),

- The highest-tier device class is used to identify the upper bound of the intra-device-type scalability.

- The lowest-tier device class is used to identify the lower bound of the intra-device-type scalability, i.e., the mandatory features.

Proposal 1: the definition of device types in 6G should fulfil the following requirements:

- To enable devices optimized for various market segments and diverse life-cycles

- To achieve unified RAT and economy of scale with common mandatory functionalities

- To efficiently support services and devices emerged after “Day One”

- To ensure scalability and forward compatibility

- To ease IoDT between infra-vendors and UE vendors, as well as device certification and thus reduce development cost and time-to-market

- To allow sufficient room for implementation-based innovation

- To reduce UE capability/feature set reporting overhead

Proposal 2: the definition of device types in 6G should enable a scalable 6GR

- The number of device types defined in 3GPP should be minimized, e.g., defining eMBB, IoT and FWA with high priority.

- The scalable 6GR should support forward compatibility to emerging devices in future.

Proposal 3: In 6GR, both intra-device-type scalability and inter-device-type scalability should be supported, but via different approaches:

- Intra-device-type scalability is supported with the scalable design over device-type-specific mandatory and device-type-specific optional functionalities and capabilities (similar to 5G NR).

- Inter-device-type scalability is supported with the scalable design based on “mandatory baseline functionality set” together with various device-type-specific functionality sets.

Proposal 4: For each device type (e.g., eMBB, IoT, FWA, etc.), the highest-tier device class can be used to define the device type.

Proposal 5: The parameters/factors for defining a device type may include:

- Maximum UE bandwidth (DL/UL)

- Peak data rate (DL/UL)

- Maximum MIMO layers (DL/UL)

- Duplex mode

- Max modulation order (DL/UL)

- Coverage

- Energy efficiency

- Mobility/speed

Proposal 6: Define a “lean” Mandatory baseline functionality set, in which each functionality shall be well justified to be truly mandatory for all device types.

Proposal 7: Use the lowest-tier 6G IoT functionality set as a template/starting point for definition of the Mandatory baseline functionality set for 6GR.

- Functionalities specially optimized for low-tier 6G IoT (e.g., extreme coverage enhancement) does not have to be included.

Proposal 8: The Mandatory baseline functionality set for all 6G device types include:

- Basic PHY features, such as waveform, modulation, coding, frame structure, numerology.

- Basic duplex modes.

- Basic initial access and Idle mode procedures.

- Basic DL/UL control channels and scheduling/HARQ procedures.

- Basic MIMO, e.g., 1 TRX chain.

- Basic power saving mechanism.

- Smallest maximum supported RF and BB UE BW.

- Meeting basic coverage target.

Proposal 9: Support 3MHz as minimum spectrum allocation (with 15kHz SCS) of 6GR for FR1 FDD. Consider to design the 6GR common signals/channels applicable to the 3MHz channel BW.

Proposal 10: Study the two options for smallest maximum UE bandwidth for lowest-tier 6G IoT devices.

- Option 1: 5MHz

  - If taking this option, the wearable devices (i.e., 6G RedCap) can be treated as a separate device type (e.g., with maximum UE bandwidth of at least 20MHz).

- Option 2: 20MHz

  - If taking this option, the wearable device (i.e., 6G RedCap) is combined into the lowest-tier 6G IoT device type.

References

- 3GPP TS36.306 V18.5.0 Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities, 2025.6

- 3GPP TR38.913 V18.0.0, Study on Scenarios and Requirements for Next Generation Access Technologies, 2024-03

- 3GPP TS36.331 V18.6.0,

- RP-251881	New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- Chairman notes of RAN1 #122, End of meeting, Bengaluru, India, Aug 25th – 29th, 2025

Text Proposal to TR38.914 v0.2.0

TP Start

5.3.2	Diverse device types

Motivations of defining diverse device types for 6G include:

•	To enable devices optimized for various market segments and diverse life-cycles

•	To achieve unified RAT and economy of scale with common mandatory functionalities

•	To efficiently support services and devices emerged after “Day One”

•	To ensure scalability and forward compatibility

•	To ease IoDT between infra-vendors and UE vendors, as well as device certification and thus reduce development cost and time-to-market

•	To allow sufficient room for implementation-based innovation

•	To reduce UE capability/feature set reporting overhead

The 6G RAN shall support diverse device types with scalable and forward compatible design to accommodate various application scenarios and services with optimized performance and cost.

The 6G RAN shall support an essential common technical foundation applicable to diverse device types to ensure economy of scale and to minimize complexity of system implementation.

The 6G RAN shall support the minimum spectrum allocation of 3MHz.

The 6G RAN shall support the maximum bandwidth of [TBD]MHz for the lowest-tier UE.

The 6G RAN shall define the diverse device types to include both essential communication capabilities and application-specific capabilities e.g., enhanced energy saving, mobility, extreme coverage, sensing and AI.

The parameters/factors for defining a device type may include:

•	Maximum UE bandwidth (DL/UL)

•	Peak data rate (DL/UL)

•	Maximum MIMO layers (DL/UL)

•	Duplex mode

•	Max modulation order (DL/UL)

•	Coverage

•	Energy efficiency

•	Mobility/speed

The values in Table 1 are used for starting point for the parameters.

- Table 1: Example of device types and Mandatory baseline functionality set

|  | Mandatory baseline functionality set | Examples of device types |  |  |
| --- | --- | --- | --- | --- |
|  |  | 6G IoT (Lowest-tier) | eMBB (Handheld UE) | FWA (e.g., CPE) |
| Max. UE bandwidth | [5~20MHz] | [5~20MHz] | [200MHz] | [400MHz] |
| Peak data rate | [DL: 5Mbps/ UL: 1Mbps] | [DL: 5Mbps/ UL: 1Mbps] | [DL: 5Gbps/ UL: 2Gbps] | [DL: 5Gbps/ UL: 2Gbps] |
| Max. MIMO layers | 1T1R | [1T1R] | [3T6R] | [4T8R] |
| UE power class | N.A. | [20dBm] | [29dBm] | [32dBm] |
| Duplex mode | FDD/TDD | FDD/TDD/HD-FDD | FDD/TDD/SBFD | FDD/TDD/SBFD |
| Max. DL modulation order | 64QAM | 64QAM | [1024QAM] | [4096QAM] |
| Max. UL modulation order | [QPSK] | [QPSK] | [256QAM] | [256QAM] |
| UE energy efficiency | Basic power saving | Enhanced power saving | Basic power saving | Basic power saving |
| Coverage | Basic coverage enhancement | Extreme coverage enhancement | Basic coverage enhancement | Basic coverage enhancement |
| Mobility/speed | N.A. | [120]km/h | 120km/h | 3km/h |

End of TP