3GPP TSG RAN Meeting #110	 RP-253050

Baltimore, USA, December 8-11, 2025

Agenda Item:	9.3.1.4

Source:	HONOR

Title:	Views on solutions for Ambient IoT in NR Phase 2

Document for:	Discussion and Decision

Introduction

In RAN#108, a new Rel-20 WID " New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2" [1] has been approved and revised in RAN#109 [2], with the objectives as follows:

Objectives - Specify necessary support for active Device(s), taking the Rel-19 specifications as the starting point, in indoor scenarios under the following conditions [RAN1]: - Deployment scenario 1 with topology 1, according to D1T1-C. - Traffic types DO-DTT, DT, DO-A. - For DO-A traffic, WGs begin discussions from Clause 6.10 “DO-A assessment” in TR 38.769. - Representative use cases rUC1 (indoor inventory), rUC2 (indoor sensor), and rUC4 (indoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing indoor NR micro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. This objective begins after RAN#111 (March 2026) This objective includes: RAN1-led - In specifying support for the above, re-use the evaluation assumptions of TR 38.769 - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Air interface for active device(s) studied for outdoor will be reused for supporting active device(s) in indoor scenarios RAN2-led - DO-A specific procedure design. - Paging and random access for active device(s). RAN3-led - Specify DO-A specific NGAP procedure. RAN4-led - Study coexistence between active Device(s) and NR/LTE in indoor scenarios [RAN4-led] - Specify RF requirements for Type 1-C Ambient IoT BS where differences are needed from Rel-19. - Specify RF requirements for active Device(s) with IF/ZIF receiver architecture - Specify RRM core requirements for active Device(s) in D1T1 - Use band n8 as an example band - Study and specify the test methodology for AIoT active devices NOTE: Strive to minimize evaluation cases in RAN1 and RAN4. - Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN1 - The objective includes specification of necessary power control and collision handling procedures between intermediate UE and BS. - No physical layer impact for signaling A-IoT resources to the intermediate UE. - No additional specification change on R2D and D2R for Device 1 in Rel-20 RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling. RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility RAN4 - Specify RF requirements for intermediate UE towards the A-IoT device. - Specify necessary updates to RF requirements for A-IoT device 1. - Specify A-IoT active device(s) RF requirements in D2T2 - Specify RRM requirements for intermediate UE due to A-IoT operation, if any. Combination with existing RRM procedures/requirements is excluded. - Specify RRM requirements for Device(s) in D2T2 if necessary - Use band n8 as an example band. - Specify device (un)availability via Direction 2 in TR 38.769, [RAN2-led, RAN1]. - This objective is applicable only for active Device(s) - Solutions shall be unified for Topology 1 and Topology 2. - This objective begins after RAN#111 (March 2026) - NOTE: Device for outdoor scenarios may be added here, pending RAN#111 decision. Note: Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band.

Discussion

In the current WID RP-252894, it is clearly stated that the coexistence of Device 1 and Device 2b/C in the same deployment in the same band is not considered：

Note: Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band.

From the WID perspective, the representative use cases supported by objective 1 are rUC1 (indoor inventory), rUC2 (indoor sensor), and rUC4 (indoor command). objective 2 supports representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). From the current commercial RFID, at least in the inventory use case, different categories of products are present. Compared with RFID, there may be commercial needs to deploy both device 1 and device 2b simultaneously in the same deployment, e.g., D1T1 or D2T2, to cover a larger area.

Observation 1: From the representative use cases supported by WID, there is a possibility of simultaneously supporting different categories of devices in the same deployment scenario.

But under the constraints described in the above note, if the operator wants to support services for two types of devices, at least two FDD spectrums are required. FDD spectrum is relatively expensive and scarce for operators, and most operators do not have the resources for two FDD spectrums. Therefore, this will greatly limit the commercial deployment of Ambient IoT.

Observation 2: Supporting two types of devices requires the operator to have at least two FDD spectrums, which greatly limits the commercial deployment of Ambient IoT features.

For UE supporting topology 2, it also needs to support simultaneous transmission and reception of at least two FDD spectrums. This will greatly increase the complexity and power consumption of UE implementation.

Observation 3：Supporting two types of devices simultaneously will greatly increase the complexity and power consumption of UE implementation.

Based on the above analysis, we believe it is not necessary to add this constraint. To reduce design complexity, a mutually exclusive approach can be adopted for handling the two types of devices. For example, the two types of devices can be processed separately by time division or frequency division of resources. Therefore, we suggest updating the WID to delete the restriction.

Proposal 1: Update WID to delete the note: Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band.

Figure 1. Time division processing of device1 and device2b/C

Figure 2. Frequency division processing of device1 and device2b/C

Conclusions

In this proposal, we discussed the note constraints in the New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2, and have the following proposal:

Observation 1: From the representative use cases supported by WID, there is a possibility of simultaneously supporting different categories of devices in the same deployment scenario.

Observation 2: Supporting two types of devices requires the operator to have at least two FDD spectrums, which greatly limits the commercial deployment of Ambient IoT features.

Observation 3：Supporting two types of devices simultaneously will greatly increase the complexity and power consumption of UE implementation.

Proposal 1: Update WID to delete the note: Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band.

References

- RP-251885, “New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2”, RAN#108, June. 9 - 13, 2025.

- RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2, RAN#109, September 2025.