3GPP TSG RAN Meeting #110						RP-253059

Baltimore, USA, December 8-11, 2025

Agenda Item:		8.2.1.0

Source:		AT&T

Title:				6G Deployment Scenarios

Document for:		Discussion/Decision

# 1	Introduction

In this document, views are provided on the remaining issues for deployment scenarios considered for 6G Evaluations, included in 3GPP TR38.914, “Study on 6G Scenarios and Requirements” [1], following the agreements in RAN#109 and RAN1#122bis and RAN1#123.

# 2	Discussion

Deployment scenarios for 6G were addressed during RAN#109, with the conclusions summarized in RP-252888 [2].  RAN1#122bis [3] and RAN1#123 [4] further discusses the evaluation assumptions following the guidance from RAN#109.

The group agreed to include several scenarios for consideration for 6G deployments—such as indoor hotspot, dense urban, rural, urban macro, sub-urban macro, high speed, extreme long distance, and urban coverage for massive connection—while further discussion is required to finalize their detailed attributes. In this contribution, we present our views on some of the remaining attributes.

For indoor, dense urban, urban, suburban macro and rural scenarios, the following remaining attributes from RAN#109 [2] were not finalized.

| UE antenna elements | TBD |
| --- | --- |
| User distribution and UE speed |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

For the User distribution and UE speed, RAN1#123 [4] agreed multiple options depending on the use case being evaluated, as follows:

RAN1#123 Agreement

For 6GR evaluation, the UE distribution and UE speed for system-level simulation is assumed as follows:

- UE number per TRxP will be dependent on the used traffic model. Other values can also be considered in the future evaluations.

- Other velocities and/or other outdoor/indoor ratio will be dependent on the used traffic model or use cases. Other values can also be considered in each of other topics.

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 80% indoor (3km/h); 20% outdoor(30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h) | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 50% indoor (3km/h); 50% outdoor(120km/h). Opt2: 20% indoor (3km/h) 40% outdoor (60km/h) 40% outdoor (120km/h) | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 80% indoor (3km/h); 20% outdoor(30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h) | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. Opt2: 20% outdoor in cars: 40km/h 80% indoor in houses: 3km/h |
| FFS: Applicability for FWA |  |  |  |  |  |

The options in the agreement from RAN1#123 can be considered for the UE distribution and UE speed for indoor, dense urban, urban, suburban and rural scenarios. In particular, for the UE number per TRxP, the number depends on the traffic model considered and the use case being evaluated. A larger number than 10 can also be considered.

For the UE distribution, some options have been considered and agreed on in TR38.901 and should be captured in the evaluation scenarios. Other options can be included as needed. Thus, our proposal is to use the following for UE distribution and UE speed for the evaluation of the agreed scenarios.

Proposal 1: Use the following UE distribution and UE speed as attributes for the evaluation of agreed deployment scenarios.

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is (10, 30) Default: 80% indoor (3km/h); 20% outdoor(30km/h). Optional: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h) | Single layer: Uniform/macro TRxP UE number per TRxP is (10, 30). Default: 50% indoor (3km/h); 50% outdoor(120km/h). Optional: 20% indoor (3km/h) 40% outdoor (60km/h) 40% outdoor (120km/h) | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is (10, 30) Default: 80% indoor (3km/h); 20% outdoor(30km/h). Optional: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h) | Single layer: Uniform/macro TRxP UE number per TRxP is (10, 30) Default: 20% outdoor in cars: 40km/h 80% indoor in houses: 3km/h Optional: 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. |
| FFS: Applicability for FWA |  |  |  |  |  |

RAN1#123 further had a discussion on the UE antenna modelling for evaluations, total number of antenna elements, antenna configurations, and antenna locations. No agreement has been reached however for multiple reasons, amongst that the lack of guidance from the plenary on the maximum UE antenna elements to be considered.

RAN1#123 note captured in [4]: Agreement

For 6GR evaluation, RAN1 to model the UE antenna as follows,

- Note: Each of other topics could further decide to use which combination(s) for the evaluations. Other combinations are not precluded for evaluations, e.g., 2T6R, 3T6R, 6T6R, 6T8R.

- Note: The antenna locations in Alt1 and Alt 2 in the following table are considered as examples and used for performance calibration. Any antenna array structures and/or antenna locations in section 7.3 in TR38.901 is possible for evaluations and up to companies to report.

- Note: The antenna locations in Alt 2 not included in section 7.3 in TR38.901 are up to companies to report.

- Note: The antenna element/location of T is a subset of the element/locations for R.

- FFS: Alt1 or Alt2 is used for each of the combination.

- FFS on UE antenna modelling for 30GHz.

- Note: The mapping between the combination and the device types might be separately discussed

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 | Applicable carrier frequency |
| --- | --- | --- | --- | --- |
| Combination0 Note: This combination is not for handheld UE. | 1 | 1T1R, | Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) Alt 2: - 1T - 1R | 700MHz, 2GHz |
| Combination1 | 2 | 1T2R, | Alt 1: - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2R: [(1, 5), or (4, 8)] as described in section 7.3 in TR 38.901. | 700MHz, 2GHz, 4GHz |
| Combination2 | 4 | 1T4R, 2T4R, 4T4R | Alt 1: - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901 | 700MHz 2GHz, 4GHz, 7GHz, 15GHz |
| Combination3 Note: This combination is not intended for handheld UE. | 8 | 1T8R, 4T8R, 8T8R | Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 | 2GHz, 4GHz, 7GHz, 15GHz Note: If number of TXRU and frequency combination is applicable |
| Combination4 Note: This combination is not for intended for handheld UE. | 16 | FFS: 4T16R, 8T16R, 16T16R | Alt 1: - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; x, y) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization | 7GHz, 15GHz |

There are multiple aspects to be considered for the UE antenna elements discussions.

It is important to have UE configurations to capture the diverse device types, in accordance with the proposal in [5] endorsed for RAN in RAN#109.

Proposal 4: In terms of diverse device types, study further:

- Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

Note: some of the above parameters/factors may be related with form factor

Note: aim to have a focused/limited set of parameters/factors for a device type

- The value(s) for the identified parameters for a device type

A minimum UE configuration that captures massive IoT devices, to be evaluated as part of RAN1 evaluations, should be captured and agreed, at least for low FR1 frequencies.

Similarly other configurations for handheld devices, on various considered frequencies should be considered for evaluations, up to 8T8R as discussed in 5G-A.

There was a discussion in RAN1 to add UE antenna configurations up to 16T16R, for 7GHz and 15GHz, primarily for FWA devices. We do not believe considering such large antenna configurations is feasible or needed for 15GHz, given that we have not converged on an architecture for these frequencies, and whether this will be feasible for such frequencies.

Also given that the 6G spec is designed to serve all types of UE, and we are not developing a separate specification for certain types of devices, it is not clear whether/how the framework for UL signalling will work for 16Tx, beyond whether the devices themselves and their hardware are capable of supporting such large number of antenna elements and corresponding TxRUs. More specifically, beamforming procedures at the UE with such large number of antennas needs further investigation, since adding more antennas yields benefits with higher digital beamforming resolution and/or hybrid beamforming, and based on discussions in RAN1#123, there didn’t seem to be any clear guidance on the feasibility of such solutions on the UE side, even for FWA. Given that, we do not see a need nor sufficient support for increasing the number of UE antenna elements and corresponding number of Tx chains to 16 for 7GHz or 15GHz. We thus make the following proposal, for UE antenna elements in 6G.

Proposal 2: Use the following UE antenna elements as attributes for the evaluation of agreed deployment scenarios.

| UE antenna elements | Applicable carrier frequency |
| --- | --- |
| 1, 2, 4 | 700MHz, 2GHz |
| 2, 4, 8 | 4GHz, 7GHz, 15GHz |

There were some remaining attributes to be finalized for some of the deployment scenarios, such as an additional ISD for rural scenario at 7500m. Note that the channel model in TR38.901 was not properly calibrated for 5000m, and it is unclear whether the channel model can faithfully be applicable for such large ISDs. A 7500m ISD thus does not seem feasible to be used with the current modelling tools in RAN1, namely in TR38.901[6].

Proposal 3: For evaluation of rural deployment scenario in 6G, an ISD of 7500m is not supported with the current channel model in TR38.901.

Finally, much of the deployment scenarios discussion has been focused on evaluations of eMBB, but new services, such as sensing and FWA need to be captured as well for the corresponding deployment scenarios.

For sensing, we believe in addition to indoor and urban environments captured in 5G-A, suburban macro deployment scenario needs to be used as a main evaluation scenario in 6G, due to the large percentage of suburban deployments in North America and other regions in the world.

Proposal 4: For sensing, in addition to urban macro, 6G uses suburban macro scenario as a main deployment scenario

Similarly, for FWA, suburban and rural deployment scenarios can be used.

Proposal 5: For FWA, 6G uses suburban and rural deployment scenarios for evaluations.

# 3 Conclusion

In this contribution we discussed the deployment scenarios for 6G in 38.914. The following proposals were made to be captured in Section 4 of 38.914.

Proposal 1: Use the following UE distribution and UE speed as attributes for the evaluation of agreed deployment scenarios.

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is (10, 30) Default: 80% indoor (3km/h); 20% outdoor(30km/h). Optional: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h) | Single layer: Uniform/macro TRxP UE number per TRxP is (10, 30). Default: 50% indoor (3km/h); 50% outdoor(120km/h). Optional: 20% indoor (3km/h) 40% outdoor (60km/h) 40% outdoor (120km/h) | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is (10, 30) Default: 80% indoor (3km/h); 20% outdoor(30km/h). Optional: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h) | Single layer: Uniform/macro TRxP UE number per TRxP is (10, 30) Default: 20% outdoor in cars: 40km/h 80% indoor in houses: 3km/h Optional: 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. |
| FFS: Applicability for FWA |  |  |  |  |  |

Proposal 2: Use the following UE antenna elements as attributes for the evaluation of agreed deployment scenarios.

| UE antenna elements | Applicable carrier frequency |
| --- | --- |
| 1, 2, 4 | 700MHz, 2GHz |
| 2, 4, 8 | 4GHz, 7GHz, 15GHz |

Proposal 3: For evaluation of rural deployment scenario in 6G, an ISD of 7500m is not supported with the current channel model in TR38.901.

Proposal 4: For sensing, in addition to urban macro, 6G uses suburban macro scenario as a main deployment scenario

Proposal 5: For FWA, 6G uses suburban and rural deployment scenarios for evaluations.

# 4	References

- 3GPP TR 38.914, Study on 6G Scenarios and requirements, Release 20 (draft)

- RP-252888 pCR on Deployment scenarios for TR 38.914 (after 2. Offline), Beijing, China

- 3GPP RAN1#122bis chair’s notes, Prague, Czech Republic

- 3GPP RAN1#123 chair’s notes, Dallas, USA

- RP-252873 Summary on diverse device types, Beijing, China

- 3GPP TR38.901 v19.0.0 (2025-06), Study on channel model for frequencies from 0.5 to 100GHz (Release 19)