3GPP TSG RAN Meeting #110					         RP-253062

Baltimore, USA, December 8-11, 2025

Source:	AT&T

Title:	Spectrum Considerations for 6G

Agenda Item:	8.2.1.3.1

Document for: 	Discussion/Decision

# 1. Introduction

At the TSG RAN #109 meeting in Beijing, China a revised SID was approved to study 6G scenarios and requirements [1]. In this document, we provide our views on spectrum considerations in support of diverse device types.

# 2. Background

3GPP has been successfully addressing the Internet-of-Things (IoT) segment since LTE Release 13 with the introduction of eMTC and NB-IoT. These technologies have subsequently been evolved as evident in Table 1. Additionally, each release has also improved latency, coverage, device complexity, power consumption, mobility, and spectral efficiency.

Table 1: Evolution of eMTC and NB-IoT in 3GPP

Since the introduction of eMTC and NB-IoT, 3GPP has introduced additional IoT technologies as shown in Figure 1. IoT-NTN comprises the non-terrestrial network (NTN) complements of eMTC and NB-IoT and was introduced in Rel. 17.  Unlike eMTC and NB-IoT, which are low power wide area technologies, Ambient IoT was specified in Rel. 19 as low power short-range 3GPP technology. And lastly, RedCap introduced reduced capability devices with reduced UE complexity and capability constraints in Rel. 17. All of these are 5G technologies, however, as depicted in Figure 1, some are 5G NR technologies (38 series) while others are 5G LTE technologies (36 series), a consequence of the TSG RAN #79 agreement  that “no NR based solution is specified for Low-Power Wide-Area (LPWA) use cases.”

Figure 1: Overview of 3GPP IoT Technologies

3GPP introduced massive IoT very late in the 4G life cycle and early eMTC/NB-IoT devices could not reliably be upgraded over-the-air to connect to 5GC. 3GPP also specified multiple technologies for massive IoT, namely, eMTC, which shares some common channels and signals with 4G eMBB, and NB-IoT as an independent RAT. Because 3GPP never specified a 5G NR based technology for massive IoT, eMTC and NB-IoT can coexist with 5G NR using dynamic spectrum sharing (DSS) or guard-band deployments (NB-IoT only).

With the introduction of RedCap, history repeated itself. Both 4G eMTC/NB-IoT and 5G (e)RedCap were designed long after the initial release of the respective G. Both 4G eMTC and 5G (e)RedCap inherited the minimum bandwidth from eMBB. The minimum bandwidth in 4G was 1.4 MHz and the minimum bandwidth in 5G was 5 MHz.

Both 4G and 5G introduced bandwidths smaller than the minimum bandwidth for eMBB in later releases. NB-IoT with 200 kHz (occupying 180 kHz) can operate standalone in GSM channels or eMBB guard bands. NB-IoT can also coexist with eMBB in overlapping spectrum, however, with much lower efficiency (2x common channel overhead). In the 5G era, NR added support for dedicated spectrum with less than 5MHz bandwidth in Release 18.

All previous G’s have been designed top down starting with the most demanding use cases (eMBB and URLLC) which were then scaled down to ever lower complexities, powers, capabilities, bandwidths …

6G networks should be designed bottom-up, meaning a single RAT is scaled from a minimum set of KPIs, 
and resources are allocated upwards to the most demanding ones. In addition to enabling vertical industries with long life cycles, this bottom-up scalability establishes a platform and toolbox approach with one set of specifications that support all use cases with minimum customization and increased economy of scale.

The departure from a top-down design paradigm with all the aforementioned benefits for verticals is additionally expected to deliver network energy savings and overhead reductions for eMBB operation. Instead of reducing network energy consumption later in the G via dedicated work items (Rel. 12 vs. Rel. 8 in LTE and Rel. 18 vs. Rel. 15 in 5G NR), the 6G RAT is designed scalable from the onset, starting from a minimum set of KPIs up to the most demanding ones.

Figure 2: 6G Massive IoT Design Paradigms

# 3. Spectrum Considerations for 6G

Both 4G LTE and 5G NR support 3 MHz spectrum allocations. So far, 3GPP TSG RAN has not yet agreed what the minimum spectrum allocation for 6GR should be. Operators, however, have made it clear, and this requirement was formally agreed in 3GPP TSG RAN #109, that 6GR should be optimized for mobile broadband services.

In 4G LTE, PSS, SSS, PBCH, PDCCH, and PDSCH can operate in bandwidths as small as 1.4 MHz. In fact, PSS, SSS, and PBCH are limited to 1.4 MHz (6 resource blocks) regardless of the carrier bandwidth (1.4, 3, 5, 10, 15, or 20 MHz). The system bandwidth is signaled in the MasterInformationBlock (MIB) in PBCH and subsequently, the PDCCH is frequency-interleaved across the entire system bandwidth. This design necessitated the introduction of the SIB1-BR, a bandwidth reduced (BR) version of the SystemInformationBlock1 in Release 13 that can be transmitted in 1.4 MHz regardless of the carrier bandwidth into which it is embedded.

This design has both benefits and drawbacks. One crucial downside of this design is the duplication of system overhead. Some common channels have to be transmitted both as narrowband (bandwidth reduced) version for IoT devices and as wideband version for mobile broadband. At the same time, this design can truly cater to both the requirements of eMBB and IoT due to their dedicated designs.

In our view, 3GPP should carefully trade-off the requirements for IoT and eMBB services, while optimizing for the latter. That being said, spectrum is a crucial and expensive resource for mobile network operators. Hence, spectrum efficiency is of paramount importance and duplication of system information should be avoided at all costs. At the same time, mobile broadband services should not be limited by a design optimized for IoT devices.

We also observe that IoT services are predominantly offered in low-band FDD spectrum, while eMBB is additionally offered in mid-band TDD spectrum. It should thus be possible and desirable to design 6GR such that in some bands, eMBB and IoT can share the same system information broadcast with a narrower bandwidth catering to both the needs of eMBB and IoT, while in other bands, where only eMBB services are offered, carriers can be configured with wideband system information broadcast transmissions. Such a design retrains the benefits of LTE (SIB1 for eMBB, SIB1-BR for IoT and eMBB in overlapping spectrum with IoT), while avoiding two different types of system information broadcast for IoT and eMBB, respectively.

Because 4G LTE and 5G NR both allow operation in 3 MHz bands, we expect 6GR to also be able to operate in 3 MHz. With the above design paradigms and considerations in mind, we suggest PSS/SSS/PBCH continue to be designed for 5 MHz (assuming 15 kHz subcarrier spacing). Other common channels should be configurable such that they can be confined to 5 MHz regardless of the carrier bandwidth if IoT and eMBB devices are serviced in overlapping spectrum. In other words, 6GR should define a narrowband SIB1-BR that is applicable to both IoT and eMBB devices. Absent IoT devices, in carriers with only mobile broadband devices, it should be possible to configure common channel transmissions in bandwidths larger than 5 MHz.

Additionally, 6GR should define UE RF requirements that can avoid band specific SAW filters to enable global and low cost IoT solutions. For example, for IoT devices the initial access bandwidth could be restricted to 3 MHz rather than 5 MHz.

Proposal:

- PSS/SSS/PBCH are designed for 5 MHz (assuming 15 kHz subcarrier spacing)

- Other common channels are configurable such that they can be confined to 5 MHz regardless of the carrier bandwidth if IoT and eMBB devices are serviced in overlapping spectrum

- Absent IoT devices, in carriers with only mobile broadband devices, it should be possible to configure common channel transmissions in bandwidths larger than 5 MHz

- 6GR should define UE RF requirements that can avoid band specific SAW filters to enable global and low cost IoT solutions

# 4. Conclusion

In this document, we provided our views on spectrum considerations in support of diverse device types. The following is proposed:

Proposal:

- PSS/SSS/PBCH are designed for 5 MHz (assuming 15 kHz subcarrier spacing)

- Other common channels are configurable such that they can be confined to 5 MHz regardless of the carrier bandwidth if IoT and eMBB devices are serviced in overlapping spectrum

- Absent IoT devices, in carriers with only mobile broadband devices, it should be possible to configure common channel transmissions in bandwidths larger than 5 MHz

- 6GR should define UE RF requirements that can avoid band specific SAW filters to enable global and low cost IoT solutions

# References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC/Verizon/NTT DOCOMO, INC./Deutsche Telekom