3GPP TSG-RAN Meeting #110									       RP-253080

Baltimore, USA, December 8-11, 2025

Agenda Item:		8.2.1.3.2

Source:		Deutsche Telekom, Vodafone, Orange, Telecom Italia, Turkcell, Odido, Spark NZ, BT, Bouygues Telecom, Telefonica, Telia Company, KPN, CK Hutchison, Telenor, KT Corp., T-Mobile USA, Boost Mobile Network, Jio Platforms

Title:		TP on diverse Device Types for 6G for TR 38.914

Document for:		Discussion and decision

## 1	Introduction

The RAN P SI on “Study on 6G scenarios and requirements” has been approved in December 2024 [1].

At the RAN#108 a multiple operator contribution in [2] defined key requirements for 6G for inclusion into the newly created TR 38.914 [3] but has not been discussed nor any agreements have been taken as basis for inclusion to the TR.

With this contribution the cosigning operators list their key requirements for 6G on the topic of diverse Device Types for inclusion into the TR 38.914.

## 2	Text Proposal

*** START TEXT PROPOSAL ***

### 5.3.2	Diverse device types

- 6G RAT shall define a limited set of diverse UE classes

- 6G UE shall support a device design of 5 MHz bandwidth for initial access in the first release

- 6G eMBB UEs shall meet the following minimum requirements:

  - Maximum bandwidth of at least >= 20 MHz in bands 600/700/800/900, incl. operation in all channel bandwidths

  - Maximum bandwidth of at least >= 40 MHz in FDD/SDL bands 1500/1800/2100/2600, incl. operation in all channel bandwidths

  - Maximum bandwidth of at least >= 200 MHz in TDD bands like n41, n77, n78 incl. operation in all channel bandwidths

  - Maximum bandwidth of at least >= 200 MHz in band n104, incl. operation in all channel bandwidths

- 6G eMBB UEs shall support following minimum requirements:

    - 2 DL MIMO layers / 2 Rx for bands < 1 GHz

    - 1 UL MIMO layer / 1 Tx for bands < 1 GHz

    - 4 DL MIMO layers / 4 Rx for bands > 1 GHz

    - 2 UL MIMO layers / 2 Tx for bands > 1 GHz

*** END TEXT PROPOSAL ***

## 3	Proposal

It is proposed by the cosigning operators to agree the above TP to be captured in the new TR 38.914 v0.2.0

## References

[1] 	RP-243327 “Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, RAN#107

[2] 	RP-251682 “Key operator requirements for 6G”, Deutsche Telekom, Vodafone, Orange, Telecom Italia, Telefonica, BT, Bouygues Telecom, Spark NZ, Odido, Turkcell, Telenor, Telus, CKH IOD UK, Jio Platforms, Reliance Jio, KPN, Boost Mobile Network, 3GPP RAN#108

[3]	RP-252882 “3GPP TR 38.914 V0.2.0”, 3GPP RAN#109