3GPP TSG RAN Meeting #110												RP-253086

Baltimore, USA, December 15-18th, 2025

Title:	Text proposal on NTN deployment scenarios

Source:	Thales, TNO, Iridium, BMW, Bosch, Airbus, Eutelsat, Novamint, Toyota, ESA CATT, CSCN, LG Electronics, Sharp, EchoStar

Agenda Item:	8.2.1.0 Deployment scenarios

Document type:	discussion

Document for:	Decision

Targeted document: TR 38.914

# Introduction

This contribution presents a Text Proposal for clause 4.10 on NTN deployment scenarios for the TR 38.914.

======================Start of the changes========================

# 4.	Deployment scenarios

# ..

## 4.10	Non-Terrestrial Networks

Editor note: orbit type and payload type will be captured in section 4.10

NTN deployment scenarios are defined to allow for the provision of services:

- where terrestrial network services are not or poorly available on a permanent (e.g. low-density populated areas and maritime areas) or temporary (e.g. where disasters disrupt terrestrial networks or damage undersea communication cables) basis including in vertical space areas (e.g., up to several kilometers altitude), and

- that can be efficiently supported by Non-Terrestrial Networks.

Accordingly, NTN will mainly contribute to:

- Achieving ubiquitous connectivity and availability of services in remote and sparsely populated areas,

- Improving overall 6G network resiliency by supporting in priority critical communications and emergency communications (messaging, voice calls).

Non-Terrestrial Networks may support messaging, enhanced Mobile BroadBand (eMBB), voice, video, positioning, sensing, delay tolerant, machine type communications, Multicast-Broadcast Services (MBS) and other data services. It may also support enablers for Navigation and Timing services.

The attributes of the NTN deployment scenarios are listed in Table 4.10

Table 4.10: Attributes for Non-Terrestrial Network Deployment Scenarios

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 1.5 GHz Around 2 GHz Around 4 GHz Around 11/14 GHz Around 20/30 GHz Around 40/50 GHz |
| Aggregated system Bandwidth | Up to [60] MHz (DL+UL): Around 1.5 GHz and Around 2 GHz Up to [100] MHz (DL+UL): Around 4 GHz Up to [200] MHz (DL+UL): Around 11/14 GHz Up to [800] MHz (DL+UL): Around 20/30 GHz Up to [1600] MHz (DL+UL): Around 40/50 GHz |
| Orbit types (Note 1) | Any orbit from VLEO (300 to 600 km altitude) to GSO (~35 786 km altitude), including LEO (600 to 2000 km altitude) and MEO (2000 to ~35000 km altitude), |
| Duplex mode | In paired bands (Satellite service allocated bands) and all orbits: FDD (Frequency Division Duplexing) or Half Duplex Frequency Division Duplexing (HD-FDD) mode In unpaired bands (Satellite service allocated bands) with Max RTD < [TBD] ms (e.g. VLEO/LEO only): TDD (Time Division Duplexing) mode (Note 4) |
| Architecture (Note: 2) | Transparent payload: RAN on ground (at feederlink gateway) Regenerative payload: RAN on board |
| Feeder link | 3GPP or non-3GPP defined radio access technology |
| Satellite access node’s antenna characteristics | Antenna array of max [2000] radio elements |
| Radio Cell type (Note: 3) | Earth fixed (GSO), Quasi Earth fixed (NGSO) or Earth moving (NGSO) |
| Beam hopping parameter | Number of simultaneous active beams: 32/16; Number of beams per satellite footprint: 1058 |
| User speed | Up to 1500 km/h for aircraft mounted terminals |
| User distribution | Below 7.125 GHz: - Case 1: 100% Outdoor - Case 2: [TBD%] Outdoor and [TBD%] in adverse propagation conditions (e.g. light indoor, in vehicle) Above 10.7 GHz: Outdoor only: [100%] Outdoor |
| Note 1: The orbit types can be characterised with a set of parameters (e.g. Maximum Round Trip Delay (ms), Maximum Doppler shift (Hz), Max Differential delay (ms), Max Differential Doppler (Hz), Max Doppler rate (Hz/s), Max delay variation (s-2). The NTN channel model type in TR 38.811 can be considered for performance evaluation. Note 2: A transparent payload forwards transparently the received 3GPP radio protocol. For regenerative architecture, the feeder link may be based on 3GPP or non-3GPP defined NTN radio access technology. Note 3: A radio cell is able to serve UEs on the earth surface as well as in the space above. It may be realised with one or several beams generated by satellites. Only part of the beams generated by a given satellite may be active. Note 4: TDD mode design should consider at least the band around 1.5 GHz. |  |

Note:

- Several types of User Equipment may be served by Non Terrestrial Networks: handheld, automotive, other moving platforms (e.g., trains, ships, airplanes, drones) and IoT devices.

- NTN may provide coverage in areas where TN also provides services (overlapping coverage), or where TN does not provide services on a permanent or temporary basis (non-overlapping coverage).

- NTN may be based on a combination of orbit types (e.g. GSO and NGSO).

======================End of the changes========================

# 4	Conclusion

Proposal: Add the text proposal in clauses 4.10 of the TR 38.914.

END