3GPP TSG RAN Meeting #110					RP-253099

Baltimore, MD, USA, December 8-11, 2025

Agenda Item:	8.2.1.4

Source:	Ericsson, AT&T, Itron, Nokia, Nordic Semiconductor, NTT DOCOMO, Semtech, Sony, Verizon, Vodafone

Title:	Massive Communication (IoT) requirements

Document for:	Decision

# 1	Introduction

One of the IMT-2030 usage scenarios is Massive Communication, which is described as follows in [1]:

Massive Communication This usage scenario extends massive Machine Type Communication (mMTC) of IMT-2020 and involves connection of massive number of devices or sensors for a wide range of use cases and applications. Typical use cases include expanded and new applications in smart cities, transportation, logistics, health, energy, environmental monitoring, agriculture, and many other areas such as those requiring a variety of IoT devices without battery or with long-life batteries. This usage scenario would require support of high connection density, and depending on use cases, different data rates, low power consumption, mobility, extended coverage, and high security and reliability.

Within the RAN study on 6G scenarios and requirements [2], RAN#109 agreed the following text proposal for TR 38.914 [3]:

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

RAN#109 also made the following agreement (not yet captured in TR 38.914):

For 6G Massive Communication (IoT), the minimum number of UE receive/transmit antenna is 1 for lowest-tier device - Reflect this agreement in the device type section of the TR

In this contribution, we discuss and provide additional input on the minimum requirements for Massive Communication.

# 2	Discussion

## 2.1	Minimum peak data rate

Cat-1/1bis and eRedCap UEs support DL/UL peak rates of 10/5 Mbps and 10/10 Mbps, respectively. Very roughly speaking, NB-IoT and eMTC UEs have an order of magnitude lower peak rates, and Cat-4 and RedCap UEs have an order of magnitude higher peak rates. Too high peak rates should be avoided for complexity and power consumption reasons, and too low peak rates should be avoided for spectral efficiency, latency, and energy consumption reasons.

An important lesson learned from the 4G IoT technologies is that while the PHY-layer peak rates can be low, they should not be unnecessarily low, especially for HD-FDD and TDD where the resulting MAC-layer peak rates may become substantially lower than the PHY-layer peak rates due to scheduling delays. For example, a Rel-13 Cat-M1 HD-FDD UE has a PHY-layer peak rate of 1 Mbps in both DL and UL but a MAC-layer peak rate of 300 kbps in DL and 375 kbps in UL, which was ultimately considered to be on the low side. This triggered various enhancements in Rel-14 and later to improve both the PHY- and MAC-layer peak rates for Cat-M1. A somewhat higher peak rate can have several advantages:

- Improved spectral efficiency and network capacity

- Efficient support of, e.g., conversational services and software/firmware upgrades

- Improved battery life since transmission/reception can finish quicker, allowing UE to go to sleep earlier

- Support of more IoT use cases, helping to achieve economies of scale for the lowest-tier 6G UE

- Higher probability that, e.g., SIB can be scheduled in a way that is efficient for all 6G UEs

Therefore, the PHY-layer peak rates of the lowest-tier UE should not be much lower than ~5 Mbps, especially since the potential UE complexity reduction from going down from ~5 Mbps to ~1 Mbps is expected to be very minor. Furthermore, the MAC layer data rate which accounts for HD-FDD or TDD operation would end up being too low. The minimum required DL/UL PHY-later peak data rates for Massive Communication may need further discussion, but our view is that a reasonable value is in the range of 5~10 Mbps.

Proposal 1:	The lowest-tier UE supports a DL PHY peak data rate in the range [5~10] Mbps.

Proposal 2:	The lowest-tier UE supports a UL PHY peak data rate in the range [5~10] Mbps.

## 2.2	Half-duplex FDD (HD-FDD) operation

In FDD bands, NB-IoT, eMTC, RedCap, and eRedCap UEs can be implemented with half-duplex operation, meaning that they are not required to transmit and receive at the same time. Other 4G/5G UEs including Cat-1/1bis and Cat-4 in practice only support full-duplex operation, which requires a duplex filter, or possibly multiple duplex filters if the UE supports multiple bands, which can become complex and increase the device cost. Since IoT devices are global devices, it is important that they can support multiple bands in a cost-efficient way, which can be enabled by HD-FDD if networks support it globally. It can be troublesome to introduce support for half-duplex FDD retroactively in the standard and in products. Therefore, it is important that half-duplex FDD operation is properly supported already in the first 6G release.

The text proposal for TR 38.914 in the Conclusions section of this contribution proposes to clarify that both FDD and TDD are supported, and that the lowest-tier FDD UE shall support HD-FDD.

Proposal 3:	Massive Communication (IoT) shall be supported in FR1 FDD and FR1 TDD.

Proposal 4:	The lowest-tier FDD UE shall support half-duplex FDD operation.

## 2.3	Smallest maximum UE bandwidth

According to a RAN1 agreement confirmed by RAN#109, the candidate values for smallest maximum UE bandwidth in FR1 that are currently being considered are 3, 5, 10, and 20 MHz. Different values may be considered for DL/UL, RF/BB, FDD/TDD. The values should be selected such that they achieve an appropriate trade-off between the complexity for the lowest-tier UE and the impact on the overall 6GR design and performance.

Regarding the impact of UE bandwidth on UE complexity, the earlier RAN1 studies for LTE-M (TR 36.888), RedCap (TR 38.875) and eRedCap (TR 38.865) can provide some guidance. According to these studies, a smaller UE bandwidth results in a lower UE complexity, but the returns are diminishing, meaning that reducing the UE bandwidth below 5~10 MHz may not give much additional complexity reduction. As an example of the diminishing returns of UE bandwidth reduction, TR 38.875 Table 7.8.2-1 estimates that reducing the UE Tx/Rx RF bandwidth from 100 to 20 MHz reduces the UE modem complexity by 31.9% in the FR1 FD-FDD 2-Rx case, whereas TR 38.865 Table 7.2.2-7 estimates that reducing the bandwidth from 20 to 5 MHz reduces the complexity by 14.31% in the FR1 FD-FDD 2-Rx case. The numbers for other cases are slightly different, but the trend is the same in all cases, i.e., bandwidth reduction from 20 to 5 MHz results in a much smaller relative complexity reduction than bandwidth reduction from 100 to 20 MHz. Further bandwidth reduction below 5 MHz is expected to yield even smaller relative complexity reduction, so from UE complexity point of view, there does not seem to be a strong need to further consider the 3-MHz option for the smallest maximum UE RF Rx bandwidth. These studies focused mainly on UEs supporting a single band, and the impact on the UE RF Tx side from supporting multiple bands is discussed further down in this section.

To maintain a common air interface for 6G IoT UEs and other 6G UEs (same SSB, CORESET#0, SIB transmissions, etc.), it is desired that the smallest mandatory UE bandwidth is not too small, to avoid negative impacts on initial access for other 6G UEs. From a pure network perspective, having all UEs support a wide bandwidth simplifies the operation of the system and provides high performance. The smallest maximum UE bandwidth will determine the bandwidth of common signals/channels (CORESET#0, SIB, etc.) on carriers that are intended to serve both the lowest-tier UEs and other UEs. However, an operator with multiple carriers should be able to configure the network so that the lowest-tier UEs are only served on some carriers while the other carriers are reserved for more capable UEs. This means that the bandwidth of the common signals/channels can be wider than the smallest maximum UE bandwidth on those other carriers. It should be noted that the bandwidths of these signals/channels may depend on, e.g., the SCS and thus effectively be band dependent.

Taking all the above aspects into account, we have the following proposals.

Proposal 5:	The smallest maximum UE RF Rx bandwidth (DL bandwidth) for the lowest-tier UE is at least 5 MHz for FR1 FDD (15 kHz SCS).

Proposal 6:	The smallest maximum UE RF Rx bandwidth (DL bandwidth) for the lowest-tier UE is at least 10 MHz for FR1 TDD (30 kHz SCS).

Proposal 7:	To enable operation on a variety of carrier bandwidths, the 6GR design supports configurability of the bandwidth(s) of common signals/channels. Hence, the network can configure the bandwidth(s) of the common signals/channels in a cell to be either within the smallest maximum UE bandwidth or wider than the smallest maximum UE bandwidth depending on whether the lowest-tier UEs (for IoT) are intended to be served.

It is important to note that the smallest maximum UE bandwidth does not determine what carrier bandwidths the UE can support. For example, FR1 RedCap UEs have a maximum UE bandwidth of 20 MHz, but they can operate on NR carriers with carrier bandwidths that are much smaller (e.g., 5 MHz) or much larger (e.g., 100 MHz). Thus, the above proposals have no implications regarding the smallest supported spectrum allocation or smallest supported carrier bandwidth.

Regarding the UE RF Tx side, it has been suggested that to enable a single, cost-efficient HD-FDD UE implementation to address a global market it is desired to facilitate implementations that do not require several band-specific analogue RF (SAW) filters. There may be different ways to enable this, for example using a relatively small maximum UE Tx (RF and/or BB) bandwidth of 3~5 MHz or using UE Tx power back-offs or UL scheduling restrictions to avoid the carrier edges. RAN4 should study feasibility from UE and NW points of view before further decisions are made regarding these aspects. The RAN4 study could potentially also cover the TDD case, where there may be stronger motivation for keeping the RF bandwidths for UE Tx and UE Rx the same.

Proposal 8:	To aid in decisions on smallest maximum UE RF Tx bandwidth (UL bandwidth), ask RAN4 to study [until June 2026] feasibility from UE and NW points of view of supporting cost-efficient HD-FDD UE implementations addressing a global market without the need for several band-specific analogue UE RF Tx (SAW) filters.

## 2.4	UE battery life

There will be Massive Communication use cases in 6G that require long battery life in the order of at least 10 years, potentially up to 15-20 years. The complexity reduction techniques can help reduce power consumption, and additional power saving techniques such as extended discontinuous reception (eDRX) and wake-up receivers (WUR) are beneficial for UEs that need a long battery life. These additional power saving techniques can be supported by 6G IoT UEs as well as ordinary 6G UEs. The UE battery life requirements (including assumptions on traffic models, channel conditions, battery capacity) for Massive Communication require further study.

Proposal 9:	Study UE battery life requirements for Massive Communication (IoT).

## 2.5	Latency, coverage, connection density

The legacy 3GPP IoT technologies are also associated with requirements on the following aspects.

- Latency: The CP/UP latency requirements for 6G UEs used for Massive Communication use cases can potentially be relaxed compared to other use cases. If the bandwidth and peak rates are not too small, it should still be possible to achieve, e.g., an end-to-end latency of no more than ~100 ms, allowing an IoT device to set up a connection and send a small packet relatively quickly, or to support a conversational voice service, which can be useful for some IoT services. Latency requirements are not discussed further here but can be revisited later.

- Coverage: Some Massive Communication use cases will require or benefit from coverage extension beyond “normal” 6G coverage, which is yet to be defined. Coverage targets are treated under agenda item 8.2.1.1.

- Connection density: Connection density requirements are expected to be addressed in the IMT-2030 TPR discussion and are not addressed further in this contribution.

Thus, the text proposal below does not address latency, coverage, and connection density aspects.

# 3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1:	The lowest-tier UE supports a DL PHY peak data rate in the range [5~10] Mbps.

Proposal 2:	The lowest-tier UE supports a UL PHY peak data rate in the range [5~10] Mbps.

Proposal 3:	Massive Communication (IoT) shall be supported in FR1 FDD and FR1 TDD.

Proposal 4:	The lowest-tier FDD UE shall support half-duplex FDD operation.

Proposal 5:	The smallest maximum UE RF Rx bandwidth (DL bandwidth) for the lowest-tier UE is at least 5 MHz for FR1 FDD (15 kHz SCS).

Proposal 6:	The smallest maximum UE RF Rx bandwidth (DL bandwidth) for the lowest-tier UE is at least 10 MHz for FR1 TDD (30 kHz SCS).

Proposal 7:	To enable operation on a variety of carrier bandwidths, the 6GR design supports configurability of the bandwidth(s) of common signals/channels. Hence, the network can configure the bandwidth(s) of the common signals/channels in a cell to be either within the smallest maximum UE bandwidth or wider than the smallest maximum UE bandwidth depending on whether the lowest-tier UEs (for IoT) are intended to be served.

The proposals above are captured by the following text proposal for TR 38.914 [3]:

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1 FDD and FR1 TDD. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD5~10] Mbps in DL and [TBD5~10] Mbps in UL for lowest-tier device. - The lowest-tier FDD UE shall support half-duplex FDD operation. - The smallest maximum UE RF Rx bandwidth for the lowest-tier UE is [at least 5 MHz] for FR1 FDD (15 kHz SCS) and [at least 10 MHz] for FR1 TDD (30 kHz SCS). - To enable operation on a variety of carrier bandwidths, the 6GR design supports configurability of the bandwidth(s) of common signals/channels. Hence, the network can configure the bandwidth(s) of the common signals/channels in a cell to be either within the smallest maximum UE bandwidth or wider than the smallest maximum UE bandwidth depending on whether the lowest-tier UEs (for IoT) intend to be served. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

In addition, we have the following proposals for further study:

Proposal 8:	To aid in decisions on smallest maximum UE RF Tx bandwidth (UL bandwidth), ask RAN4 to study [until June 2026] feasibility from UE and NW points of view of supporting cost-efficient HD-FDD UE implementations addressing a global market without the need for several band-specific analogue UE RF Tx (SAW) filters.

Proposal 9:	Study UE battery life requirements for Massive Communication (IoT).

We also note that TR 38.914 should be updated to capture the following RAN#109 agreement:

For 6G Massive Communication (IoT), the minimum number of UE receive/transmit antenna is 1 for lowest-tier device - Reflect this agreement in the device type section of the TR

# References

M.2160-0 (11/2023), “Recommendation ITU-R M.2160-0: Framework and overall objectives of the future development of IMT for 2030 and beyond”, ITU-R, November 2023.

RP-251395, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, RAN#108, June 2025.

TR 38.914, “Study on 6G Scenarios and requirements”, RAN.