3GPP TSG RAN Meeting #110					RP-253100

Baltimore, USA, December 15-18, 2025

Agenda Item:	8.2.1.3.2           Operational requirements: Diverse device types

Source:	Thales, Sateliot, Iridium, Viasat, ST Engineering iDirect, Novamint, Eutelsat, TNO, EchoStar, Airbus, Gatehouse Satcom, ESA

Title:	TP on NTN devices

Document type/for: Discussion/Decision

# 1	Introduction

This document aims at providing requirements for NTN in clause 5.3.2 “Diverse device types” of the TR 38.914.

# 2	Discussion

6G should be able to cover all people on Earth, and all areas, including oceans. With the seamless integration of the various 3GPP-compliant satellite networks into terrestrial networks, it shall be possible to deliver global coverage for basic mobile broadband, FWA, and IoT services.

- Achieving ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improving network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables.

This may be achieved with a variety of devices adapted to the installation/usage conditions.

# 3 Text proposal for TR 38.914

Based on the above discussion we propose the following text proposal.

====================Start of the changes==================

## 5.3	Operational requirements

Editor note: Spectrum, co-existence, topology and other operational requirements

…

### 5.3.2	Diverse device types

5.3.2.x NTN

Several types of User Equipment may be served by Non Terrestrial Networks: handheld, automotive and IoT devices as well as moving platforms/building mounted terminals or Small size antenna array (e.g. size smaller than 20 x 20 cm). Moving platforms refer to vehicles, trains, ships, airplanes, drones.

The characteristics of the NTN capable UE to be addressed by the 6G Radio access network are listed in Table 5.3.2.x

Table 5.3.2.x: Attributes for Non-Terrestrial Network capable UE

| UE types | Min antenna gain | Min Transmit Power | Max Noise figure | UE Ground speed | Coverage |
| --- | --- | --- | --- | --- | --- |
| IoT and smartphones (FR1-NTN) | -5 dB | 20 dBm | 7 dB | Up to 500 km/h (on board high speed trains) | Outdoor, Adverse propagation conditions (NOTE1) |
| VSAT type 1 (NOTE2, NOTE3) | Equivalent to the one of 20 cm aperture antenna (FR2-NTN), 0 to 3 dBic (FR1-NTN) | Equivalent to 26 dBm (FR1-NTN) and 2 Watts (FR2-NTN) | [4] dB (FR2-NTN), 7 dB (FR1-NTN) | Up to 1500 km/h (aircraft) Up to 250 km/h (cars) Up to 500 km/h (high speed train) Up to TBD km/h (NGSO satellite) | Outdoor only |
| VSAT type 2 | Equivalent to the 60 cm aperture antenna | TBD | TBD | Same as above | Outdoor only |
| NOTE1: Adverse propagation conditions (e.g. Light indoor, In car) requires Maximum Coupling Loss comparable to 3GPP defined NB-IoT. It may lead to service rate degradation. NOTE2: It refers to cars, buses, trains, drones, vessels, aircrafts, building mounted devices and possibly on board satellites NOTE3: For cars, see RP-232733 and 5GAA “Maximising the benefit of future satellite communications for automotive” |  |  |  |  |  |

In the table above FR2-NTN refers to all equipment operating in bands above 10 GHz (e.g., it includes Ku band equipment using FR1-NTN or FR2-NTN numerology).

For drones/UAV, not only UE ground speed is an important attribute, but also the UE acceleration.

==================End of the changes========================

# 4	Conclusion

Proposal: Add the text proposal in clauses 5.3.2 of the TR 38.914.