3GPP TSG RAN Meeting #110	                                              	                    RP-253108

Baltimore, USA, Dec. 8-11, 2025

Agenda item:	9.3.1.3

Title:	Discussion on AI/ML for NR air interface Phase 2

Source:	OPPO

Document for:	Discussion

# Introduction

In the revised WID [1] of AI/ML for NR air interface Phase 2, it marked the following checkpoints in RANP#110.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Data collection [RAN2]: - Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection

In this contribution, we analyze the inputs from RAN4 and SA working group and briefly share our views on the aspects of inter-vendor training collaboration and data collection.

# Inter-vendor training collaboration

WT#3: Whether and how to support NW standardized signaling of dataset and model parameter sharing for non-OTA (Over The Air) scenarios, where the dataset and model parameter is received from gNB:

- gNB->OAM/CN->UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity;

- CN->UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity

In SA2#172, the SID was proposed to investigate and identify potential architecture and system level enhancements to support inter-vendor training collaboration Direction A, Option 4-1 and Option 3a-1 [2].

From SA2 perspective, two ways are now under consideration to address the dataset exchange in Direction A, Option 4-1 and model parameter exchange in Direction A, Option 3a-1. One is from gNB->OAM/CN->UE-side training entity, where gNB is NW-side dataset/model parameter collection entity. The other is from CN->UE-side training entity, in which CN is NW-side dataset/model parameter collection entity. Details for both ways will be further studied in SA2, and the conclusion will be expected in SA#112 in June 2026.

Observation 1: From SA2 perspective, two ways are proposed and studied to address non-OTA dataset exchange in Direction A, Option 4-1 and model parameter exchange in Direction A, Option 3a-1.

From RAN4’s perspective, - Option 3 is feasible for RAN4 to complete. RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model. - RAN4 agrees that the test decoder of option 3 is the same as the reference decoder of direction C and the reference encoder of option 3 is the same as the reference encoder of direction C. If so, RAN4 believes that direction C should be feasible to complete too. - Direction A 3a-1 is feasible for RAN4 to complete from reference encoder perspective. Hence, RAN1 could possibly consider the RAN4 reference encoder for the standardized encoder model structure for Direction A 3a-1. - RAN4 agreed the detailed steps to develop the model.

Regarding Direction C and Direction A, Option 3a-1, in the LS from RAN4 [3], the conclusions were made in RAN4#117. RAN4 has concluded the feasibility of Direction C with fully specified reference model, wherein the test decoder of option 3 can be used as the reference decoder of Direction C and the reference encoder of option 3 can be the same as the reference encoder of Direction C. In addition, Direction A, Option 3a-1 is also feasible by using the RAN4 reference encoder as the standardized encoder model structure.

Observation 2: From RAN4 perspective, it is feasible to support fully specified reference model for Direction C and specified reference encoder for Direction A, Option 3a-1.

Besides the feasibility issue, to facilitate the practical deployment of Direction C and Direction A, Option 3a-1, the following aspect should be further considered.

- Content/format of reference encoder parameters exchange: to facilitate the UE-side model training in Direction A, Option 3a-1, the reference encoder parameters should be exchanged from NW-side to UE-side. The content and format of exchanged parameters should be specified in RAN1.

Proposal 1: For Direction A, Option 3a-1, the content/format of reference encoder parameters exchange should be addressed in RAN1.

Currently in RAN1, the dataset content/format for Direction A, Option 4-1 is discussed and specified to facilitate the inter-vendor training collaboration for two-sided AI/ML models. This is the simplest way to address the inter-vendor collaboration issue without any specification on model structures. Therefore, to make the two-sided model for CSI compression workable, it is important to prioritize the specification support of Option 4-1. Afterwards, Option 3a-1 can be started by reusing some designs for Option 4-1, e.g., pairing ID, performance target.

Proposal 2: To facilitate the inter-vendor training collaboration, supports both Option 4-1 (with higher priority) and Option 3a-1 under Direction A in RAN1.

# Data collections

RAN2 started UE-side data collection discussion in R18, some discussed options have SA involvement. Based on RAN request [4], both SA2 and SA5 have an individual SID in R20 [5] [6]. SA2 had finished their study with some open issues [7]:

As feedback to request in RP-243316, SA2 would like to provide information on the outcome of the related SA2 study FS_AIML_CN_Ph2 (SP-250810).

SA2 has reached interim agreements with remaining open issues on how to support the transfer of standardized data over UP for UE data collection for UE-side model training as described in clauses 7.1.1 and 7.2.1 of TR 23.700-04 v1.2.0.

Additionally, SA2 would like to ask for RAN/RAN2 to provide feedback on whether Option 2 UP is selected.

Observation 3: For UE-side data collection, SA2 had finished their study with some alternatives in B5G R20.

As for SA5, based on SA5 SID work plan in B5G R20, SA5 SID will be concluded in Jun 2026.

Observation 4: For UE-side data collection, SA5 B5G R20 SID will be concluded in Jun 2026.

Meanwhile 6G R20 study on UE-side data collection also starts across SA and RAN2. There is risk of having duplicated discussion between B5G and 6G, especially considering that SA5 B5G SID is still ongoing. It will be hard to choose one solution without having the whole picture across different working groups in B5G. So, our suggestion is to let SA2/SA5 do the study in B5G, but no normative work happens in B5G. B5G study outcome can be the starting point for 6G study, it will be better if normative work discussion on UE-side data collection can be moved from B5G to 6G.

Observation 5: There is risk to have duplicated discussions between B5G and 6G if normative work discussion on UE-side data collection happens in B5G.

Based on above observations, we propose:

Proposal 3: For UE-side data collection, no normative work is recommended from RAN perspective for B5G.

# Conclusion

To draw attention from readers, allow us to repeat observations and proposals in what follows.

Inter-vendor training collaboration

Observation 1: From SA2 perspective, two ways are proposed and studied to address non-OTA dataset exchange in Direction A, Option 4-1 and model parameter exchange in Direction A, Option 3a-1.

Observation 2: From RAN4 perspective, it is feasible to support fully specified reference model for Direction C and specified reference encoder for Direction A, Option 3a-1.

Proposal 1: For Direction A, Option 3a-1, the content/format of reference encoder parameters exchange should be addressed in RAN1.

Proposal 2: To facilitate the inter-vendor training collaboration, supports both Option 4-1 (with higher priority) and Option 3a-1 under Direction A in RAN1.

Data collection

Observation 3: For UE-side data collection, SA2 had finished their study with some alternatives in B5G R20.

Observation 4: For UE-side data collection, SA5 B5G R20 SID will be concluded in Jun 2026.

Observation 5: There is risk to have duplicated discussions between B5G and 6G if normative work discussion on UE-side data collection happens in B5G.

Proposal 3: For UE-side data collection, no normative work is recommended from RAN perspective for B5G.

# References

[1] RP-252445, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2.

[2] S2-2511314, New WT for R20 5GA AIML_CN_Ph2 work for ML model parameters or dataset sharing.

[3] R4-2522434, LS on RAN check point of R20 NR_AIML_air_Ph2.

[4] RP-243316 LS on AI/ML UE sided data collection.

[5] SP-250810 Revised SID on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML) Phase 2.

[6] SP-250867 Study on AI/ML management phase 3.

[7] S2-2510958 LS on AI/ML UE sided data collection.