3GPP TSG RAN Meeting #110	                                 RP-253116

Baltimore, USA, December 8-11, 2025

Source: 	CMCC (moderator)

Title:	pCR for 38.914 on Key performance indicators for 6G

Agenda item:	8.2.1.1

Document for:	Approval

- Introduction

This contribution gives text proposal for TR 38.914 V0.2.0 [1] on key performance indicators (KPIs) based on the latest ITU working document [2].

- Text Proposal for TR 38.914

===========================Start of Changes===============================

## 5	Requirements and key technical principles

## 5.1	Key performance indicators

Editor note: More KPIs can be added

### 5.1.0	General

Editor note:	Aspects related to IMT-2030 TPRs and 3GPP RAN KPIs will be included in this section

Editor note:	The KPIs captured in RAN#109 focus on ITU IMT-2030 TPRs. The definitions are subject to revision based on further discussion and progress in ITU

Editor note:	Evaluation methodology to be added for each KPI.

This section describes the definitions and requirements of all KPIs.

The KPI targets defined in this section as well as in Report ITU-R M.[IMT-2030. TECH PERF REQ] [3] shall be supported by next generation access technologies. The evaluation methodology defined in Report ITU-R M.[IMT-2030.EVAL] [4] shall be used to evaluate the targets specified in Report ITU-R M.[IMT-2030. TECH PERF REQ] [3].

### 5.1.1	Peak data rate

[Peak data rate is the theoretical maximum [achievable] data rate under ideal conditions, which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by:

Rp = W × SEp

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands, then the total peak data rate is

R=i=1Q  Wi × SEpi

where Wi and SEpi (i = 1, …, Q) are the component bandwidths and spectral efficiencies respectively.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is TBD Gbit/s.

–	Uplink peak data rate is TBD Gbit/s.]

### 5.1.2	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is 60 bit/s/Hz.

–	Uplink peak spectral efficiency is 30 bit/s/Hz.

These values were defined assuming an antenna configuration to enable TBD spatial layers (streams) in the downlink and TBD spatial layers (streams) in the uplink.

### 5.1.3	5th percentile user data rate

The 5th percentile user data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

For the evaluation of uplink user experienced data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account.

In case of one frequency band and one layer of transmission reception points (TRxP), the downlink 5th percentile user data rate could be derived from the 5th percentile user spectral efficiency through the following equation. Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the 5th percentile user data rate, Ruser is given by:

Ruser = W × SEuser

For the downlink, in case bandwidth is aggregated across multiple bands (one or more TRxP layers), the 5th percentile user data rate will be summed over the bands.

The targets in Table 5.1.3-1 shall be fulfilled using parameters in accordance with [4].

Table 5.1.3-1: 5th percentile user data rate

| Deployment Scenario | DL 5th percentile user data rate (Mbit/s) | UL 5th percentile user data rate (Mbit/s) |
| --- | --- | --- |
| Dense Urban | 300 | 50 |

### 5.1.4	5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (Ti) denoting the number of correctly received bits of user i, Ti the time over which the data bits are received and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to the following equation:

The minimum requirements for 5th percentile user spectral efficiency for various deployment scenarios are summarized in Table 5.1.4-1.

Table 5.1.4-1: 5th percentile user spectral efficiency

| Deployment Scenario | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot | 0.9 | 0.63 |
| Dense Urban | 0.675 | 0.45 |
| Rural | 0.36 | 0.135 |

### 5.1.5	Average spectral efficiency

Average spectral efficiency (corresponds to "spectrum efficiency" in Recommendation ITU R M.2160 [2]) is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to the following equation:

The minimum requirements for average spectral efficiency for various deployment scenarios are summarized in Table 5.1.5-1.

Table 5.1.5-1: Average spectral efficiency

| Deployment Scenario | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot | 27 | 20.25 |
| Dense Urban | 23.4 | 16.2 |
| Rural | 9.9 | 4.8 |

### 5.1.6	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through the following equation:

Carea = ρ × W × SEavg

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

The target value for area traffic capacity in the downlink is 40 Mbit/s/m² for Indoor Hotspot deployment scenario.

### 5.1.7	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active [or battery efficient (e.g. inactive)] state.

The minimum requirements for user plane latency are:

-	4ms for IC

-	1ms for HRLLC

Assuming unloaded conditions for small IP packets, (e.g. 0 byte payload + IP header), for both downlink and uplink.

### 5.1.8	Control plane latency

Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state).

The minimum requirement for control plane latency is 20 ms.

### 5.1.9	Connection density

Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2).

Connection density should be achieved for a given bandwidth and a given number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability.

The minimum requirement for connection density is 106 devices per km2.

### 5.1.10	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

The minimum requirement for the reliability is 1-10-5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms in channel quality of coverage edge for the Indoor Factory , assuming small application data (e.g. 20 bytes application data + protocol overhead).

### 5.1.11	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

UE Speed Support:

0, 0-10km/hr, 10-120km/hr, 120-500km/hr.

The following classes of mobility are defined:

-	Stationary: 0 km/h

-	Pedestrian: 0 km/h to 10 km/h

-	Vehicular: 10 km/h to 120 km/h

-	High speed vehicular: 120 km/h to 500 km/h

-	Airplane: 500 km/h to 1000/1200 km/h

Table 5.1.11-1 defines the mobility classes that shall be supported.

Table 5.1.11-1: Mobility classes

|  | Indoor Hotspot | Dense Urban | Rural |
| --- | --- | --- | --- |
| Mobility classes supported | Stationary, Pedestrian | Stationary, Pedestrian, Vehicular | Pedestrian, Vehicular, [High speed vehicular] |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 5.1.11-2. This assumes the user is moving at the maximum speed in that mobility class in each of the deployment scenarios.

Table 5.1.11-2: Traffic channel link data rates normalized by bandwidth

| Deployment Scenario | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot | 2.25 | 10 |
| Dense Urban | 1.68 | 30 |
| Rural | 0.88~1.2 | 120 |
|  | 0.495~0.675 | 500 |

Note: For each mobility value different normalized traffic channel link data rate should be defined.

These values were defined assuming an antenna configuration as described in Report ITU-R M.[IMT-2030.EVAL] [4].

### 5.1.12	Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a UE cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the UE and the radio access network.

Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. The results can report the Mobility interruption time in other scenarios, where applicable.

### 5.1.13	Bandwidth

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the 6GR is defined for the purpose of IMT-2030 evaluation.

The requirement for bandwidth is at least 400 MHz.

### 5.1.14	Positioning

Positioning is the ability to estimate the position of connected UEs.

The positioning accuracy is the 90% point of the CDF of the device positioning error for [both] horizontal [and vertical] direction[s].

Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis). [Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude)].

The minimum requirements for the device positioning accuracy for various deployment scenarios are summarized in Table 5.1.14-1.

Table 5.1.14-1: Positioning accuracy

| Deployment Scenario | Horizontal Accuracy | [Vertical Accuracy] |
| --- | --- | --- |
| Indoor Factory |  |  |
| Urban Macro |  |  |

### 5.1.15	Energy efficiency

Editor note: The text captured in RAN#109 is based on ITU-R WP5D and is for ITU-R evaluation; 3GPP internal evaluation metrics will be captured in future meetings.

Energy efficiency is an important metric of sustainability. Energy efficiency of the 6GR and 6G RAN can be characterized by evaluating network and device energy efficiency.

Energy efficiency can be estimated as the relative energy consumption (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

6GR shall have the capability to support low relative energy consumption in the selected load cases, considering at least the unloaded (L= 0%) and loaded scenario of [0% < L ≤ 15%] / [15% < L ≤ 30%] / [30% < L ≤ 50%] / [0% < L ≤ 30%], where L is load level, i.e., network resource utilization ratio.

Results should report the evaluated energy efficiency values and the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies.

Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

### 5.1.16	Composite requirement

Editor note: The text captured in RAN#109 is based on ITU-R WP5D and is for ITU-R evaluation; 3GPP internal evaluation metrics will be captured in future meetings.

The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP.

This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface.

The minimum performance requirement of composite requirement is summarized in Table 5.1.16-1.

Table 5.1.16-1: Composite requirement

| Deployment Scenario | Data rate (Mbit/s) | Latency (ms) | Packet success probability | Number of users (/TRxP) |
| --- | --- | --- | --- | --- |
| Dense Urban | DL = 30 Mbps/UL=10 Mbps | DL=10 ms / UL=30 ms (or DL+UL total 40 ms) | 99% | 6 |

### 5.1.17	Sensing-related requirement

Sensing-related capabilities are measured in terms of the following technical performance requirements:

•	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing target when it is present, and false alarm probability is the associated probability that a sensing target is detected when no sensing target is present.

•	Horizontal/Vertical Localization Accuracy: It is defined as the absolute difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing target. The value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. Both the horizontal and vertical localization accuracy are in Cartesian coordinates.

•	Velocity Accuracy: Velocity accuracy is defined as the absolute difference between the estimated velocity and the actual velocity of the sensing target. The value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

•	[Sensing Resolution: TBD]

In the above technical performance requirements, the sensing target is a passive object, which is not connected to the radio network for communication.

The minimum requirements for Sensing-related requirements are summarized in Table 5.1.17-1.

Table 5.1.17-1: Sensing-related requirements

| Test environment | Indoor Factory | Urban Macro |
| --- | --- | --- |
| Detection Probability |  | [95%] |
| False Alarm Probability |  | [5%] |
| Horizontal Localization Accuracy |  | [5m or 10m] |
| Vertical Localization Accuracy |  |  |
| Velocity Accuracy |  |  |

### 5.1.18	AI-related requirement

AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications, including.

- 6G RAN and 6GR facilitating AI applications i.e. “Network for AI”.

- AI to improve 6G and 6G RAN performance, i.e. “AI for network”.

6GR and 6G RAN should support AI-related capabilities, which refer to the ability for the 6GR and 6G RAN to support mechanisms and/or signaling related to AI functionalities. This should include support for one or more of the following:

- Data collection

- Distributed processing

- Distributed learning

- AI model training

- AI computing

- AI model inference

- or other AI capabilities of the 6GR.

### 5.1.19	Resilience and Extended Connectivity

This requirement includes the following capabilities:

Resilience: It refers to capabilities of the networks and systems to continue operating correctly during and after a service disruption, such as the loss of primary source of power, etc. The 6GR and 6G RAN should support functionalities over the radio interface that enable continuous operation or rapid temporary restoration during and after disturbance to radio infrastructure.

Extended Connectivity: It refers to the capability to provide connectivity in areas that are currently uncovered or scarcely covered due to challenges arising from geographical conditions, such as the lack of ground infrastructure (e.g., fiber cables and power supply), where users desire access to communication services. The 6GR and 6G RAN should have functionalities that support providing geographically extended connectivity in these areas.

Functionalities related to resilience and extended connectivity may include, but are not limited to, support for connecting to one or more of the following advanced network infrastructures within the terrestrial component of 6GR and 6G RAN:

- HIBS

- Relays and repeaters

- Shared network architectures to provide redundancy and backup

- Base station onboard a vehicle, airship and/or vessel

- or other mechanisms for these capabilities using network infrastructures within the terrestrial component of 6GR and 6G RAN

===========================End of Changes===============================

- Reference

- 3GPP TR 38.914 V0.2.0, Study on 6G Scenarios and Requirements, 2025-09.

- “Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Revision 1 to Document 5D/TEMP/456-E (Chapter 5 - Annex 5.6 - Report on the 50th meeting of Working Party 5D), Geneva, 7-16 October 2025.