3GPP TSG RAN Meeting #110	                                 RP-253119

Baltimore, USA, December 8-11, 2025

Source: 	CMCC

Title:	Discussion on 3GPP KPIs on sensing

Agenda item:	8.2.1.2

Document for:	Discussion & Decision

- Introduction

In RAN#109 meeting, the following text corresponding to sensing key performance indicators (KPIs) in RP-252947 was approved for TR 38.914 [1].

Sensing-related capabilities are measured in terms of the following technical performance requirements: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error. • [Sensing Resolution: TBD]

This contribution gives proposals to define additional 3GPP KPIs on sensing.

- Metrics for detection and tracking use cases

According to RP-252947, Detection Probability and False Alarm Probability, Horizontal/Vertical Localization Accuracy and Velocity Accuracy are approved as sensing metrics. But the values for confidence level are not yet decided. According to the latest discussion in ITU-R WP5D#50, the following text was approved, where 90% percentile point is chosen for defining localization and velocity accuracy[2].

4.14 Sensing-related requirements Sensing-related capabilities are measured in terms of the following technical performance requirements: - Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing target when it is present, and false alarm probability is the [associated] probability that a sensing target is detected when no sensing target is present. - Horizontal/Vertical Localization Accuracy: It is defined as the absolute difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing target. The value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. Both the horizontal and vertical localization accuracy are in Cartesian coordinates. - Velocity Accuracy: Velocity accuracy is defined as the absolute difference between the estimated velocity and the actual velocity of the sensing target. The value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. In the above technical performance requirements, the sensing target is a passive object, which is not connected to the radio network for communication. This requirement is defined for the purpose of evaluation in the Integrated Sensing and Communication usage scenario.

Since the definitions of these three metrics in TR 38.914 are captured from ITU, we suggest keeping the same percentile point as that in ITU for accuracy definition.

Proposal 1: Use 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of localization/velocity accuracy, in the definitions of localization/velocity accuracy.

Sensing resolution describes the shortest distance between two adjacent targets within a parameter space (e.g. range and Doppler) to distinguish these two targets. Theoretically, the practical resolution is influenced by many other factors including signal-to-noise-ratio and algorithm, making it difficult for analytical calculation. Rayleigh criterion is usually adopted to approximately describe the resolution analytically.

An analytical definition for resolution, which can be calculated without simulations, is useful for system design. But there is no need to consider it as a metric. On the other hand, when the detection probability and localization accuracy fulfil the requirements, practical resolution can be partially characterized, for that successfully distinguishing two adjacent targets does improve detection probability.

Considering the above analysis and the possible difference in simulation assumptions and implemented algorithms when evaluating practical resolution, we prefer not to define sensing resolution as a sensing metric.

Proposal 2: Not to define sensing resolution as a sensing metric.

Meanwhile, in practice, sensing performance is not only determined by system design and algorithm, but the number of targets as well. For example, detection algorithms with adaptive threshold suffers from nearby target. Estimation of the threshold is significantly influenced by nearby targets or their sidelobes. The assumption of additive white Gaussian noise is broken when power from other targets falls into the region for threshold estimation. It indicates that even if the requirements of performance can be achieved when observing a small number of targets, the real performance in deployment can be much worse, which is unacceptable from operator’s position.

To address the issue, sensing target density is defined in TR 22.870 as follows[4]:

Sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing area.

From RAN perspective, this definition should also be translated into technical metrics which can be evaluated explicitly.

We recommend defining sensing capacity as a metric for sensing, which evaluates the ability to sense multiple targets with acceptable performance. And it is defined as the maximum number of the targets per sector when sensing results of all targets in observation zone fulfil QoS requirements with 95% probability. The QoS requirements are defined by other KPIs (except sensing capacity itself) on sensing. Note that, according to this definition, non-compliance of sensing KPIs with a given low probability is allowed.

For better understanding, a simplified drop-based detection capacity simulation of 1-D CA-CFAR detection is presented in Figure 4. Detection probability of 100% is selected as the only KPI for simplicity. And the sensing capacity is about 4 per km according to the curve.

| (a) Range profile of a single drop with 3 targets | (b) Probability of detecting all targets |
| --- | --- |

Figure 1. A simplified drop-based simulation to figure out sensing capacity

The evaluation methodology for sensing capacity can be found in Annex A.

Therefore, we have the following proposal.

Proposal 3: Define sensing capacity as a sensing metric with the following definition:

- Sensing capacity is defined as the maximum number of the targets per sector when sensing results of all targets in observation zone fulfil QoS requirements with [95%] probability. The QoS requirements are defined by other KPIs (except sensing capacity itself) on sensing.

- Metrics concerning new use cases

To provide new services beyond object detection and tracking (e.g. environment reconstruction identified in TR 38.870[4]), additional metrics which are different from detection/false alarm probability, localization accuracy and velocity accuracy, are needed. However, the requirements for these new services are yet to be further studied.

Taking environment awareness/reconstruction assisting communication as an example, the reconstructed physic world or radio map serves for communication purposes. Therefore, those existing communication KPIs can be directly used to characterize the service performance.

Considering these use case related requirements for potential sensing services, we have the following proposals.

Proposal 4: RAN should consider or not preclude use case specific metrics/KPIs for potential new sensing services beyond objection and tracking.

- KPIs on sensing

In TS 22.137, KPIs related to different use cases are defined for 5G, which can be taken as a starting point for 6G KPIs on sensing[3]. Meanwhile, requirements related to UAV supervision, safe & economic UAV transportation and road digitalization are also studied in TR 22.870 for 6G sensing[4]. These requirements can also be found in Annex B for convenience.

Taking TR 22.870 as a reference while considering our deployment requirements, we suggest the KPIs on sensing as follows.

Table 1 KPIs on sensing

| Use case | Positioning Accuracy |  | Velocity Accuracy [m/s] | Max sensing service latency [ms] | Refreshing rate [Hz] | Sensing Capacity [per sector] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal [m] | Vertical [m] |  |  |  |  |  |  |
| UAV detection | 10 | 10 | 1 | 1000 | 1-10 | 40 | 1 | 1 |
| UAV tracking | 1-2 | 1-2 | 1 | 100-1000 | 1-10 | 40 | 0.1-1 | 1 |
| Automotive detection and tracking | 1 | N/A | 1 | 1000-5000 | 10 | 40 | 1 | 3 |
| Human detection | 1 | N/A | N/A | 1000 -5000 | 10 | 40 | 5 | 2 |

Proposal 5: Support the KPIs listed in Table 1 for 6G sensing.

- Conclusion

In this contribution, we provide our view on 6G sensing use cases and related KPIs.

Proposal 1: Use 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of localization/velocity accuracy, in the definitions of localization/velocity accuracy.

Proposal 2: Not to define sensing resolution as a sensing metric.

Proposal 3: Define sensing capacity as a sensing metric with the following definition:

- Sensing capacity is defined as the maximum number of the targets per sector when sensing results of all targets in observation zone fulfil QoS requirements with [95%] probability. The QoS requirements are defined by other KPIs (except sensing capacity itself) on sensing.

Proposal 4: RAN should consider or not preclude use case specific metrics/KPIs for potential new sensing services beyond objection and tracking.

Proposal 5: Support the KPIs listed in Table 1 for 6G sensing.

- Reference

- RP-252947, pCR for 38.914 on Key performance indicators for 6G, 3GPP TSG RAN Meeting#109, Beijing, China, September 15-18, 2025

- Minimum requirements related to technical performance for IMT-2030 radio interface(s), 7 Oct 2025

- 3GPP TR 22.137, "Service requirements for Integrated Sensing and Communication", 2024

- 3GPP TR 22.870, "Study on 6G Use Cases and Service Requirements", 2025

- Annex A (informative)

Evaluation methodology for sensing capacity

| Step 1 | Initialize evaluation parameters. Decide the number of targets dropped. |
| --- | --- |
| Step 2 | Implement Mont-Carlo system-level simulations with the given number of targets. Generate sensing transmit signal. Obtain sensing received signal and perform signal processing to estimate the interested parameters of all targets. |
| Step 3 | Count the cases that all targets fulfil QoS requirements (depending on use case). Calculate the probability that all targets fulfil QoS requirements. |
| Step 4 | If necessary, update the number of targets dropped and repeat Step.1 to Step.3, until the probability that all targets fulfil QoS requirements equalling to 95%. |

- Annex B (informative)

Requirements from TR 22.870

Table 7.4.6-1:	Performance requirements of sensing results for low-altitude UAV supervision

| Scenario |  | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate (Hz) | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s] |  |  |  |  |
| Low-altitude UAV supervision | UAV intrusion detection | Outdoor | ≥95 | ≤10 | ≤10 | N/A | N/A | [10] | [≥5] | [≤1000] | [≤1] | [≤5] | [≤5] |
|  | UAV trajectory tracking | Outdoor | ≥90 | 1-2 | 1-2 | 3-5 | 3-5 | N/A | N/A | 100~1000 | ≤1 | ≤5 | ≤5 |

Table 7.6.6-1:  Performance requirements for road digitalization

| Scenario | Service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Sensing target density （note 3） | Sensing target Velocity | Max sensing service latency [s] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |
| Road digitalization | Outdoor | 95 | 0.5 (vehicle) 1（pedestrian） | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | ≤ 750 vehicles Per 1000 m x 24 m ≤100 pedestrian per 1000 m x 24 m (note 1) | Static/moving, up to 5 km/h | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
|  | Outdoor | 95 | 3 | N/A | 3 | N/A | 1 | 1 m/s | ≤85 vehicles per 1000 m x 24 m (note 2) | Moving, up to 120 km/h | 1~5 | ≤ 0.1 | ≤ 5 | ≤ 5 |
| NOTE 1: One crossroad area is approximated by 1000m dual three-lane carriageway (24 m), by assuming 250 m per direction at the crossroad. NOTE 2: Based on an assumption of dual three-lane carriageway (1000 m x 24 m). NOTE 3: The definition of sensing target density is described in Clause 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.17.6-1:	Performance requirements for Safe and Economic UAV transport

| Scenario | Sensing service availability [%] | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing Resolution |  | Max sensing service latency [ms] | Refreshing Rate [s] | Missed Detection [%] | False Alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Safe and economic UAV transport | 99.999 | 99.99 | ≤ 50 | 50 | 5 | 5 | 10 | 0.5 | ≤ 100 | 0.1 - 1 | 0.1% (notes 2, 3) | 1% (note 2) |
| NOTE 1: The listed positioning accuracy and sensing resolution are required for the detection of proximal objects within a safety area of the UAV and validate the reported location of the UAV itself. NOTE 2: Safety impact of missed detection is substantially higher compared to the false alarm, here fore the Missed Detection is assigned a more stringent value compared to the False Alarm. NOTE 3: To enable safe operation of BVLOS UAV flights in mixed airspace (manned & unmanned) and in high risk areas (e.g. above cities) the value for missed detection of 0.1% corresponds to the lower value of the missed detection range specified in Table 6.2-1 of TS 22.137 [6] for sensing service category 2. |  |  |  |  |  |  |  |  |  |  |  |  |