3GPP TSG RAN Meeting #110	RP-253125

Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.3.2

Source:	CMCC

Title:	Views on diverse device types

Document for:	Discussion

- Introduction

During RAN#109, the number of device types supported by 6GR and the parameters/factors constitute to device types were discussed, with the following agreements[1].

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, at least form factor limitations is considered. Sstudy further: - The parameters related to form factor limitations, e.g., the number of Tx antennas/chains, Rx antennas/chains, power classes, etc. - Other Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

In this contribution, we will provide our views on devices types supported by 6GR.

- Discussion on device types

  - Supported device types

In NR day-1 design, standardization prioritized eMBB and URLLC requirements, while mMTC requirement continued to utilize NB-IoT and eMTC designs. Release 17 introduced RedCap device types that are compatible with NR's idle state design. Aside from a RedCap-specific initial BWP configuration, which is introduced in case that initial BWP BW of eMBB exceeds maximum BW of RedCap, most other idle state design could be largely reused. Since UE-specific configuration can be applied after connection establishment, the impact on connected state design is minimal.

For 6G, definition of the lowest-tier device type has big impact on air interface design. With NB-IoT and eMTC having been deployed for nearly a decade, and considering regular technology evolution plus potential 4G spectrum refarming in the 6G era, 6GR should support a device type offering comparable battery life and cost to existing LPWA technologies. Therefore, the lowest-tier 6G device type should feature low power consumption and wide-area coverage for massive communication services. As discussed in our companion contribution [2], requirements for this device type include comparable cost and battery life target as NB-IoT/eMTC. Meeting these requirements imposes design constraints on parameters/factors determination of this device type. For instance, a maximum bandwidth limited to 5 MHz would impact 6GR's unified air interface architecture. Therefore, definition of this device type and determination of corresponding parameters should be done as early as possible.

Regarding high-end IoT device types, with 5G RedCap deployment only beginning and the ecosystem still maturing, whether a comparable device type should be introduced as a replacement in 6G day-1 requires further discussion. Moreover, since 6G aims for a scalable design compatible with the lowest-tier device type, supporting this high-end IoT device type can be naturally realized. Thus, the necessity of define a high-end IoT device type is not clear now.

And another device type which is clear now is eMBB type UE, i.e. smart phones, which requires high throughput for supporting immersive communication. The 6G air interface design should prioritize eMBB requirements, and parameter selection for lowest-tier devices must not compromise eMBB performance.

Other device types, such as low-end eMBB, FWA/CPE, and XR, etc., can also be supported via scalable design. For example, even low-end eMBB devices (comparable to 5G normal UEs) can support Carrier Aggregation (CA) to achieve higher throughput. And for FWA/CPE devices, they can optionally support larger bandwidth, large numbers of transmit/receive antennas and higher-order modulation to achieve higher throughput. So, the maximum supported single carrier bandwidth seems does not fully present the device capability. And in the future, other device types may be introduced, as long as the design is forward compatible, they can be supported, so the definition of such device types is not urgent. And we should identify the impacts of device type definitions on 6G design first. And RAN1 is proceeding to find the common design for different device types.

Observation：The lowest-tier device type has the greatest impact on 6GR's scalable design, and the parameters should be decided early, without compromising eMBB performance. Others High-end IoT, low-end eMBB, FWA/CPE, XR, and other device types can be accommodated through scalable design.

Proposal 1：Device type of 6G at least includes, a lowest-tier device type with comparable cost, coverage, and battery life requirement as NB-IoT/eMTC, and a normal eMBB device type supporting immersive communication.

Proposal 2: For other device types than lowest-tier device type and normal eMBB device type, they can be supported by 6GR scalable design, how they affect 6GR design should be firstly identified.

  - Parameters for device types

Parameters should be distinguished from requirements. For example, coverage performance, battery life, latency requirements, and connection density all fall under requirements. While device type parameters are the specific capability areas needed to satisfy these requirements. Therefore, we recommend that at least the following physical parameters can be considered for device type characteristics,

  - Maximum UE bandwidth

  - Tx/Rx antenna number and maximum MIMO layers

  - Peak data rate

  - Full-duplex / Half-duplex

  - Modulation order

For example, for lowest-tier device type, the required minimum number of UE Tx/Rx antenna determines not only the number of RF chain which affect device cost, but also the baseband processing complexity. It further impacts the UE power consumption heavily. An antenna number of 1 is recommended for both UE transmitter and receiver.

For Maximum UE bandwidth and Peak data rate, as we analyzed in TR38.875[3], the reduction of them can contribute to cost reduction. And another important effect is power consumption. The clock frequency of baseband processing depends on the supported channel bandwidth and peak data rate. The sampling clock frequency is proportional to the channel bandwidth, considering proper oversampling rate for good link performance. Furthermore, the clock frequency of a certain blocks (e.g., encoder and decoder) is determined by peak data rate. To achieve comparable power consumption to the existing LPWA technologies, a maximum channel bandwidth of e.g. 5 MHz and peak data rate of e.g. a few Mbps is suggested. And this peak data rate can satisfy the reference bit rate of Industrial wireless sensors as stated in RedCap SI, i.e. “The reference bit rate is less than 2 Mbps (potentially asymmetric e.g. UL heavy traffic) for all use cases and the device.”, and also Video Surveillance use case, i.e., reference economic video bitrate would be 2-4 Mbps.

And for half-duplex, as analyzed in TR38.875, “the main contributor of the cost reduction is the duplexer/switch block. Depending on the implementation, as indicated by some sourcing companies, removing the duplexer may also reduce the insertion loss in both the Rx and Tx chains and as a result, the PA power can be reduced, and the LNA sensitivity requirement can be relaxed which allows for potential UE complexity reduction.”, so it contributes to both cost reduction and power saving. Half-duplex can be supported by lowest-tier device type.

And the modulation order is related to peak data rate, at least 16QAM can be considered.

Proposal 3: at least the following parameters can be considered for device type definition,

  - Maximum UE bandwidth

  - Tx/Rx antenna number and maximum MIMO layers

  - Peak data rate

  - Full-duplex / Half-duplex

  - Modulation order

Proposal 4: The following parameter values can be considered for lowest-tier device type,

  - Maximum UE bandwidth: 5MHz for FDD

  - Tx/Rx antenna number and maximum MIMO layers: 1Tx, 1Rx, 1layer

  - Peak data rate: ~5Mbps

  - Full-duplex / Half-duplex: Half-duplex is supported and full duplex FDD and TDD can be device implementation

  - Modulation order: 16QAM

  - Minimum spectrum allocation and smallest maximum UE bandwidth

During RAN1#123 meeting, the following agreement was made,

Agreement If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR, - Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable - Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations Agreement - For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least - Overall device complexity - Overall system performance impact - Energy efficiency for both BS and UE - Aim at a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority

For the smallest maximum supported RF and BB UE BW, considering the power consumption and cost reduction analyzed in section 2.2, we propose Opt2: 5MHz in RAN1 agreement. It can support the same PSS/SSS length as NR, i.e. 12RB with a bandwidth of 2.16MHz and 4.32MHz for 15KHz and 30KHz SCS respectively. For TDD 30KHz SCS, since years of battery life is required for lowest tier device type, the introduction of larger bandwidth should not significantly reduce battery life. Whether 10MHz bandwidth is suppored for TDD can be further discussed.

For the minimum spectrum allocation, 3MHz may exist in some countries or regions, but if “Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations” is supported, the SSB design will include more symbols since the frequency bandwidth is small, and also the common channel will last a longer time, which will bring latency impact on mobile broadband service. So we propose option 1, i.e. Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

Proposal 5: The smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, is Opt2: 5MHz at least for FDD.

Proposal 6: If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR, support

- Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

- Text proposal

Based on above analysis, it is proposed to capture following text proposal.

Proposal 7: It’s recommended to capture the following TP in TR 38.914.

< Unchanged text omitted >

5.3.2	Diverse device types

- 6G RAT shall define a limited set of device type, e.g., lowest tier UE, normal UE.

- 6G UE shall support 5 MHz as the minimum supported bandwidth for FDD.

< Unchanged text omitted >

- Conclusion

This contribution discusses the device type related issue, and provides the following proposals:

Observation：The lowest-tier device type has the greatest impact on 6GR's scalable design, and the parameters should be decided early, without compromising eMBB performance. Others High-end IoT, low-end eMBB, FWA/CPE, XR, and other device types can be accommodated through scalable design.

Proposal 1：Device type of 6G at least includes, a lowest-tier device type with comparable cost, coverage, and battery life requirement as NB-IoT/eMTC, and a normal eMBB device type supporting immersive communication.

Proposal 2: For other device types than lowest-tier device type and normal eMBB device type, they can be supported by 6GR scalable design, how they affect 6GR design should be firstly identified.

Proposal 3: at least the following parameters can be considered for device type definition,

  - Maximum UE bandwidth

  - Tx/Rx antenna number and maximum MIMO layers

  - Peak data rate

  - Full-duplex / Half-duplex

  - Modulation order

Proposal 4: The following parameter values can be considered for lowest-tier device type,

  - Maximum UE bandwidth: 5MHz for FDD

  - Tx/Rx antenna number and maximum MIMO layers: 1Tx, 1Rx, 1layer

  - Peak data rate: ~5Mbps

  - Full-duplex / Half-duplex: Half-duplex is supported and full duplex FDD and TDD can be device implementation

  - Modulation order: 16QAM

Proposal 5: The smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, is Opt2: 5MHz at least for FDD.

Proposal 6: If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR, support

- Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable

Proposal 7: It’s recommended to capture the following TP in TR 38.914.

< Unchanged text omitted >

5.3.2	Diverse device types

- 6G RAT shall define a limited set of device type, e.g., lowest tier UE, normal UE.

- 6G UE shall support 5 MHz as the minimum supported bandwidth for FDD.

< Unchanged text omitted >

- References

- RP-252873, Summary of 6G Diverse Device Types (including Minimum UE Bandwidth), Apple Inc. (Moderator).

- RP-253203, Requirements for massive communication in 6G, CMCC.

- 3GPP TR 38.875 V17.0.0, 2021.03.