# 3GPP TSG RAN meeting #110                                         RP-253134

Baltimore, USA, December 8-11th, 2025

Source: 			CMCC

Title:  		    Discussion on 6G Voice

Agenda item:	8.2.1.4

Document for: 	Discussion

Introduction

Voice services are the basic services of each generation of mobile communications. Meanwhile, in the evolution of voice technology, it could be continuously integrated with other technologies, such as AI. As operator, China Mobile provides users with a new generation of call products named “new call” based on intelligent network. Leveraging AI algorithms and big data, this service upgrades traditional voice and video call services, bringing the UEs a new experience characterized by stability, high definition, visual interaction, and intelligent efficiency. In this contribution, we will share our views on voice in 6G.

Discussion

At 6G day1, we should still start by providing voice services, building a low-latency, high-definition calling experience and evolving toward immersive voice and 3D calls. To this end, 6G should make carrier-grade voice a native, fundamental capability, integrate regulatory functions such as emergency call and lawful interception (LI) from day one, and proactively introduce new legal requirements like digital identity, trusted positioning, and immersive emergency communications.

Proposal 1: Support Voice over 6G (Vo6G) from Day 1 together with emergency call and lawful interception (LI).

Constrained by UE capabilities and the initial immaturity of SA deployments, operators widely adopted EPS Fallback during 5G’s commercial launch, routing 5G voice traffic back to 4G. The fallback mechanisms are implemented with redirection-based or handover-based fallback. For the redirection-based fallback, the gNB release the UE to LTE frequency with RRCRelease message. The UE selects target LTE cell and performs random access. For the handover-based fallback, the gNB sends handover command with RRCReconfiguration message with LTE frequency and PCI. The UE performs random access towards the target cell. Even after VoNR became fully available, the large installed base of VoLTE-only devices compelled operators to keep EPS Fallback as a safety net. As a result, today’s networks operate a dual-track voice architecture—VoNR alongside EPS Fallback—with both paths anchored in a unified IMS.

Network deployment typically expands from densely populated urban areas toward suburban and rural areas, resulting in discontinuous 6G network coverage during the early stages of 6G. Due to the fact that voice is an optional feature, there are also differences in UE capabilities, such as some UEs will support 6G-native voice plus 5G fallback, while others will be limited to 5G/VoNR only. Consequently, voice services will face the same fragmentation dilemma that plagued 5G’s early years. Given the extensive deployments of 5G networks, we recommend introducing a “5G Fallback” mechanism for 6G: whenever a UE or the network does not yet support native 6G voice, the call can seamlessly fallback to 5G/VoNR.

Observation：Early stages of 6G will face discontinuous 6G network coverage.

Proposal 2: Support of voice fallback and handover to 5G.

Furthermore, to support the real-time communication requirement anywhere, it is also important to support voice via NTN including both GEO and LEO to provide voice service when there is less TN coverage. In addition, LPWA devices should support voice to provide voice service for diverse IoTs.

Proposal 3: 6G NTN and LPWA support voice.

A text proposal is also provided in the annex and suggested to be captured to the TR 38.914.

Proposal 4: Capture the text proposal in section 4 to TR 38.914.

Conclusion

In this contribution we share our views on the requirement of 6G voice. Observation and proposals are as follows:

Observation：Early stages of 6G will face discontinuous 6G network coverage.

Proposal 1: Support Voice over 6G (Vo6G) from Day 1 together with emergency call and lawful interception (LI).

Proposal 2: Support of voice fallback and handover to 5G.

Proposal 3: 6G NTN and LPWA support voice.

Proposal 4: Capture the text proposal in section 4 to TR 38.914.

Text proposal

## 5.4.x	Voice in 6G

The 6G radio and 6G RAN architecture shall support Voice in 6G with the necessary support to ensure that emergency services like emergency call and lawful interception (LI). Voice fallback and handover to 5G are supported for Voice in 6G. Furthermore, NTN and LPWA support Voice in 6G.