3GPP TSG-RAN Meeting #110	RP-253157

Baltimore, US, December 08-11, 2025

Agenda item	: 8.2.1.0

Source	: Jio Platforms Limited

Title	: High Rise Building Deployment in 6G deployment scenarios

Document for	: Discussion

# Introduction

6G deployment scenarios were discussed in RAN#108 and RAN#109, while agreements were captured in TR 38.914 [1]. To our opinion there are missing key aspects of 6G deployment scenarios in the TR. This contribution provides Jio Platforms view on 6G deployment scenarios.

# Discussion

- With the rapid urbanization and increasing number of high-rise residential and commercial buildings, ensuring consistent cellular coverage and quality of service (QoS) across all floors has become a major challenge.

- Existing cellular network deployments are primarily optimized for horizontal (2D) coverage and struggle to provide adequate vertical (3D) coverage in tall buildings

## Current Deployment Scenario

In the current deployment scenarios macro gNodeBs are typically deployed at rooftop or tower-top heights optimized for street-level and mid-rise coverage. In such scenarios signal propagation is primarily designed for outdoor users, with limited indoor penetration since building materials (reinforced concrete, glass, metallic coatings) cause high penetration losses and that prevents a good indoor coverage from the outdoor small cell.

Observation 1: The current Macro cell-based deployment with primary focus on beam forming in horizontal plane architecture is insufficient to provide reliable and uniform 5G coverage and capacity across all floors of high-rise buildings.

To anticipate such coverage restrictions, small indoor cells are deployed selectively, often leading to inconsistent or overlapping coverage zones, while repeaters are only effective for FDD bands and not being success in the TDD bands because of the requirements of TDD synchronization.

Observation 2: The current Macro cell-based deployment with the assistance of indoor an/or repeaters based deployment seems to be insufficient and expensive, non-cost-effective solution for high-rise buildings.

## High Rise Building Deployment Problems

There are many noticed and encountered problematic scenarios, as:

- Poor Coverage at Higher Floors:

Weak received signal strength (RSRP/RSRQ) and high path loss at upper floors due to antenna down tilt and limited vertical beam coverage.

- High Uplink Interference:

UEs at upper floors may connect to distant overshooting cells, increasing uplink interference and reducing overall cell capacity.

- Frequent Handovers and Ping-Pong Events:

Signal fluctuation between nearby cells leads to unstable connections and increased signalling load.

- Inconsistent User Experience:

Users experience degraded throughput, call drops, and latency variation depending on floor level and location within the building.

- Limited Indoor Solutions:

Absence of proper indoor systems (IBS, DAS, small cells) results in dependency on outdoor macro signals, worsening indoor KPIs.

Observation 3: The lack of optimized 3D radio planning, inadequate indoor coverage solutions, and poor vertical beam management result in degraded signal quality, increased interference, and inconsistent user experience in multi-story environments.

# Proposed deployment Scenario

To better characterize and optimize performance in such scenarios, it is proposed that TR 38.914 include explicit guidance on solution approaches tailored to multi-floor user distributions and dense vertical traffic patterns.

Proposal 1: RAN Plenary to study to include in TR 38.914 an explicit guidance on deployment approaches tailored to high-rise building multi-floor user distributions.

Considering these solutions in TR 38.914 would provide operators with consistent guidelines for high-rise deployments, which are increasingly common in dense urban 6G environments and require dedicated modelling beyond conventional 2D assumptions.

# Conclusion

In this contribution, the following observations and proposals are provided:

Observation 1: The current Macro cell-based deployment with primary focus on beam forming in horizontal plane architecture is insufficient to provide reliable and uniform 5G coverage and capacity across all floors of high-rise buildings.

Observation 2: The current Macro cell-based deployment with the assistance of indoor an/or repeaters-based deployment seems to be insufficient and expensive, non-cost-effective solution for high-rise buildings.

Observation 3: The lack of optimized 3D radio planning, inadequate indoor coverage solutions, and poor vertical beam management result in degraded signal quality, increased interference, and inconsistent user experience in multi-story environments.

Proposal 1: RAN Plenary to study to include in TR 38.914 an explicit guidance on deployment approaches tailored to high-rise building multi-floor user distributions.

# References

3GPP TR 38.914 V0.2.0: Study on 6G Scenarios and Requirements

# Annex

In this contribution we propose adding the text proposal in section 4 of 3GPP TR 38.914

>>>>>>>>>>>>>>>>>>>>>>>>>>>>> skip non changed text <<<<<<<<<<<<<<<<<<<<<<<<

4. Deployment scenarios

Editor note: Sensing related aspects will be included in section 4

Editor note: More deployment scenarios can be added

High-level descriptions on deployment scenarios including carrier frequency, aggregated system bandwidth, network layout / ISD/ Orbit, BS / UE antenna elements, UE distribution / speed and service profile are proposed in this TR. It is assumed that more detailed attributes and simulation parameters, for example, the channel model, BS / UE Tx power, number of antenna ports, etc. should be defined in the new RAT study item.

…..

>>>>>>>>>>>>>>>>>>>>>>>>>>>>> skip unrelated text <<<<<<<<<<<<<<<<<<<<<<<<

…..

4.13 High Rise Building Coverage scenario

High-rise urban environments introduce unique vertical-dimensional challenges that are not fully addressed in current NR system design assumptions.

Enhanced 3D beamforming and vertical sectorization should be considered, enabling finer vertical beam granularity and dynamic floor-level coverage shaping. This includes support for adaptive vertical cell splitting (e.g., low/mid/high floors) and AI-assisted elevation-aware beam design.

A multi-layer vertical coverage architecture is proposed, defining separate coverage layers (street-level, mid-building, and upper-floor/rooftop) coordinated through multi-TRP and CoMP mechanisms.

Some of its attributes are listed in Table 4.13.1.

Table 4.13.1: Attributes for High Rise Building Coverage scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz |
|  | Around 7 GHz |
|  | Around 4GHz+Around 7GHz |
|  | Around 30 GHz |
| Aggregated system bandwidth | Around 4 GHz: Up to [200] MHz (DL+UL) |
|  | Around 7 GHz: Up to [400] MHz (DL+UL) |
|  | Around 30 GHz: [1] GHz (DL+UL) |
| Layout | Single layer: |
|  | - Indoor floor (Open office) |
|  | Note: Co-site & single-layer deployment for Around 4GHz+Around 7GHz is considered. |
| ISD | 20m for around 30GHz |
|  | TBD on other carrier frequencies |
|  | (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements | Around 7GHz: Up to 1024 Tx and Rx antenna elements |
|  | Around 30GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements | Around 4GHz: Up to 8 Tx and Rx antenna elements |
|  | Around 7GHz: Up to 8 Tx and Rx antenna elements |
|  | Around 30GHz: Up to 32 Tx and Rx antenna elements |
| Multi-TRP operation | Around 4GHz and 7GHz: single TRP, or coherent joint transmission multi-TRP |
|  | Around 30GHz: single TRP, or non-coherent joint transmission multi-TRP |
|  | Note: TRP(s) can be all DL+UL TRP, or randomly selected from {DL+UL TRP, UL-only TRP}. |
| User distribution and UE speed | 100% Indoor, 3km/h, |
|  | [10] users per TRxP |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>> end of proposed changes <<<<<<<<<<<<<<<<<<<<<<<<