3GPP TSG RAN Meeting #110	RP-253158

# Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.0

Source:	Samsung

Title:	Discussion on UE antenna configurations

Document for:	Discussion and Decision

1	Introduction

With the expanding frequency range and diverse deployment scenarios expected for 6G, it becomes increasingly important to define baseline UE antenna configurations that ensure consistent evaluations and aligned expectations across vendors and operators. Unlike 5G, 6G use cases (e.g., ubiquitous coverage at sub-7 GHz, enhanced uplink, sensing-assisted services, and extended new FR operation) introduce requirements for more unified and capability-appropriate antenna configurations.

The proposed antenna configurations aim to (i) reflect realistic industrial design constraints for handheld and non-handheld form factors, (ii) provide minimum performance guarantees across low-, mid-, and high-band frequency ranges, and (iii) allow scalability for advanced multi-antenna or multi-panel implementations at higher frequencies such as around 30 GHz. Establishing such UE antenna configurations at this stage is essential for harmonized link budget assumptions, evaluations, and physical-layer feature designs throughout the 6G study.

2	Discussion

The proposed UE antenna configurations are designed to balance practical device constraints with performance expectations across different spectrum ranges. At lower frequencies (e.g., around 700 MHz and 2 GHz), physical device size limits the number of implementable antennas, leading to 1Tx/2Rx as a realistic minimum configuration for handheld devices. At mid-band frequencies (e.g., around 4–7 GHz), the form-factor allows additional antenna elements, enabling up to 4Rx configurations as minimum requirement to support enhanced MIMO, improved downlink coverage, and uplink beamforming gains. These configurations are aligned with expected 6G coverage and capacity targets without imposing excessive hardware cost or power consumption on commercial smartphones.

For high-band operation (e.g., around 30 GHz), multi-panel UE architectures become a practical necessity to achieve adequate beamforming gain and overcome increased propagation loss. The proposed minimum (6Tx/6Rx) and maximum configurations reflect feasible designs derived from current mmWave UE experience, while extending the capability to support advanced 6G beam management and mobility requirements.

For non-handheld UEs such as FWA or CPE devices, relaxed form-factor constraints justify higher Tx/Rx configurations, particularly in mid-band and high-band frequencies. For IoT-type devices, the proposal reflects strict cost, size, and power limitations, resulting in simplified 1Tx/1–2Rx configurations.

Constraint for handheld (e.g., smartphone) device

When considering UE antenna configurations for handheld devices, it is critical that 3GPP avoids adopting assumptions that are unrealistic with respect to actual device industrial design and hardware integration limits. Modern smartphones incorporate multiple coexisting radio subsystems—such as Wi-Fi (2.4/5/6 GHz), GNSS (GPS/Galileo/GLONASS), Bluetooth, NFC, UWB, and multiple 5G/6G cellular bands—within an extremely limited physical volume. This results in strong constraints on antenna placement, spacing, and achievable isolation.

Furthermore, commercial handheld products must also support complex band combinations and carrier aggregation configurations, which impose additional requirements on RF front-end design, tunable matching networks, and antenna multiplexing structures. These coexistence requirements significantly restrict the number of cellular Tx/Rx antenna elements that can be simultaneously deployed without causing degradation in performance, increased mutual coupling, or excessive power consumption.

For this reason, assumptions such as ≥4Tx at sub-7 GHz or overly dense Rx arrays in smartphone form factors do not represent feasible commercial designs and should not be adopted for baseline 6G evaluations. Over-ambitious antenna configurations would lead to unrealistic link budget assumptions, unachievable beamforming gains, and inconsistent system-level performance analysis across vendors.

Therefore, for handheld UEs, 3GPP should explicitly ensure that any antenna configuration defined as a minimum or recommended baseline is grounded in realistic smartphone industrial design limits, RF coexistence constraints, and proven multi-radio integration feasibility. This principle is essential to maintain credibility in system-level studies and to ensure that 6G performance targets remain aligned with deployable commercial devices.

Constraint for Non-Handheld Devices (e.g., FWA / CPE)

Although non-handheld devices offer greater physical volume and fewer industrial-design constraints than smartphones, it is still important that 3GPP avoids assuming excessively large antenna arrays for these devices. In practice, FWA/CPE products must meet strict cost targets to remain competitive in the consumer market, where even modest increases in antenna count lead to substantial increases in RF front-end components (PA/LNA chains, switches, filters), PCB complexity, calibration requirements, and thermal management overhead. Moreover, mass-market FWA devices typically operate under tight cost budgets and are subject to aggressive pricing constraints. Therefore, while some extension beyond handheld UE is feasible, configurations with overly high numbers of Tx/Rx antenna elements may not be commercially viable and should not be used as baseline evaluation assumptions for 6G studies.

Constraint for IoT Devices (e.g., Massive IoT)

For IoT UEs, the primary design drivers are lowest cost, minimal power consumption, and extremely compact form factors. These devices frequently utilize single-chip RF/BB solutions with very limited antenna interfaces, rely on low-cost PCB antennas, and are optimized for minimal feature sets to achieve long battery life and ultra-low manufacturing costs. As a result, IoT UEs cannot support multi-element antenna configurations beyond 1Tx/1–2Rx without fundamentally breaking cost, size, and power budgets. Overstating the antenna capability of IoT devices would invalidate system-level assumptions related to coverage, link budget, RSRP measurement performance, and device availability expectations. For this category, 3GPP should strictly maintain minimal antenna configurations that align with realistic ultra-low-cost IoT hardware designs.

Proposal#1: the following UE antenna configuration should be supported for all deployment scenarios

| UE antenna elements | Handheld device (e.g., Smartphone) • Around 700 GHz:1 Tx and 2 Rx antenna elements as minimum requirement and up to 1 Tx and 4 Rx antenna elements • Around 2 GHz: 1 Tx and 2 Rx antenna elements as minimum requirement and up to 2 Tx and 4 Rx antenna elements • Around 4 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 2 Tx and 4 Rx antenna elements • Around 7 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 2 Tx and 6 Rx antenna elements • Around 30 GHz: 6 Tx and 6 Rx antenna elements as minimum requirement and up to 8 Tx and 8 Rx antenna elements Non-handheld device (e.g., FWA) • Around 700 GHz:1 Tx and 2 Rx antenna elements as minimum requirement and up to 4 Tx and 4 Rx antenna elements • Around 2 GHz: 1 Tx and 2 Rx antenna elements as minimum requirement and up to 4 Tx and 4 Rx antenna elements • Around 4 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 4 Tx and 4 Rx antenna elements • Around 7 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 4 Tx and 8 Rx antenna elements • Around 30 GHz: 6 Tx and 6 Rx antenna elements as minimum requirement and up to 16 Tx and 16 Rx antenna element Iot device (e.g., massive IoT) :1 Tx and 1 Rx antenna elements as minimum requirement and up to 1 Tx and 2 Rx antenna elements Note: Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. (Except around 30 GHz) Note: For around 30 GHz, with multi-panel UE, it is the number of simultaneously used antenna elements. |
| --- | --- |

3	Conclusions and Proposals

3GPP should adopt the proposed UE antenna configurations as realistic and commercially deployable baselines across handheld, FWA, and IoT device categories for all 6G evaluation scenarios.

Proposal#1: the following UE antenna configuration should be supported for all deployment scenarios

| UE antenna elements | Handheld device (e.g., Smartphone) • Around 700 GHz:1 Tx and 2 Rx antenna elements as minimum requirement and up to 1 Tx and 4 Rx antenna elements • Around 2 GHz: 1 Tx and 2 Rx antenna elements as minimum requirement and up to 2 Tx and 4 Rx antenna elements • Around 4 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 2 Tx and 4 Rx antenna elements • Around 7 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 2 Tx and 6 Rx antenna elements • Around 30 GHz: 6 Tx and 6 Rx antenna elements as minimum requirement and up to 8 Tx and 8 Rx antenna elements Non-handheld device (e.g., FWA) • Around 700 GHz:1 Tx and 2 Rx antenna elements as minimum requirement and up to 4 Tx and 4 Rx antenna elements • Around 2 GHz: 1 Tx and 2 Rx antenna elements as minimum requirement and up to 4 Tx and 4 Rx antenna elements • Around 4 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 4 Tx and 4 Rx antenna elements • Around 7 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 4 Tx and 8 Rx antenna elements • Around 30 GHz: 6 Tx and 6 Rx antenna elements as minimum requirement and up to 16 Tx and 16 Rx antenna element Iot device (e.g., massive IoT) :1 Tx and 1 Rx antenna elements as minimum requirement and up to 1 Tx and 2 Rx antenna elements Note: Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. (Except around 30 GHz) Note: For around 30 GHz, with multi-panel UE, it is the number of simultaneously used antenna elements. |
| --- | --- |