3GPP TSG RAN Meeting #110	RP-253162

# Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.3.2

Source:	Samsung

Title:	Discussion on Device Types

Document for:	Discussion and Decision

1	Introduction

As part of the ongoing 6G study, TSG RAN agreed that a clear definition of 6G UE device types is necessary to support scalable device types.

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Proposal 3: To investigate further: in RAN ONLY - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation - Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains, Number of Rx antennas/chains, Power classes, Maximum UE bandwidth (DL/UL), Peak data rate (DL/UL), Maximum MIMO layers (DL/UL) - Duplex mode, Max modulation order (DL/UL), CA/spectrum aggregation (DL/UL), UE processing capabilities, Coverage, Energy efficiency - Mobility/speed, Sensing, AI - Note: some of the above parameters/factors may be related with form factor - Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

Following this direction, both RAN1 and RAN4 have initiated discussions on scalable design characterizations relevant to 6GR, including antenna configurations, RF performance expectations, form-factor–driven constraints, and device-class differentiation for new 6G use cases. RAN1 has primarily focused on physical-layer aspects such as common features for all device types, while RAN4 has examined implementation feasibility with consideration on performance and complexity trade-offs, requirements, and more. These discussions highlight the need for a consolidated and coherent framework for 6G device types, which motivates the proposals presented in this contribution.

Agreement (RAN1#123) - For scalable 6GR design for diverse device types, RAN1 can at least consider the following, targeting applicable to all 6G device types, - Basic initial access procedures from RAN1 perspective - Other PHY features after initial access procedure, e.g., Other DL/UL control, scheduling/HARQ - Coverage features to meet the identified coverage target - Energy saving both at BS and UE sides - MRSS - Note: whether these features are supported, mandatory or optional is separate discussion

- Recommendation from FL based on inputs for RAN4#117 - Avoid duplicating the device type categorization discussion. RAN4's focus should be on evaluating the fundamental parameters and implementation feasibility, while leaving the device type decision to RAN. - Identify the distinct implementation types that need specific RAN4 requirements, which may not directly map to the final RAN device types. - 5G NR implementation types/features with specific requirements could be considered as starting point - Concentrate studies on evaluating the implementation feasibility with consideration on performance and complexity trade-offs for the identified types with following core parameters - Number of Tx/Rx - Maximum channel bandwidth - Power class - Duplex mode - Modulation order - Others (TBD) - Note: Realistic "baseline" and "maximum" values should be considered for the applicable parameters per RAN4 identified type and frequency range. - Study the relationship of RAN discussed device types and RAN4 requirements, ensuring a scalable framework for future releases. Including but not limited to the following aspects: - The feasibility of specifying a unified requirement set per device type, or whether RAN decided device types have direct impact on RAN4 specified requirements - How to accommodate implementation-specific and feature-based requirements - How to accommodate baseline vs. maximum or mandatory vs. optional capability requirements in terms of different types - How to address the potential overlapping capabilities among different types in terms of RAN4 requirements - Collaboration with RAN: Consolidate the findings from RAN4 studies into a formal input to RAN, including: - Feasible RF/BB capability combinations for the types which require specific RAN4 requirements including baseline/mandatory and maximum/optional capability - Comprehensive summary of implementation restrictions and trade-offs.

2	Discussion

As highlighted in the ongoing RAN and WG discussions, directly defining a large number of detailed 6G device types based on individual capability parameters (e.g., antenna configurations, maximum bandwidth, mobility range, feature sets) is neither scalable nor desirable at this stage of the 6G study. The extensive list of potential parameters and use-case considerations identified in Proposal 3 and Proposal 4 indicates that a bottom-up approach deciding each item one by one, would lead to unnecessary complexity, slow consensus, and potential market fragmentation.

To avoid these issues, we propose to adopt a top-down, framework-first approach for defining 6G device types. Instead of finalizing concrete device types or detailed parameter values, 3GPP should first agree on a high-level, scalable framework structure consisting of the following elements:

(1) Two top-level device type groups

- A minimal set of primary device-type groups is proposed:

- eMBB Device Group: General-purpose, performance-oriented devices (e.g., smartphones, PCs, XR-capable form factors, FWA terminals).

- IoT Device Group: Low-cost, low-power devices optimized for massive deployment and simple connectivity.

These two groups capture the broad performance envelope expected in 6G, while keeping the number of categories minimal and avoiding proliferation of device types.

Proposal 1: support only eMBB and IoT for device group to support diverse UE types

(2) Sub-type based on e.g., form factor or use case

- Within each top-level group, finer differentiation is achieved through Sub-types, based on:

- Form-factor constraints (e.g., handheld, non-handheld/FWA, wearable, industrial module, sensor-type IoT), or

- Use-case characteristics (e.g., high mobility, fixed-location, ultra-low-power).

- These Sub-types determine the relative capability ranges of each device type.

This ensures flexibility and future-proofing as 6G form factors continue to evolve.

Proposal 2: minimize sub-type but with clear differentiation between them such as form-factor constraints.

(3) Common Function Features (CFF) for all device Types

All device types and Sub-types must support a mandatory Common Function Feature (CFF) baseline, which defines the minimum capabilities required for 6G interoperability.

- CFF includes, but is not limited to:

- Basic initial access and synchronization

- Essential RRC and security procedures

- Baseline modulation/coding, HARQ, and MAC operation

- PLMN selection, MIB/SIB reception, and UE capability signaling

- Baseline power management functions

CFF ensures that all 6G devices, regardless of group or Sub-type, provide a consistent minimum behavior to the network.

Proposal 3: continue to discuss CFF in RAN1.

(4) Add-on feature sets for sub-type–specific capability scaling

Above the CFF baseline, each sub-type supports a scalable set of add-on Features that extend performance or capability as needed by the target form factor or use case (e.g., extended MIMO capability for handheld eMBB, higher power for FWA, enhanced UL reporting for IoT-sensing devices).

(5) Service-/vertical-specific capabilities expressed via profile flags

To avoid creating a separate device type for every service or vertical, feature-specific capabilities should be indicated through Profile Flags, rather than expanding the device-type list.

These may include:

- NTN support

- Integrated sensing capability

- High-accuracy positioning

- Data-collection optimization

- More

Using flags rather than device-type definitions prevents fragmentation and maintains a limited, well-structured device-type taxonomy.

Proposal 4: use profile flag to avoid creating a separate device type for every service or vertical, feature-specific capabilities.

3	Conclusions and Proposals

3GPP should adopt the proposed top-down framework, two device-type groups with sub-types, mandatory CFF, scalable add-on features, and profile flags, as the basis for defining 6G device types.

Proposal 1: support only eMBB and IoT for device group to support diverse UE types

Proposal 2: minimize sub-type but with clear differentiation between them such as form-factor constraints.

Proposal 3: continue to discuss CFF in RAN1.

Proposal 4: use profile flag to avoid creating a separate device type for every service or vertical, feature-specific capabilities.

| UE antenna elements | Handheld device (e.g., Smartphone) • Around 700 GHz:1 Tx and 2 Rx antenna elements as minimum requirement and up to 1 Tx and 4 Rx antenna elements • Around 2 GHz: 1 Tx and 2 Rx antenna elements as minimum requirement and up to 2 Tx and 4 Rx antenna elements • Around 4 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 2 Tx and 4 Rx antenna elements • Around 7 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 2 Tx and 6 Rx antenna elements • Around 30 GHz: 6 Tx and 6 Rx antenna elements as minimum requirement and up to 8 Tx and 8 Rx antenna elements Non-handheld device (e.g., FWA) • Around 700 GHz:1 Tx and 2 Rx antenna elements as minimum requirement and up to 4 Tx and 4 Rx antenna elements • Around 2 GHz: 1 Tx and 2 Rx antenna elements as minimum requirement and up to 4 Tx and 4 Rx antenna elements • Around 4 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 4 Tx and 4 Rx antenna elements • Around 7 GHz: 1 Tx and 4 Rx antenna elements as minimum requirement and up to 4 Tx and 6 Rx antenna elements • Around 30 GHz: 6 Tx and 6 Rx antenna elements as minimum requirement and up to 16 Tx and 16 Rx antenna element Iot device (e.g., massive IoT) :1 Tx and 1 Rx antenna elements as minimum requirement and up to 1 Tx and 2 Rx antenna elements Note: Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. (Except around 30 GHz) Note: For around 30 GHz, with multi-panel UE, it is the number of simultaneously used antenna elements. |
| --- | --- |