3GPP TSG RAN Meeting #110					RP-253173

Baltimore, MD, December 8 – 11, 2025

Agenda Item:	8.2.1.1

Source:	Ericsson

Title:	Network and UE energy efficiency beyond ITU-R

Document for:	Approval

# 1	Introduction

Energy efficiency is a key requirement for both the base station (BS) and devices (UE). ITU-R WP5D is currently defining minimum requirements for KPIs related to energy efficiency for IMT-2030 radio interfaces [2] (see latest agreed text in Annex A).

Proposal 1: For 3GPP internal evaluation, define energy efficiency as a quantitative KPI in 3GPP for both network and device.

Proposal 2: For 3GPP internal evaluation, at least empty load and partial load cases should be evaluated in 6GR study.

-	Evaluate via system level simulations (SLS)

-	Other evaluation methods are not precluded when relevant

Proposal 3: For 3GPP internal evaluation, UE/network power model is to be discussed in RAN1.

In RAN#109, additional methodologies for 3GPP internal studies to evaluate KPIs on energy efficiency going beyond those defined by ITU-R were discussed, and the following agreements were made:

# 2	Discussion

Compared with 5G, 6G will introduce, wider bandwidths, and larger antenna arrays. These factors inevitably increase energy demands in both baseband and RF domains for BS and UE. As agreed in RAN#109, energy efficiency is defined as a quantitative KPI evaluated via system-level simulation (SLS) as the baseline methodology. Rather than solely focusing on user perceived throughput (UPT), such evaluations should consider QoS satisfaction for different service types, using relevant metrics such as UPT, latency, or other KPIs depending on the service and scenario under evaluation.

In many scenarios, QoS-based energy evaluation (latency, reliability, service continuity) is more relevant than throughput/UPT-only evaluation.

Consider QoS-based energy saving metrics depending on the service and scenario under evaluation, instead of relying solely on throughput-based metrics.

For SLS, we think a relative evaluation model should be used as in NR, where a technique’s energy savings is quantified by comparing against a baseline for the same scenario and over the same duration and at the same load. The UE/BS energy consumption models should include slot- and symbol-level power consumption, reflecting bandwidth/time occupancy and Tx/Rx direction of symbols in a slot. System-level results can be reported per-slot or averaged across slots, regardless of the detailed symbol-level calculation approach. We note that this approach is different from the TPR on energy efficiency defined by ITU-R [2] where energy efficiency is expressed as the energy consumption at load L relative to the energy consumption at full load (L = 100%).

Adopt the relative energy efficiency evaluation methodology used in previous RAN1 studies for NR, comparing the energy consumption associated with one or more energy efficiency technique(s) to the energy consumption of a baseline scenario measured over the same duration and at the same system load.

In NR Releases 16-19, extensive work was carried out in 3GPP to develop UE and BS power models:

- UE power consumption models: TR 38.840 serving as NR baseline UE power consumption model, while extensions for low-power-wake-up receivers, reduced capability devices, and other techniques are captured in TRs 38.869 and 38.875. We believe these should remain the baseline for 6G.

- BS power consumption models: TR 38.864 defining BS power consumption model for two BS categories, Category 1, and Category 2.

As agreed in RAN#109, specifics of the BS and UE power consumption models are to be discussed in RAN1. Such discussions are already ongoing, including extensions to support 7-15 GHz operation.

# 3 	Text proposal for TR 38.914

Based on the discussion above we propose the following text proposal for TR 38.914 [1]. Since ITU-R has updated their draft report including an update on the definition of the TPR for energy efficiency, the same update is needed for TR 38.914. The change in Section 5.1.15.1 below reflects that update. Section 5.1.15.2 captures our proposal for 3GPP internal requirements.

*** Begin Text Proposal ***

### 5.1.15	Energy efficiency

#### 5.1.15.1	ITU Minimum Technical Performance Requirements

-

Energy efficiency is an important metric of sustainability. Energy efficiency of the candidate RIT/SRIT can be characterized by evaluating network and device energy efficiency.

Energy efficiency can be estimated as the relative energy consumption (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

The RIT/SRIT shall have the capability to support low relative energy consumption in the selected load cases, considering at least the unloaded (L= 0%) and loaded scenario of [0% < L ≤ 15%] / [15% < L ≤ 30%] / [30% < L ≤ 50%] / [0% < L ≤ 30%], where L is load level, i.e., network resource utilization ratio.

Proponents should report the evaluated energy efficiency values and the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies.

Proponents are encouraged to describe mechanisms of the RIT/SRIT that improve the support of low energy consumption operation for both network and device.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario.

Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

#### 5.1.15.2		3GPP Internal Requirements

Energy efficiency is an important KPI for sustainability and shall be considered as a basic principle in the 6G design. KPIs and methodologies for evaluating them are defined for both network (BS) and device (UE).

##### 5.1.15.2.1	Network Energy Efficiency

Network energy efficiency is a quantitative KPI evaluated at least by system simulation; other evaluation methods can be used when relevant. The KPI is defined as the relative energy savings of one or more network energy efficiency technique(s) quantified by comparing the energy consumption of those techniques vs. the energy consumption for a reference scenario evaluated over the same time duration and at the same system load, L. At least zero load (L = 0) and partial load shall be evaluated.

In addition to the network energy efficiency KPI, quality of service (QoS) satisfaction at the UE for a given service type and scenario is to be considered for performance impact evaluation. [Details of QoS definition are FFS].

For network energy efficiency evaluation, base station energy consumption model(s) agreed in RAN1 shall be used.

##### 5.1.15.2.2 	UE Energy Efficiency

UE energy efficiency is a quantitative KPI evaluated at least by system simulation; other evaluation methods can be used when relevant. The KPI is defined as the relative energy savings of one or more UE energy efficiency technique(s) quantified by comparing the energy consumption of those techniques vs. the energy consumption for a reference scenario over the same time duration and at the same system load L. At least zero load (L = 0) and partial load shall be evaluated.

In addition to the UE energy efficiency KPI, quality of service (QoS) satisfaction at the UE for a given service type and scenario is to be considered for performance impact evaluation. [Details of QoS definition are FFS].

For UE energy efficiency evaluation, UE energy consumption model(s) agreed in RAN1 shall be used.

*** End Text Proposal ***

# 4	Conclusion

In this contribution we propose to add the text proposal in Section 3 to TR 38.914 Clause 5.1.15.

# References

3GPP TSG RAN TR 38.914 v.0.2.0, “Study on 6G Scenarios and requirements”, October 2025.

ITU-R,  PRELIMINARY DRAFT NEW REPORT ITU-R M.[IMT-2030.TECH PERF REQ], “Minimum requirements related to technical performance for IMT-2030 radio interface(s),” Annex 5.6 to Document 5D/989-E, October 27, 2025.

# Annex A: Draft ITU-R minimum requirement for energy efficiency for IMT-2030 radio interface(s) [2]

## 4.16	Energy efficiency for sustainability

Energy efficiency is an important metric of sustainability. Energy efficiency of the candidate RIT/SRIT can be characterized by evaluating network and device energy efficiency.

Energy efficiency can be estimated as the relative energy consumption (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

The RIT/SRIT shall have the capability to support low relative energy consumption in the selected load cases, considering at least the unloaded (L= 0%) and loaded scenario of [0% < L ≤ 15%] / [15% < L ≤ 30%] / [30% < L ≤ 50%] / [0% < L ≤ 30%], where L is load level, i.e., network resource utilization ratio.

Proponents should report the evaluated energy efficiency values and the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies.

Proponents are encouraged to describe mechanisms of the RIT/SRIT that improve the support of low energy consumption operation for both network and device.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario.

Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].