3GPP TSG-RAN Meeting #110			RP-253182

Baltimore, USA, December 8-11, 2025

Agenda item:	8.2.1.0

Title:	Views on 6G deployment scenarios

Source:	NTT DOCOMO, INC.

Document for:	Discussion

# 1	Introduction

In [1], it is mentioned to identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. Accordingly, the 6G deployment scenarios have been discussed in RAN#108/109 and agreements were captured in TR 38.914 [2]. There are still missing key aspects of 6G deployment scenarios in the TR, and this contribution provides NTT DOCOMO’s view on 6G deployment scenarios.

# 2	Discussion

First, from an operator perspective, we believe that it would be important to consider realistic values that can be implemented/deployed when determining the assumptions for 6G deployment scenarios to truly deliver value to customers both in existing and new spectrum. In fact, looking back at 5G NR, some assumptions were far from practical setting, which has created a gap between theoretical performance and commercial performance, resulting in a difficulty to meet customer expectations. On the other hand, it would be also important to include advanced values to clearly show the gains from 5G NR. Therefore, we think it would be important to ensure that the baseline values for the assumptions are practical while including advanced settings.

Observation 1

- To make truly valuable 6G, it would be important to ensure that the baseline values for the deployment scenarios are practical while including advanced settings in both existing and new spectrum.

At the RAN#108 meeting, the TR for scenarios and requirements was updated with new deployments scenarios and key performance indicators. In the updated TR, more than 10 candidate deployment scenarios are captured. To enable early evaluation at RAN1, we should conclude the assumptions for at least the general deployment scenarios (e.g., Indoor hotspot, Dense urban, Rural, and Urban macro) in RAN1#110.

One of the most important remaining assumption that depends on the carrier frequency is the configuration of the BS/UE antenna. In 5G NR, the number of antenna elements for FR2 (e.g., 30 GHz, 70 GHz) was not practical. Thus, the practical number of antenna elements should be evaluated in addition to the larger number for FR3 (e.g., 7 GHz, 15 GHz).

Proposal 1

- Practical number of BS/UE antenna elements for FR3 should be used for evaluation as the baseline.

Accordingly, we propose the TP in the next section for 6G deployment scenarios.

# 3	Text proposals on deployment scenarios in TR38.914

Start of TP

## 4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 2 GHz: Up to 200 MHz (DL+UL) NOTE3. Around 4 GHz: Up to 300 MHz (DL+UL) NOTE3 Around 7 GHz: Up to 400 MHz (DL+UL) NOTE3 Around 15 GHz Up to 400 MHz (DL+UL) NOTE3 Around 30 GHz: Up to 1GHz (DL+UL) NOTE3 |
| Layout | Single layer: - Indoor floor (Open office) , 120m x 50m |
| ISD | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to [8] Tx and Rx antenna elements Around 15 GHz: Up to [8] Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, 10 users per TRxP NOTE4 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| NOTE 1: TBD NOTE 2: TBD NOTE 3: TBD NOTE 4: TBD |  |

## 4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro layer: Around 700MHz Around 2GHz Around 4 GHz Around 7 GHz Around 15GHz Around 2 GHz + Around 4 GHz Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700 MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 200MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: [100]m All micro TRxPs are all outdoor |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | Around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to [8] Tx and Rx antenna elements Around 15 GHz: Up to [8] Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 80% indoor (3km/h); 20% outdoor (30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| NOTE 1: TBD NOTE 2: TBD |  |

## 4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.3.

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7 GHz (ISD 1) Around 700 MHz + Around 7 GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to [8] Tx and Rx antenna elements |
| User distribution and UE speed | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 50% indoor (3km/h); 50% outdoor (120km/h). Opt2: 20% indoor (3km/h) 40% outdoor (60km/h) 40% outdoor (120km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 2 GHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 2 GHz + Around 30GHz Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to [8] Tx and Rx antenna elements Around 15 GHz: Up to [8] Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 80% indoor (3km/h); 20% outdoor (30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.] This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 700 MHz + Around 7 GHz Around 4 GHz +Around 7 GHz Around 2 GHz + Around 7 GHz + Around 30 GHz Around 2 GHz+ Around 700 MHz Around 7 GHz +Around 4 GHz + Around 2 GHz+Around 700 MHz |
| Aggregated bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer Hex. Grid |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 2 GHz: Up to 4 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to [8] Tx and Rx antenna elements Around 15 GHz: Up to [8] Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. Opt2: 20% outdoor in cars: 40km/h 80% indoor in houses: 3km/h |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

End of TP

# 4	Conclusion

In this contribution, the following observations and proposals are provided.

Observation 1

- To make truly valuable 6G, it would be important to ensure that the baseline values for the deployment scenarios are practical while including advanced settings in both existing and new spectrum.

Proposal 1

- Practical number of BS/UE antenna elements for FR3 should be used for evaluation as the baseline.

Proposal 2

- Adopt the TP in Section 3 for 6G deployment scenarios in TR38.914.

# 5	References

[1] RP-251395, “Revised SID: Study on 6G Scenarios and requirements” CMCC, Verizon, NTT DOCOMO, Deutsche Telekom

[2] RP-252882, “3GPP TR 38.914 V0.2.0: Study on 6G Scenarios and Requirements”