3GPP TSG RAN Meeting #110						RP-253188

Baltimore, USA, December 8-11, 2025

Agenda Item:		8.2.1.5

Source:		Nokia, T-Mobile USA, Ericsson, Qualcomm

Title:				Requirements for AI enablers in 6G radio and RAN

Document for:		Approval

# 1	Introduction

In Release 18 and Release19 3GPP RAN has been developing AI enablers and general AI framework for data collection and life cycle management based on a set of use cases like AI Mobility, AI Beam Management, AI CSI prediction and CSI compression and Location use cases. Time pressure resulted in no stage 3 development for two sided models in Rel-19 and with very limited management capabilities for MNO to manage trained models, and no standards-based solution for UE sided data collection. While further work on these areas is expected still for 5G NR in Rel20, we see that it is important to consider and define requirements for 6G radio AI enablers and AI framework studies so that the 6G radio enablers and solutions will meet Mobile Network Operator’s (MNO’s) needs and requirements from the very beginning.

In this contribution we discuss what aspects and requirements for AI related data collection and transfer needs are important for MNOs and thus, need to be addressed as part of the 6G studies. We also propose the corresponding text proposals to the TR 38.914 on 6G Scenarios and Requirements.

# 2	Discussion

The current 3GPP 5G NR air interface solutions for data collection and life cycle management (LCM) are still under development. The 5G NR solutions are built on existing 5G NR methods and procedures and tailored to address the initial use cases.  In 6G it is important that AI solutions and enablers for 6G radio and 6G RAN are developed in more general manner from the beginning. The requirements that 3GPP RAN#105 agreed for 56G NR UE side data collection in the way forward of RP-242351 form good basis for the UE side data collection of AI for 6G radio as well.

To meet the MNO’s AI related data collection and transfer needs, the following aspects must be addressed for UE side data collection for 6G radio and 6G RAN:

1. Data Security and Integrity: The collected data collected must be secured, with full guarantees of data integrity and confidentiality during transfer.

2. User Privacy and Consent: The system must respect user privacy, ensuring anonymity and compliance with user consent requirements.

3. MNO Control Over Data Transfer: MNOs must have full control over the data collection process, including initiating, terminating, and managing the transfer of data to the server for UE-side data collection. This control should not require a Service Level Agreement (SLA) to manage the transfer process.

4. Visibility and Transparency: MNOs must have full visibility over the standardized data, ensuring monitoring and management of the data collection process.

5. Future-proof and Extensible Design: The design should be flexible and scalable to adapt to future 6G AI use cases and evolving technology.

# 3 	Text proposal

We propose the following text to the 6G Scenarios and Requirements TR 38.914.

************ start of changes****************

## 5.5	Key technical principles

************ start of changes****************

### 5.5.1	AI for 6G radio and RAN

6G radio and 6G RAN shall be designed to fulfil the following requirements for UE side data collection and transfer:

-	Data Security and Integrity: The collected data collected shall be secured, with full guarantees of data integrity and confidentiality during transfer.

-	User Privacy and Consent: The system shall respect user privacy, ensuring anonymity and compliance with user consent requirements.

-	MNO Control Over Data Transfer: MNOs shall have full control over the data collection process, including initiating, terminating, and managing the transfer of data to the server for UE-side data collection. This control shall not require a Service Level Agreement (SLA) to manage the transfer process.

-	Visibility and Transparency: MNOs shall have full visibility over the standardized data, ensuring monitoring and management of the data collection process.

-	Future-proof and Extensible Design: The design shall be flexible and scalable to adapt to future 6G AI use cases and evolving technology.

************ end of changes****************

# 4	Conclusion

In this contribution we have discussed MNO’s expectations and requirements for AI for 6G radio and 6G RAN and especially for UE side data collection and transfer.  In Section 3 we have proposed text to capture these requirements in the 6G Scenarios and Requirements TR 38.914.