3GPP TSG RAN Meeting #110                                          RP-253191

Baltimore, USA, December 8-11, 2025

Source:	 ZTE Corporation, Sanechips

Title:	Requirements for 6GR services

Agenda Item:	8.2.1.4

Document for:   Discussion and Decision

- Introduction

In RAN#107 meeting, the revised SID for study on 6G scenarios and requirements was endorsed as below [1].

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

Further, based on the progress until RAN#109 meeting, the initial version of TR 38.914 for outcome of the above 6G RAN study was agreed in [2]. The related agreement reflected in TR 38.914 on service requirements are copied below.

5.4 Requirements of New and Existing Services ### 5.4.0 General This section describes the requirements of new and existing services. ### 5.4.1 Mobile Broadband ### 5.4.2 Immersive Communication ### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture) ### 5.4.4 Sensing Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs. ### 5.4.5 AI ### 5.4.6 Voice ### 5.4.7 Regulatory Services

In this contribution, we provide our views on the requirements of new and existing services including positioning service, sensing, AI services, HRLLC, Immersive communication, etc. The details can be found in section 2. The corresponding TPs are also provided in section 3.

- Discussion

For 6G, ITU-R has defined usage scenarios and overarching aspects of IMT-2030 as shown in Figure 1. The requirement for services includes immersive communication, sensing, AI, HRLLC, etc.

Figure 1 IMT-2030 scenarios from ITU-R

In 3GPP, new services and corresponding requirements are being studied in SA1 for 6G. After the SA1 discussions for several meetings, 3GPP 6G use case TR 22.870 [3] has been completed covering new use cases and requirements on System and Operational Aspects, AI, ISAC, Ubiquitous Connectivity, Massive Communication and Verticals, etc. For the upcoming 6G RAN study, it’s recommended to consider the typically identified services.

  - Immersive communication

In 5G NR, traffic for eXtended reliability (XR) is introduced in Rel-17 [6], which is characterized by quasi-periodic, large and variable packet size, stringent latency requirement and high reliability requirement. For 6GR, according to [3][6], immersive communication provides service of connecting remote user, remote robot, AI agent, generative AI and even the virtual digital world, which contains new features compared to 5G NR with more diverse traffic type, e.g., haptic data or kinaesthetic senses [8]. Additionally, as shown in Figure 2, when the number of sensors increases, haptic packets are aggregated to generate a haptic frame, where the frame sizes vary.

Figure 2 Typical scenario and haptic steam for immersive communication

For immersive communication, the QoS requirements are different among data streams. According to [8], the haptic stream has the QoS requirement of 99.9999% on reliability and less than 5ms on latency, while video stream has the QoS requirement of 99% on reliability and around 10ms on latency. For 6G immersive communication, good immersive experience relies on not only all streams meet their own QoS requirements, but also the synchronization among different streams associated with the same immersive application e.g. for haptic stream and video stream [8]. Therefore, composite requirement of data rate, latency, reliability, capacity should be considered [2].

6G RAN design shall provide mechanisms to support Immersive communication service and consider the combination with positioning, sensing, computing and AI capabilities.

  - Massive Communication (IoT)

6G aims to support the agreed existing and new services, e.g., immersive communication, HRLLC and massive connection, along with the consideration on scalable and forward-compatible design. As shown in Figure 3, introduction of device types with well-balanced design across more factors, e.g., hardware restriction, market scale/fragmentation and impacts on technical design (e.g., initial access and common channel) are still needed. In our views, the lowest-tier device, i.e., scalable UE, is designed for the reduced capability scenarios, offering lower complexity and cost. According to the discussion in [9], the suggested basic capabilities for scalable UE is {5MHz for 15KHz SCS/10MHz for 30KHzSCS,1Rx/1Tx 64QAM (or 16QAM) for DL/16QAM for UL, half-duplex}, which is similar to eRedcap introduced in Rel-18. It is noted that 10Mbps is the target peak data rate for Rel-18 eRedcap. Therefore, it would be natural to initially consider around 10Mbps as the minimum peak date rate for 6GR. Additionally, it seems no strong motivation to differentiate DL and UL peak data rate.

Figure 3 Device types and use cases in deployed in 5G and suggested for 6G

PHY DL/UL minimum peak data rate is around 10 Mbps for lowest-tier device.

  - Sensing

In Rel-19 ISAC channel modeling study, the use cases of detection and tracking of UAV, pedestrian, Vehicle, and AGV had been justified, and the corresponding channel modeling had been completed. Therefore, the support of these use cases should be considered in 6GR RAN1 study.

Furthermore, some new use cases have been included in SA1 for 6G ISAC [3]. Notably, ship detection, infrastructure collapse monitoring, and structural health monitoring have garnered substantial support from operators [4]. The commercial outlook and public safety benefits of these use cases are highly positive.

- Ship detection           (b) Infrastructure Collapse Monitoring    (c) Structural health monitoring on a bridge

Figure 4 Examples of SA1 defined new use cases on 6G ISAC

A digital twin of the radio environment and network state acts as a continuously updated virtual replica that integrates sensing-derived information (e.g., UE/BS positions, velocities, blockage dynamics, reflectors, and radio propagation features) with communication-side measurements (CSI, beam statistics, interference maps, load). A lot of new use cases have been captured in SA1 for 6GR targeting topics including energy efficiency, system structure, AI and ISAC [4]. This digital twin maintains multi-timescale radio/environment maps and predictive models of link visibility and mobility, enabling the network to simulate “what-if” outcomes, anticipate channel variation, and plan actions before conditions change. By unifying geometry, mobility, and multipath semantics, the digital twin provides a shared substrate for beam management, mobility control, resource scheduling, and cooperative transmission, while minimizing signaling by distributing distilled, decision-ready context to base stations and UEs.

Practically, the digital twin supports sensing-assisted communication by guiding proactive beam selection (including indirect paths via reflectors), predictive handover, and interference coordination based on forecasted LoS/NLoS transitions and short-term blockage risks. It closes the loop between sensing and communication: sensing updates the twin; the twin outputs candidate beam sets, expected SNR distributions, or fallback links; and communication outcomes feed back to refine the twin’s models. This leads to faster beam training, reduced control overhead, improved reliability under mobility, and better spectrum/energy efficiency, all while keeping the scope firmly within 6G sensing and communication functionalities.

Figure 5 Digital twin with sensing & positioning

Besides 5G-A use cases (UAV, Pedestrian, Vehicle, AGV), 6GR design should support sensing services including:

- Ship detection

- Infrastructure collapse monitoring

- Structural health monitoring

- digital twin to assist communication

  - AI services

According to discussion in the latest ITU-R WP 5D meeting, the AI-related capabilities which refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications have been discussed, including“Network for AI” and “AI for network”. On the other hand, 62 AI-related use cases have been agreed in AI chapter of TR 22.870 [3] until now, which include requirements about AI service and computing, AI agents, AI-related collaborative tasks and so on. Some key scenario and requirements include:

- AI service and computing

AI service and computing include the following aspects: 1) AI/ML model training, inference and AI model management; 2) AI service QoS (e.g. latency, inference accuracy) guarantee and service continuity; 3) AI related computing resource selection/coordination/monitoring.

- AI agents

AI agents can be divided into two types: 1) 3rd party AI agents on applications of UE or application server; 2) AI agents on 6G network. For the 3rd party AI agents, it may invoke 3GPP service (e.g. communication, sensing, AI) or other AI agents via intent. For AI agents on 6G network, it may provide the 3GPP service towards UE or third party based on the received request via intent.

- AI-related collaborative tasks

The AI-related collaborative task may include: 1) collaborative AI task via AI agents; 2) collaborative AI task on multiple UEs; 3) AI services to enable collaboration.

Base on these discussions, certain scenario requirements deserve the attention of the RAN side. RAN needs to consider which AI-related capabilities need to be supported and how these capabilities can be adapted to align with the requirements of these scenarios, like QoS assurance for new AI service and AI traffic, providing services through user intent/AI agent, collaborative task between AI applications via multiple UE/AI agents.

-  For the AI service, the round-trip latency for data transmission and the latency for AI inference should be considered jointly. New traffic transmission characteristics (e.g. burst data rate, error tolerance, etc.) for GenAI may be considered as well which pends to SA2 discussion on QoS framework.

- Intent can provide a novel way for users to interact with the network. The RAN side need to acquire assistance information provided by the application layer to ensure the consistency in understanding of intent from user. If the intent involves multiple devices/applications, it’s equally important to ensure coordination and consistency across devices. 6G network needs to fully understand and interprets the intent, translating it into specific service and resource allocation for each participating device, monitoring task progress, and dynamically adjusting resources as needed with involvement of RAN.

- With AI service introduced by 6G network, the network can leverage AI-related capabilities to empower UEs and applications to collaboratively accomplish more complex task based on the different capabilities of data collection devices. For example, UE may first undergo AI inference at the device level before transmitting results to the 6G network for further inference and model updates, or directly transmit raw data to the 6G network for comprehensive multi-source data fusion or complex model training.

6G design should support AI services including:

- Cross-layer information exchange needs to be supported, including the assistance application layer information sharing to RAN directly or via CN.

- RAN should support a uniform mechanism to transfer AI related data/information to CN.

- RAN should support AI related data collection and management in an efficient way.

- RAN should support and guarantee the performance of intent-driven intelligent cooperation among devices.

- RAN should support AI enabled energy efficiency for the 6G system

  - Voice

The new voice service related use cases have been captured in SA1 TR 22.870 [3], which are considering not only network and IMS simplification and enhancement but also new user experience with new capabilities e.g. immersive, AI, sensing, NTN, and so on.

Furthermore, SA2 have agreed key issue for supporting voice services for 6G, including the study on supporting voice services if they are not part of the serving 6G network. This requires features such as voice fallback to be supported from day 1. Given that VoNR deployments are mature and sufficiently supported in the field now, we can focus on voice fallback to 5G as a baseline.

Based on this, the following proposal is made:

6G RAN design shall provide mechanisms to support voice service not only to simplify and improve the existing design, but also to consider the collaboration with immersive, AI, sensing, NTN capabilities. In addition, Voice fallback to 5G should also be supported as a day1 feature.

  - Positioning

In the SA1 TR 22.870 [3], numerous use cases for 6G positioning have been captured, e.g. use case on Immersive gaming, seamless Immersive Reality in Education, high-precision Industrial control, and Remote and Automatic construction as shown in the Figure 5.

Figure 6 Examples of SA1 defined use cases on 6G positioning

As described below in TR 22.870 [3], 6G system shall be able to support location and positioning services.

5.4.2 Support of legacy services The 6G system shall be able to support the following services: - Mission Critical Services, i.e., MCPTT, MCData, MCVideo, ref TS 22.179 [53], TS 22.280 [54], TS 22.281 [55]. and TS 22.282 [56], - Message service, ref TS 22.262 [57], - SMS, ref TS 22.101 [58], - Multimedia communication services, ref TS 22.101 [5f], TS 22.261 [14], TS 22.173 [59], - IMS Multimedia Telephony Service, ref TS 22.261 [14], TS 22.173 [59], - Roaming services, ref TS 22.011 [60], TS 22.101 [58], TS 22.261 [14], - Location and positioning services, ref TS 22.261 [14], TS 22.071 [61], - Broadcast and Multicast Services, ref TS 22.261 [14],

In addition, ITU-R has decided to define a quantitative TPR for 6G positioning, the corresponding test environment scenarios may include both indoor and outdoor scenarios. The corresponding evaluation methodology is also under discussion in ITU-R WP5D where the wireless-dependent positioning method is supposed to be used.

From RAN perspective, we see a lot of commonality between sensing and positioning, for example, reference signal design, and physical layer procedure design. Thus, we think the support of 6GR wireless-dependent positioning can be studied with sensing together. Considering the lessons learned from NR positioning, we suggest to make 6GR design simpler than NR including framework, procedure, RS design, etc. Also, RAN-external positioning methods such as GNSS should be supported together considering their successful commercialization.

6G RAN design shall provide mechanism to enable RAT-embedded and RAN-external positioning techniques.

  - HRLLC

In NR, research on physical layer enhancements for URLLC was performed [5], with use cases mainly focus on industry and verticals. To achieve high reliability and low latency, NR supports numerous features from Rel-15 to Rel-19, such as CG-PUSCH, PDSCH/PUSCH mapping type B, low-SE CQI/MCS table, self-slot A/N feedback, processing capability 2, higher PDCCH aggregation level, PDSCH/PUSCH repetition, pre-emption/cancellation transmission, span-based PDCCH monitoring, intra-UE prioritization, SPS HARQ-ACK deferral, retransmission of cancelled HARQ-ACK, PUCCH cell switching, sub-band full duplex, TSN etc.

For 6G, ITU has defined usage scenarios and overarching aspects of IMT-2030, including one vertex of the hexagon dedicated to HRLLC.

Meanwhile, in the SA1 new use cases for 6G [2], 29 use cases are captured for industry and verticals which are typical HRLLC scenarios. For example, 3D factory model based AR guided task operation, Spatial Computing enabled Dynamic Material Management, independent 6G Local Network for Factory, network slice for Industry IoT etc.. The communication requirements for high data rate, lower latency and higher reliability for multiple connected machines are needed. Beyond these, network resilience, cooperation with positioning, sensing, computing and AI capabilities are also important for industry and vertical scenarios.

Thus, it is critical to meet HRLLC service requirements from 6G day 1. The typical set of PHY techniques from NR can be the starting point. Scalable designs and different sets of mandatory features for different IoT requirements/device types are essential.

6G RAN design shall provide mechanisms to support HRLLC service, and consider combination with positioning, sensing, computing and AI capabilities.

  - Security requirements for RAN

Overall security framework at RAN level strongly depends again on SA study and the outcome of this study. At RAN level, security for both UP and CP needs to be considered and lower layer security is important for 6G. Compared to 5G, it is likely that the main differences in security framework for 6G may come from the requirement to support lower layer security and also to support secure system information. These aspects need to be considered along with SA decisions for overall security framework and should be studied in conjunction with overall RAN/Core network architecture. So, a separate section should be added in the TR for security requirements at RAN level as proposed below.

RAN security requirements for 6G should include lower layer security and secure system information broadcast taking into consideration the overall network architecture in RAN and core.

  - SON and MDT

Self-Organising Networks (SON), which encompasses solutions for network self-configuration and self-optimisation and functionality to minimize the need for extensive driving tests (Minimization of Drive Tests – MDT) are both essential building blocks that enable low-cost deployments for early 6G networks. During the RAN2#132 meeting, SON/MDT is discussed together with AI/ML, sensing and QoE as potential use cases for 6G data transfer. It is agreed to study the data types, QoS and termination points of these use cases as a starting point. However, details on what is included in 6G for SON/MDT and QoE depends on RAN plenary.

Considering that SON/MDT related features rely extensively on support of the corresponding UE reports, it is necessary that these are specified as day 1 features in 6G. So, it is proposed to capture the requirement to support SON/MDT as part of the RAN TR. RAN3 has approved the following general principle for 6G RAN architecture in RAN3#130:

The 6G RAN architecture shall support self-configuration and self-optimization.FFS on which features should be recommended to be standardized from day1.

The following SON/MDT features need be supported from 6G Day1 which are important to initial 6G network deployment and enabling efficient data collection, including PCI confliction resolution, ANR, and continous MDT.

6G RAN design shall provide support Self-Organizing Networks (SON) and should enable low cost deployment of 6G networks by supporting Minimization of Drive Tests (MDT).

- Text Proposal

Based on the proposals in section 2 for the suggested services, the corresponding TPs are proposed in this section.

----------------------------------------------- Start of Text Proposals for TR 38.914 ---------------------------------------------------

## 5.4	Requirements of New and Existing Services

### 5.4.0	General

This section describes the requirements of new and existing services.

### 5.4.1	Mobile Broadband

6GR shall support existing Broadcast services.

### 5.4.2	Immersive Communication

6GR shall provide mechanism to support Immersive communication service.

6GR shall meet the composite communication performance requirements in Table 5.1.16-1 for immersive communication service.

6GR shall provide mechanisms to combine Immersive Communication service with positioning, sensing, computing and AI capabilities to support Immersive type use cases in TR22.870.

### 5.4.3	Massive Communication (IoT)

The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT):

- 6G Massive Communication (IoT) shall be supported for FR1.

  - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB

    - Prioritize 6GR design for eMBB

  - The above usage scenario should not overlap with Ambient IoT and NB-IoT

- [PHY or MAC] [minimum] peak data rate is 10[TBD] Mbps in DL and 10[TBD] Mbps in UL for lowest-tier device.

Editor note:	“6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

### 5.4.4	Sensing

Editor note:	More sensing use cases can be included depending on further discussion.

The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

6GR shall at least fulfill sensing requirements as described in section 5.1.17.

6GR shall additionally support use cases of ship detection and/or tracking, infrastructure collapse monitoring, and structural health monitoring.

6GR shall enable sensing services for digital twin to assist communication.

The 6GR shall enable, and improve if suitable, state-of-art sensing techniques, such as RAN-embedded (based on RAN wireless sensing signal) and RAN-external (e.g. Radar, Lidar, Camara, IEEE802.11 station, UWB, etc.).

The 6GR sensing shall exploit high bandwidth, massive antenna systems, network architecture/ functionalities. 6GR sensing shall support indoors/outdoors scenarios and horizontal/vertical dimensions.

6GR design targets for sensing service include:

1.	Reduced network complexity

2.	Reduced device cost

3.	Reduced device power consumption

4.	Efficient signalling over the air interface and in the network

5.	Scalability (support for large number of sensing objects)

6.	High security

7.	High availability

8. Sensing data collection, transmission, processing in an efficient and secured way

### 5.4.5	AI

- 6GR design shall support AI services including:

- Cross-layer information exchange needs to be supported, including the assistance application layer information sharing to RAN directly or via CN.

- RAN shall support a uniform mechanism to transfer AI related data/information to CN.

- RAN shall support AI related data collection and management in an efficient way.

- RAN shall support and guarantee the performance of intent-driven intelligent cooperation among devices.

### 5.4.6	Voice

6GR shall simplify and improve the existed design to support voice service via TN and/or NTN.

6GR shall support collaboration with immersive, AI, sensing, NTN capabilities to provide improved experience for voice call, e.g. immersive voice call, multimodality enabled voice call, intent triggered voice call etc.

Furthermore, 6GR should also enable support for incremental deployment of voice feature by enabling support for voice fallback to 5G when voice service is not part of the serving 6G network.

### 5.4.7	Regulatory Services

5.4.8	Location/Positioning services

6GR shall provide location/positioning services to meet the positioning requirements in Table 5.1.14-1, considering integration sensing, positioning and communication with a lean air interface design.

The 6GR shall enable positioning techniques including RAT-embedded and RAN-external.

The 6GR positioning shall exploit high bandwidth, massive antenna systems, network architecture/ functionalities and deployment of massive number of devices. 6GR positioning shall support indoors/outdoors usage scenarios and horizontal/vertical dimensions.

5.4.9 HRLLC

6GR RAN design shall support Hyper Reliable and Low Latency Communication services to meet performance requirements for typical use cases on Industry and Verticals in TR 22.870.

6GR RAN design shall support network resilience to enhance the redundancy and availability of HRLLC service considering a unified interface design with mobile broadband service.

6GR RAN shall provide mechanisms to combine HRLLC with positioning, sensing, computing and AI capabilities for Industry and Vertical scenarios in TR22.870.

5.4.10	Security and Privacy related requirements relevant for Radio Access

The RAN design shall ensure support for integrity and confidentiality protection of radio signalling messages, including messages between RAN and Core network nodes and lower layer signalling messages in RAN.

The RAN design shall ensure the ability to support integrity and confidentiality protection of user plane messages, including messages between RAN and Core network nodes, with the use of such security to be configurable during security set-up.

The RAN design shall ensure the ability to support integrity protection of system information messages.

The RAN design shall ensure support for the allocation and use of identities to provide user privacy, e.g. reduce the need for sending any permanent identities in the clear.

The RAN design shall ensure the efficient establishment of RAN security mechanisms.

The RAN design shall ensure resilience against jamming.

5.4.11	SON/MDT related requirements

The RAN design for 6GR shall fulfill the following requirements:

-	User / application level QoS and QoE monitoring capability by UEs and network elements shall be supported.

-  Features which are important to initial 6G network deployment and enabling efficient data collection should be supported from Day1, including PCI conflict resolution, ANR, and continuous MDT

----------------------------------------------- End of Text Proposals for TR 38.914 ---------------------------------------------------

- References

- RP-250810 Revised SID: Study on 6G Scenarios and requirements  CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

- RP-252882 3GPP TR 38.914 V0.2.0 Study on 6G Scenarios and Requirements (Release 20)

- TR 22.870 (V0.4.1), Study on 6G Use Cases and Service Requirements

- RP-253192 Views on 6G ISAC use cases and requirements  ZTE Corporation, Turk Telekom, Verizon, China Telecom, China Unicom, Telecom Italia, CAICT, ETRI, Queens University Belfast, Pengcheng Laboratory, Shanghai Jiao Tong University, Sanechips, CATT

- TR 38.824, Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)

- TR 38.838, Study on XR (Extended Reality) evaluations for NR

- TR 22.856, Study on Localized Mobile Metaverse Services

- TR 22.261, Service requirements for the 5G system

- RP-253199, Views on Diverse device types	ZTE Corporation, Sanechips