3GPP TSG RAN Meeting #110                                         RP-253194

Baltimore, USA, December 8-11, 2025

Source:        ZTE Corporation, Sanechips

Title:           Views on operational requirements for spectrum

Agenda Item:   8.2.1.3.1

Document for:  Discussion and Decision

- Introduction

In RAN#107 meeting, the revised SID for study on 6G scenarios and requirements was endorsed as below [1].

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

Further, based on the progress until RAN#109 meeting, the initial version of TR 38.914 towards to outcome of the above 6G RAN study was agreed [2]. The spectrum related descriptions are highlighted in yellow below.

### 5.1.13 Bandwidth Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2030 evaluation. The requirement for bandwidth is at least 400 MHz. 5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks. 5.3 Operational requirements Editor note: Spectrum, co-existence, topology and other operational requirements 5.3.1 Spectrum Editor note: e.g. spectrum and co-existence related 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

In this contribution, we provide our views on some further operational requirements for spectrum. It is noted that the detailed techniques are under discussion in RAN working groups, especially RAN1 and RAN2. Some high level requirements are suggested in this contribution. Any working group technique discussion should be avoided in RAN plenary discussion.

- Discussion

In NR TR 38.913 section 10.1, high level requirements on spectrum are listed. We think the similar requirements as marked in yellow below are also needed in 6GR for channel bandwidth scalability, duplexing flexibility, and spectrum range. Some modification is needed based on RAN SID and WG SID [3].

NR TR38.913 10.1 Spectrum 10.1.1 Void 10.1.2 Channel bandwidth scalability Bandwidth scalability means the ability to operate with different bandwidth allocations. It is a qualitative KPI. 10.1.3 Void 10.1.4 Duplexing flexibility Duplexing flexibility means the ability of the access technology to adapt its allocation of resources flexibly for uplink and downlink for both paired and unpaired frequency bands. 10.1.5 Support of shared spectrum Next Generation Radio Access Technologies should support efficient mechanisms to share either licensed or unlicensed spectrum with other IMT/Non-IMT systems. 10.1.6 Spectrum range Next Generation Radio Access Technologies should support potential use of frequency range up to 100 GHz.

For NR channel bandwidth scalability, it is described as the ability to operate with different bandwidth allocations, and claimed that it is a qualitative KPI. for 6GR, although it's up to operator's availability, the performance of the typical deployment should be still guaranteed. For example, as highlighted in [4], the system design, e.g., the common signal/channel design should be based on the assumption on bandwidth with at least 5MHz@15 kHz SCS.

For duplexing flexibility, the requirement of NR is still needed in 6GR. Subband full duplex can be considered as additional requirements if needed.

For spectrum range, it should be up to 52.6GHz in 6GR based on RAN WG SID[3].

Furthermore, it should be natural to aim at designing efficient configurations for spectrum utilization in terms of signaling overhead. For example, for carriers within the same band, or even in proximity bands, the same set of RRC parameters can be shared for saving RRC signaling overhead. For another example, for BWPs within the same carrier, some common RRC parameters should be shared such as subcarrier spacing, etc.

Also, it is common understanding that fast BWP, carrier and cell switching with high reliability are needed to adapt to dynamic traffic, flexiblely support UE/network power saving. Specifically, some useful NR features such as BWP switching, UL Tx switching, and low band CA switching should be still considered in 6GR with shorter switching gap/interruption time and higher reliability for switching indication.

In addition, we think power efficiency is very important for a green 6GR system, including both DL and UL. Especially for the UL CA case across different carriers or bands, efficient power allocation is needed to achieve high data rates, UE power saving and large UL coverage.

Finally, in 5G NR, the attributes of the serving cell (e.g. PCell or SCell) are fixed and configured via RRC. Changing UE’s PCell to an SCell requires initiating handover procedure, which results in MAC reset and SCell release & add. This not only increases interruption latency but also affects the data traffic. Meanwhile, in 5G NR, RLM/RLF is only performed on PCell. Upon failure is detected on PCell, the UE has to initiate RRC re-establishment even if the quality of SCell is stable. This results in even longer interruption and more serious impacts on user experience. In 6GR, more flexible cell/carrier switching and recovery mechanism should be considered for high robustness. For example, the network can indicate the UE to switch the primary cell/carrier when the PCell is detected to deteriorate. Moreover, the UE can perform fast recovery over SCell when failure is detected on PCell.

For operational requirements on spectrum, 6GR should aim to:

- Support channel bandwidth scalability to operate with different frequency resource allocations, without sacrifying performance of major deployment scenarios with larger channel bandwidth;

- Support duplexing flexibility;

- Support potential use of frequency range focusing on up to 52.6 GHzl

- Support efficient configuration related to spectrum utilization;

- Support fast channel bandwidth adaption with high reliability;

- Support efficient power utilization;

- Support fast cell/carrier switching and recovery.

Based on the proposal, the corresponding TP for TR38.914 is proposed below:

----------------------------------------------- Start of Text Proposals for TR 38.914 ------------------------------------------------

5.3.1	Spectrum

Editor note: e.g. spectrum and co-existence related

6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR.

6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

6GR supports channel bandwidth scalability to operate with different frequency resource allocations without performance degradation for the deployment with larger channel bandwidth. It is a qualitative KPI.

6GR supports duplexing flexibility, which is the ability of the access technology to adapt its allocation of resources flexibly for uplink and downlink for both paired and unpaired frequency bands.

6GR supports potential use of frequency range focusing on up to 52.6 GHz.

6GR aims to support efficient configurations related to spectrum utilization.

6GR aims to support fast channel bandwidth adaption with high reliability.

6GR aims for efficient power utilization especially for simultaneous transmission on different carriers/bands.

6GR aims to support fast cell/carrier switching and recovery for high robustness.

----------------------------------------------- End of Text Proposals for TR 38.914 -------------------------------------------------

- References

- RP-250810 Revised SID: Study on 6G Scenarios and requirements  CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

- RP-252882 3GPP TR 38.914 V0.2.0 Study on 6G Scenarios and Requirements (Release 20)

- RP-251881 New SID: Study on 6G Radio NTT DOCOMO (Moderator)

- RP-253199 Views on Diverse device types ZTE Corporation