3GPP TSG RAN Meeting #110                                                                                     RP-253212

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.4

Source:	HONOR

Title:	Requirements of supporting AI services for 6G RAN

Document for:	Discussion and Decision

- Introduction

In RAN#108, RAN plenary approved the 6G SID [1] and one of the objectives is to support various services and functionalities. In this contribution, we propose requirements for supporting AI services in 6G RAN.

- Discussion

  - Data management

In the TR 22.870 [2], SA WG1 has defined the 6G system data which is generated and controlled by the 6G system. 6G system data, e.g., AI/ML data, sensing data, energy efficiency information and NF context, is different from user traffic data which is application-level data transmitted transparently between the UE and DN, thus presenting new requirements for 6G data collection framework. All types of AI/ML data are collectively referred to as 6G system data which includes inference data, monitoring data, training data and model data.

In 5G NR, data collection and data delivery has been discussing for specific use cases. In our view, 6G Day-1 RAN design should take the existing and potential use cases for data services into account to design the signaling and procedures for data collection and data delivery with addressing both universal and scenario-specific requirements.

As more use cases evolve, the RAN node may need to accumulate large amount of data for various purposes, potentially leading to the duplicated collection of same or similar data, as well as the wastage of energy and resources. It would be beneficial if the RAN node could be aware of the data content and adapt accordingly for performance improvement via data management. For example, measurement data reported by UEs could be stored in RAN and accessible to multiple data consumers within 3GPP system to support various services, e.g., analyzed and utilized by native AI for flexible resource allocation and optimized network performance.

Proposal 1: 6G RAN should support Data Management for AI services to fulfil the data service requirements, e.g. duplication detection, data storage and data distribution.

  - Transfer requirements

In 5G, AI/ML LCM process is discussed case by case and data could be transferred via multiple paths, e.g., UCI/DCI/RRC to meet the requirements of different cases. 6GR should study how to unify the requirements across various scenarios and then design a common data framework for 6G system data. Based on the conclusion in 5G, AI/ML data could be modeled in the following aspects:

- Data size: AI/ML data size may vary depending on the data types. Typically, AI/ML data transfer could vary from several kbits to Mbits (e.g., the data volume two-sided CSI compression can be varied from 36k bytes to around 236M bytes [3]).

- Transmission latency: The latency requirements will differ significantly across data types and scenarios, e.g., inference data reporting is time-critical and should be reported every few milliseconds, while transferring the training data to training entity could be done in days or weeks.

- Reliability: The reliability requirements may differ based on the data types, e.g., model transfer requires robust encryption and integrity verification to protect the model’s structure and parameters, while dataset transfer could tolerate minor transmission errors.

- Transfer frequency: Transfer frequency should be viewed as one of the key data characteristics for real-time of near-real-time process. Inference/monitoring data is often transferred frequently and model transfer for offline training typically occurs Irregularly.

- Privacy & Security protection: Securing data transmission are essential to improving the performance and reliability of AI systems. The integrity and authenticity of data have a direct impact on the training quality and inference accuracy of AI models.

Compared with traditional traffic, AI/ML data transfer in 6G involves diverse endpoint pairs:

- Multiple end point pairs: AI/ML data transmission in 6G involves diverse endpoint pairs, including UE-CN, UE-RAN, RAN-CN, and UE-RAN&CN. With the presence of more scenarios, the other nodes could also be candidate end points, e.g. OAM, SF which depends on SA.

Proposal 2: 6G RAN shall support efficient transmission for AI/ML data with considering the following requirements:

- Evaluable transmission requirements: the transmission requirements could be evaluated through the following dimensions, i.e. Data size, transmission latency, transmission reliability, transfer frequency.

- Ensure privacy & security protection.

- Deterministic end point pairs: the end points pair could be UE/CN, UE/RAN, RAN/CN, UE/RAN&CN.

  - Consideration on designing principle

Considering system-level solution, the 6G system shall support mechanisms for data transmission that minimize the impact to traditional services. For compatibility issues, NR has some limitations to support the AI/ML scenarios and 6GR should consider enhancement of control plane and user plane or introducing new data plane. In any case, it is necessary for the network to distinguish different services and provide corresponding guarantee.

6GR should natively support a scalable and forward-compatible data handling framework to meet the requirements of new service. Considering the various data type for AI/ML, the transmission requirements are quite different and Qos-like scheme should be considered to meet the transmission requirements and provide the corresponding functionalities for AI/ML service.

Proposal 3: Support the data handling framework for AI services with the following principles:

Differentiation of AI service from traditional services;

Differentiation between AI services with Qos-like scheme.

- Conclusions

In this contribution, we provide our views on requirement of supporting AI services in 6G first release. Based on the analysis, we propose:

Proposal 1: 6G RAN should support Data Management for AI services to fulfil the data service requirements, e.g. duplication detection, data storage and data distribution.

Proposal 2: 6G RAN shall support efficient transmission for AI/ML data with considering the following requirements:

- Evaluable transmission requirements: the transmission requirements could be evaluated through the following dimensions, i.e. Data size, transmission latency, transmission reliability, transfer frequency.

- Ensure privacy & security protection.

- Deterministic end point pairs: the end points pair could be UE/CN, UE/RAN, RAN/CN, UE/RAN&CN.

Proposal 3: Support the data handling framework for AI services with the following principles:

Differentiation of AI service from traditional services;

Differentiation between AI services with Qos-like scheme.

- References

- RP-251881, “New SID: Study on 6G Radio”, RAN#108, June 2025.

- TR 22.870, “Study on 6G Use Cases and Service Requirements”, 3GPP.

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, 3GPP.