3GPP TSG RAN Meeting #110								RP-253221

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.0

Source:	Xiaomi

Title:	Discussion on 6G UE antenna configuration

Document for:	Discussion and Decision

# Introduction

In the RAN#107 meeting, the following objectives were added in the updated SID [1]:

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

At RAN#108 meeting, several communication deployment scenarios for 6G Radio (6GR) were discussed and agreed upon, including indoor hotspot (InH), dense urban (DU), rural, urban macro (UMa), sub-urban macro (SMa), etc. At RAN#109, the details on attributes of communication deployment scenarios were further discussed, including carrier frequency, aggregated system bandwidth, layout, ISD, user distribution and speed, etc., and section 4 of TR 38.914 were upgraded accordingly [2]. However, the modelling of UE antenna remains under discussion. In RAN1#122bis and RAN1#123 meeting, RAN1 evaluation assumptions were discussed but the assumptions on UE antenna models were not agreed.

In this contribution, our views on UE antenna configuration of the 6GR deployment scenarios are given.

# UE antenna configuration

6G is expected to support various kinds of terminals e.g., handheld UE, IOT devices, etc. Different kind of 6G terminals could have different form factor restriction, UE capabilities, and antenna configurations. Given that the market for handheld UEs will still be the most important market for 6G, the UE configurations for handheld UEs shall serve as the baseline. Of course, the UE configurations of other 6G terminals such as IOT devices must also be considered.

Comprehensively considering the necessity of multi-antenna deployment for a handheld UE and its restrictions in respects of physical size and power consumption, it is preferred to consider 1 Tx and 2 Rx antennas (i.e., 1T2R, (M, N, P, Mg, Ng; Mp, Np)=(1,1,2,1,1;1,1)) for carrier frequencies around 700MHz and 2GHz, and consider 2 Tx and 4 Rx antennas (i.e., 2T4R, (M, N, P, Mg, Ng; Mp, Np)=(1,2,2,1,1;1,2)) for carrier frequencies around 4GHz and 7GHz.

Proposal 1: For handheld UE,

- consider 1T2R ((M, N, P, Mg, Ng; Mp, Np)=(1,1,2,1,1;1,1)) for carrier frequencies around 700MHz and 2GHz,

- consider 2T4R ((M, N, P, Mg, Ng; Mp, Np)=(1,2,2,1,1;1,2)) for carrier frequencies around 4GHz and 7GHz.

For IoT UEs, which typically operating at lower frequency bands, e.g. 700MHz or 2GHz, a single antenna (i.e., 1T1R)  configuration is used. For those working at 4GHz, single transmit antenna and up to 2 receive antennas (i.e., 1T2R) can be considered.

Proposal 2: For IoT UEs, consider 1T1R at 700MHz or 2GHz, and 1T2R for 4GHz.

# Conclusion

In this contribution, the deployment scenarios for 6GR TN are discussed, and the following proposals are made:

Proposal 1: For handheld UE's,

- consider 1T2R ((M, N, P, Mg, Ng; Mp, Np)=(1,1,2,1,1;1,1)) for carrier frequencies around 700MHz and 2GHz,

- consider 2T4R ((M, N, P, Mg, Ng; Mp, Np)=(1,2,2,1,1;1,2)) for carrier frequencies around 4GHz and 7GHz.

Proposal 2: For IoT UEs, consider 1T1R at 700MHz or 2GHz, and 1T2R for 4GHz.

# References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, Inc., Deutsche Telecom, Mar. 12-14, 2025

- 3GPP TR 38.914 v 0.2.0, Study on 6G Scenarios and requirements, Sep. 2025