3GPP TSG RAN Plenary Meeting #110						   RP-253231

Baltimore, USA, December 8-12, 2025

Source:	Tejas Networks, CEWiT, Indian Institute of Technology Madras (IITM), Indian Institute of Technology Kanpur (IITK)

Title:		    Discussion on 6G Deployment Scenarios

Agenda item:	    8.2.1.0 – Deployment Scenarios

Document for:   Discussion and Decision

# Introduction

The RAN technical specification group started a study “Study on 6G Scenarios and requirements” in RAN#107 with an objective to provide guidance towards technical work in RAN WGs and an input to ITU-R to develop minimum technical performance requirements for IMT-2030. One of the main aspects of the is the identification of practical 6G deployment scenarios along with their attributes and related performance requirements. The study outcomes are being updated in the running technical report TR 38.914 [1].

In this contribution, we present our proposals on:

- Considering additional attributes in the Rural deployment scenario

- Attributes and requirements for two new deployment scenarios to be considered in 6GR evaluation studies

  - Single Cell with Large Coverage

  - Indoor Factory

# 2. Rural Deployment Scenario

The population and geographical distributions of rural villages in India, (and also applicable to most of the Asian countries), is not uniform, as shown in Figure 1. In India, more than 50% of the population lives in rural parts of the country. A group of villages with an approximate population of 1000 to 2000 is administered from a village (Gram Panchayat) with a higher population of approximately 5000 - 10000.

The smaller villages are not located continuously, but instead, are separated by a distance of approximately 2 – 6 km. To provide telecom services to the people living in such environments, it is important for 3GPP and ITU to consider this geographical distribution of rural population (described above and depicted in Figure 1) for evaluation of 6G Radio Interface Technology (RIT).

Figure 1: A general representation of cellular user distribution and deployment of network infrastructure in rural parts of India. A moderately populated (5000-10000 people) geographical area is surrounded by sparsely distributed areas of smaller population (1000-2000 people)

In the current version of TR 38.914, [1] and the moderator summary [2], the rural deployment scenario for multi cell deployments has the following configurations.

Table 2.1.1 Attributes of Rural deployment scenario

| Carrier Frequency | Aggr. system Bandwidth | Layout | ISD | UE distribution and UE speed |
| --- | --- | --- | --- | --- |
| 700 MHz | Up to 60 MHz (DL+UL) | Single layer: Hex Grid | 1732m, 5000m and [7500m] | [15% outdoor vehicles (120km/h), 15% outdoor (3 km/h) and 70% indoor (3 km/h), or 50% outdoor vehicles (120km/h), 50% indoor (3 km/h)] [10] users per TRxP |
| 2 GHz | Up to 200 MHz (DL+UL) |  | 5000m |  |
| 4 GHz | Up to 300 MHz (DL+UL) |  | 1732m |  |
| 7 GHz | Up to 400 MHz (DL+UL) |  | 1732m |  |

## 2.1 Inter-Site Distance (ISD)

As described earlier in this section, the general rural deployment in India places base stations at the locations of the administrative head of a group of villages. This location, referred to as the Gram Panchayat (GP), is typically a central village with a higher population. It is surrounded by villages of smaller population, which are spread out sparsely throughout the region. Based on the information on GPs and associated villages provided by the Government of India [3], in Figure 2, we demonstrate the ISD (or cell radius) requirements to provide terrestrial connectivity to Indian rural areas.

Figure 2: CDF of maximum distance (km) between a Gram Panchayat (GP) and its associated less populated villages.

From Figure 2, we observe that 80% of the villages are located 4 km away from a GP, which corresponds to an ISD of ~6.3 km. In RAN# 109, the agreed ISDs for the 700 MHz carrier frequency are 1.732 km and 5 km [1], [2]. These ISDs which translate to a cell radius of 1 km and 2.887 km respectively, can provide terrestrial connectivity to only 60% of the villages in rural India (see Figure 2).

Observation 1: To increase the reach of terrestrial connectivity to at least 85% of villages in rural India, the Rural deployment scenario should also include an ISD of 7.5 km (cell radius of 4.33 km)

Proposal 1: Consider an ISD3 of 7.5 km at least for FWA deployments (Section 4.3 of TR 38.914), in order to maximize connectivity in rural areas.

## 2.2 UE Distribution and Speeds

The current UE distribution being considered in TR 38.914 does not give a complete picture of rural scenarios. We would like to point out that rural speeds are not just limited to pedestrians on one extreme and cars on the highway on the other extreme. The sparsely distributed villages are often connected by rural non-highway roads.

Observation 2: A realistic representation of rural UEs includes pedestrians, buses, two-wheelers, farm vehicles, trains and highway vehicles.

Proposal 3: In order to capture a realistic representation of rural UEs, the distribution and speeds in Section 4.3 of TR 38.914, for ISD 1.732 km and 5 km, should be: 40% indoor (3 km/h), 20% outdoor (3 km/h), 20% outdoor ([30-60] km/h), 20% (120 km/h).

# Single Cell with Large Coverage

More extreme cases of rural areas in India, which include remote villages, require isolated cells with a coverage of around 10 km. The extreme long distance deployment scenario in Section 4.7 of TR 38.914 considers coverage ranges of 100 km and above. We find this to be impractical at least in the Indian context.

A single cell deployment scenario with a coverage on the order of 10 km is necessary for very low density, rural areas where the primary mode of wireless communication is through terrestrial means.  The following figure shows a possible layout for the single cell deployment with 3 sectors.

Figure 3: Isolated Macro Cell with a cell radius of up to 10 km.

For this deployment scenario, we propose 2 different cases of UE types based on the height. The first UE type is a handheld device with a height of 1.5m and the second is a Customer Premises Equipment (CPE) style device with a height of 5m (Fixed Wireless Access).

The extreme rural deployments as described above are associated with the following attributes, shown in the tables below.

Table 3.1: Attributes for Extreme Rural eMBB

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 600 MHz (Around 700 MHz) |
| System Bandwidth | 20 MHz (DL+UL) |
| Layout | Single cell layout (Isolated Macro cell) |
| Cell radius | Up to 10 km |
| User density and UE speed | 10 users per cell (distributed uniformly) 40% Outdoor pedestrian: 3 kmph 20% Outdoor in cars: 30 kmph 40% Indoor: 0-3 kmph |
| UE height | 1.5 m |
| Traffic model | Full buffer |
| Applicable KPI | DL Coverage (Note 1), UL Coverage (Note 1), 5th percentile spectral efficiency, Average spectral efficiency |

Table 3.2: Attributes for Extreme Rural FWA (CPE)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 7GHz |
| System Bandwidth | 50 MHz (TDD) |
| Layout | Single cell layout (Isolated Macro cell) |
| Cell radius | Up to 10 km |
| User density and UE speed | 100% outdoor with 0 kmph |
| UE height | 5m |
| Traffic model | Full buffer |
| Applicable KPI | DL Coverage (Note 1), UL Coverage (Note 1), 5th percentile spectral efficiency, Average spectral efficiency |

Note 1: Coverage is expressed as the Maximum Link Distance with a corresponding target data rate (x Mbps).

Proposal 4: Consider 600 MHz as a candidate carrier frequency for extreme rural deployment scenarios, for long distance coverage in low density areas.

Proposal 5:  To handle extreme rural cases of India and other similar geographies where the primary means of communication is through terrestrial technologies, the following options can be considered:

- Option 1: Define a new deployment scenario called "Single Cell with Large Coverage” as Section 4.xx in TR 38.914 [1]

  - Add Tables 3.1 and 3.2

- Option 2: Define a single cell layout (Isolated cell) with a cell radius of 10 km, lower user density and mobility (<= 120 kmph) as an additional configuration under section 4.7 “Extreme long-distance coverage in low density area” in TR 38.914 [1]

  - Add Tables 3.1 and 3.2

# Indoor Factory (InF)

In 5G studies, the Indoor Factory (InF) deployment scenario is extensively used for the evaluation of various functionalities. Moving forward, the InF deployment scenario is equally important for certain usage scenarios e.g. Immersive Communication, HRLLC and Integrated Sensing and Communication (ISAC). The InF scenario focuses on factory halls of varying sizes and with varying levels of density of "clutter", e.g. machinery, assembly lines, storage shelves, etc. [4]. The key characteristics of this deployment scenario are clutter type, clutter density, height of Tx and Rx and LOS/NLOS. The InF scenario is further classified into InF-SL, InF-DL, InF-SH, InF-DH and InF-HH based on (i) density of the clutter (S - sparse and D - dense) and (ii) the height of Tx and Rx (L - low and H - high).

Figure 4: Indoor factory deployment scenario with 18 base stations, with a floor dimension of 300 m by 150 m.

Table 4.1 provides the possible attributes of InF-SL, InF-DL, InF-SH, InF-DH and InF-HH scenarios.

Table 4.1: Attributes for Indoor Factory

| Parameters |  | InF-SL (sparse clutter, low BS) | InF-DL (dense clutter, low BS) | InF-SH (sparse clutter, high BS) | InF-DH (dense clutter, high BS) | InF-HH (high Tx, high Rx) |
| --- | --- | --- | --- | --- | --- | --- |
| Layout | Room size | Rectangular: 45000 m2 i.e. 300 x 150 m |  |  |  |  |
|  | Ceiling height | 5-25 m | 5-15 m | 5-25 m | 5-15 m | 5-25 m |
| ISD |  | 50 m |  |  |  |  |
|  | Effective clutter height hc | < 10 m |  |  |  |  |
|  | External wall and ceiling type | Concrete or metal walls and ceiling with metal-coated windows |  |  |  |  |
| Clutter type |  | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Any |
| Typical clutter size, dclutter |  | 10 m | 2 m | 10 m | 2 m | Any |
| Clutter density r (percentage of surface area occupied by clutter) |  | Low clutter density (< 40%) | High clutter density (≥ 40%) | Low clutter density (< 40%) | High clutter density (≥ 40%) | Any |
| BS antenna height |  | Clutter-embedded, i.e. the BS antenna height is below the average clutter height |  | Above clutter |  | Above clutter |
| UT location | LOS/NLOS | LOS and NLOS |  |  |  | 100% LOS |
|  | Height | Clutter-embedded |  |  |  | Above clutter |
| User distribution and UE speed |  | 100% Indoor, 3 kmph |  |  |  |  |
| Service profile |  | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |  |  |  |
| Applicable KPI |  | UE positioning, Sensing related capability, Reliability |  |  |  |  |

Proposal 6: Add Indoor Factory (InF) as one of the deployment scenarios for 6G.

# Conclusions

Observation 1: To increase the reach of terrestrial connectivity to at least 85% of villages in rural India, the Rural deployment scenario should also include an ISD of 7.5 km (cell radius of 4.33 km)

Proposal 1: Consider an ISD3 of 7.5 km at least for FWA deployments (Section 4.3 of TR 38.914), in order to maximize connectivity in rural areas.

Observation 2: A realistic representation of rural UEs includes pedestrians, buses, two-wheelers, farm vehicles, trains and highway vehicles.

Proposal 3: In order to capture a realistic representation of rural UEs, the distribution and speeds in Section 4.3 of TR 38.914, for ISD 1.732 km and 5 km, should be: 40% indoor (3 km/h), 20% outdoor (3 km/h), 20% outdoor ([30-60] km/h), 20% (120 km/h).

Proposal 4: Consider 600 MHz as a candidate carrier frequency for extreme rural deployment scenarios, for long distance coverage in low density areas.

Proposal 5:  To handle extreme rural cases of India and other developing countries where the primary means of communication is through terrestrial technologies, the following options can be considered:

- Option 1: Define a new deployment scenario called "Single Cell with Large Coverage” as Section 4.xx in TR 38.914 [1]

  - Add Tables 3.1 and 3.2

- Option 2: Define a single cell layout (Isolated cell) with a cell radius of 10 km, lower user density and mobility (<= 120 Kmph) as an additional configuration under section 4.7 “Extreme long-distance coverage in low density area” in TR 38.914 [1]

  - Add Tables 3.1 and 3.2

Proposal 6: Add Indoor Factory (InF) as one of the deployment scenarios for 6G.

Proposal 7: Consider the agreed proposals of Annexure 1 to be included in the consolidated pCR.

# References

1. RP-252882, Study on 6G Scenarios and Requirements, TR, 38.914, September 2025.

2. RP-252915, pCR on Deployment scenarios for TR 38.914, September 2025.

3. https://LGDirectory.gov.in

4. TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

# Annexure

Text Proposal to TR 38.914

If Option 1 (Proposal 5) is agreed, a new section 4.xx should be created in TR 38.914 to include the following:

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

4.xx Single Cell with Large Coverage

This deployment scenario is intended for single macro cell deployment in very low-density rural areas where the primary mode of communication is through terrestrial means.  The following figure shows a possible layout for the single cell deployment with 3 sectors.

Figure x: Isolated Macro Cell with a cell radius of up to 10 km.

The Single Cell with Large Coverage is associated with the following attributes, shown in the tables below.

Table x: Attributes for Extreme Rural eMBB

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 600 MHz (Around 700 MHz) |
| System Bandwidth | 20 MHz (DL+UL) |
| Layout | Single cell layout (Isolated Macro cell) |
| Cell radius | Up to 10 km |
| User density and UE speed | 10 users per cell (distributed uniformly) 40% Outdoor pedestrian: 3 kmph 20% Outdoor in cars: 30 kmph 40% Indoor: 0-3 kmph |
| UE height | 1.5 m |
| Traffic model | Full buffer |
| KPI | DL Coverage (Note 1), UL Coverage (Note 1), 5th percentile spectral efficiency, Average spectral efficiency |

Table x: Attributes for Extreme Rural FWA (CPE)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 7GHz |
| System Bandwidth | 50 MHz (TDD) |
| Layout | Single cell layout (Isolated Macro cell) |
| Cell radius | Up to 10 km |
| User density and UE speed | 100% outdoor with 0 kmph |
| UE height | 5 m |
| Traffic model | Full buffer |
| KPI | DL Coverage (Note 1), UL Coverage (Note 1), 5th percentile spectral efficiency, Average spectral efficiency |

Note 1: Coverage is expressed as the Maximum Link Distance with a corresponding target data rate (x Mbps).

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

If Option 2 (Proposal 5) is agreed, Section 4.7 in TR 38.914 should be updated to include the following additional configuration:

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

4.7 Extreme long-distance coverage in low density area

Table x: Attributes for Extreme Rural eMBB

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 600 MHz (Around 700 MHz) |
| System Bandwidth | 20 MHz (DL+UL) |
| Layout | Single cell layout (Isolated Macro cell) |
| Cell radius | Up to 10 km |
| User density and UE speed | 10 users per cell (distributed uniformly) 40% Outdoor pedestrian: 3 kmph 20% Outdoor in cars: 30 kmph 40% Indoor: 0-3 kmph |
| UE height | 1.5 m |
| Traffic model | Full buffer |
| KPI | DL Coverage (Note 1), UL Coverage (Note 1), 5th percentile spectral efficiency, Average spectral efficiency |

Table x: Attributes for Extreme Rural FWA (CPE)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 7GHz |
| System Bandwidth | 50 MHz (TDD) |
| Layout | Single cell layout (Isolated Macro cell) |
| Cell radius | Up to 10 km |
| User density and UE speed | 100% outdoor with 0 kmph |
| UE height | 5 m |
| Traffic model | Full buffer |
| KPI | DL Coverage (Note 1), UL Coverage (Note 1), 5th percentile spectral efficiency, Average spectral efficiency |

Note 1: Coverage is expressed as the Maximum Link Distance with a corresponding target data rate (x Mbps).

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# 8. Appendix A

## Single Cell Simulation Results

We performed link level simulations for 5G NR PUSCH with the following parameters.

Table A.1: Simulation Parameters

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 600 MHz, 700 MHz and 7GHz |
| Waveform | CP-OFDM |
| Sub carrier spacing | 30 kHz |
| Number of resource blocks per user | 10 |
| Antenna Elements | 4 UE, 64 BS (600 MHz, 700 MHz) and 8 UE, 1024 BS (7 GHz) |
| Modulation | QPSK |
| Speeds | 0 kmph, 3 kmph and 30 kmph |

From the link level simulations, we obtained the SNR at which a target BLER of 10% was achieved. We also calculated the corresponding throughput. The tables, Table A.2 – A.4 present LLS evaluations for the Single Cell scenario with different carrier frequencies and UE speeds.

Table A.2 LLS results for Rural Macro with 600 MHz for 3 different UE speeds

| UE speed | Number of RBs per user | Code rate | Modulation | SNR for 10% BLER (dB) | Throughput (Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0 kmph | 10 | 449/1024 | QPSK | -2.4 | 2.26 |
|  | 10 | 308/1024 | QPSK | -4.4 | 1.59 |
|  | 10 | 251/1024 | QPSK | -5.784 | 1.26 |
| 3 kmph | 10 | 449/1024 | QPSK | 2.68 | 2.26 |
|  | 10 | 308/1024 | QPSK | 0.926 | 1.59 |
|  | 10 | 251/1024 | QPSK | -0.332 | 1.26 |
| 30 kmph | 10 | 449/1024 | QPSK | 2.9 | 2.26 |
|  | 10 | 308/1024 | QPSK | 1.08 | 1.59 |
|  | 10 | 251/1024 | QPSK | -0.2 | 1.26 |

Table A.3 LLS results for Rural Macro with 700 MHz for 3 different UE speeds

| UE speed | Number of RBs per user | Code rate | Modulation | SNR for 10% BLER (dB) | Throughput (Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0 kmph | 10 | 449/1024 | QPSK | -2.4 | 2.2 |
|  | 10 | 308/1024 | QPSK | -4.4 | 1.5 |
|  | 10 | 251/1024 | QPSK | -5.8 | 1.2 |
| 3 kmph | 10 | 449/1024 | QPSK | 2.4 | 2.2 |
|  | 10 | 308/1024 | QPSK | 0.6 | 1.5 |
|  | 10 | 251/1024 | QPSK | -0.5 | 1.2 |
| 30 kmph | 10 | 449/1024 | QPSK | 3.06 | 2.2 |
|  | 10 | 308/1024 | QPSK | 1.2 | 1.5 |
|  | 10 | 251/1024 | QPSK | -0.09 | 1.2 |

Table A.4 LLS results for Rural Macro with 7GHz for a stationary UE/CPE

| UE speed | Number of RBs per user | Code rate | Modulation | SNR for 10% BLER (dB) | Throughput (Mbps) |
| --- | --- | --- | --- | --- | --- |
| 0 kmph | 10 | 526/1024 | QPSK | -2.32 | 2.13 |
|  | 10 | 379/1024 | QPSK | -3.361 | 1.53 |
|  | 10 | 308/1024 | QPSK | -4.4 | 1.27 |

The SNR values in the above tables are used in the link budget calculations to obtain the coverage or the maximum link distance at which the target throughout is achieved. The link budget sheet is attached to this contribution.

The following tables show Coverage (Maximum Link Distance) results for 3 different target throughputs and 10% BLER.

Table A.5 Maximum Link Distance computations for UE with 1.5m and CPE with 5m height, different carrier frequencies and speeds at 2 Mbps UL Throughput

|  | All values at 2 Mbps UL Throughput |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Outdoor 5m (26 dBm + 0dBi) | Indoor 5m (26 dBm + 0dBi) | Outdoor 1.5m 23dBm, 30kph | Indoor 1.5m 23dBm, 3kph |  |
| 600 MHz | 4.7 km | 4 km | 2.1 km | 1.8 km |  |
| 700 MHz | 4.4 km | 3.7 km | 1.9 km | 1.7 km |  |
|  | Outdoor 5m (26 dBm + 5 dBi) | Indoor 5m (26 dBm + 5 dBi) | Outdoor 5m (26 dBm + 5 dBi), 55m BS height | Outdoor 5m (23 dBm + 0 dBi) | Indoor 5m (23 dBm + 0 dBi) |
| 7 GHz | 4.3 km | 3.7 km | 6.03 km | 2.7 km | 2.3 km |

Table A.6 Maximum Link Distance computations for UE with 1.5m and CPE with 5m height, different carrier frequencies and speeds, at 1.5 Mbps UL Throughput

|  | All values at 1.5 Mbps UL Throughput |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Outdoor 5m (26 dBm + 0dBi) | Indoor 5m (26 dBm + 0dBi) | Outdoor 1.5m 23dBm, 30kph | Indoor 1.5m 23dBm, 3kph |  |
| 600 MHz | 5.3 km | 4.6 km | 2.4 km | 2.07 km |  |
| 700 MHz | 4.9 km | 4.2 km | 2.2 km | 1.95 km |  |
|  | Outdoor 5m (26 dBm + 5 dBi) | Indoor 5m (26 dBm + 5 dBi) | Outdoor 5m (26 dBm + 5 dBi), 55m BS height | Outdoor 5m (23 dBm + 0 dBi) | Indoor 5m (23 dBm + 0 dBi) |
| 7 GHz | 4.6 km | 4.01 km | 6.43 km | 2.9 km | 2.49 km |

Table A.7 Maximum Link Distance computations for UE with 1.5m and CPE with 5m height, different carrier frequencies and speeds, at 1.2 Mbps UL Throughput

|  | All values at 1.2 Mbps UL Throughput |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Outdoor 5m (26 dBm + 0dBi) | Indoor 5m (26 dBm + 0dBi) | Outdoor 1.5m 23dBm, 30kph | Indoor 1.5m 23dBm, 3kph |  |
| 600 MHz | 5.8 km | 5 km | 2.6 km | 2.2 km |  |
| 700 MHz | 5.4 km | 4.6 km | 2.37 km | 2.08 km |  |
|  | Outdoor 5m (26 dBm + 5 dBi) | Indoor 5m (26 dBm + 5 dBi) | Outdoor 5m (26 dBm + 5 dBi), 55m BS height | Outdoor 5m (23 dBm + 0 dBi) | Indoor 5m (23 dBm + 0 dBi) |
| 7 GHz | 4.98 km | 4.27 km | 6.85 km | 3.09 km | 2.65 km |