3GPP TSG-RAN Meeting #110	RP-253238

# Baltimore, US, December 08-11, 2025

Agenda item:		8.2.1.2

Source:	Xiaomi

Title:	Requirements for architecture and migration

Document for:		Discussion and Decision

# Introduction

In RAN#109 meeting, based on [1], RAN plenary discussed the requirements for architecture and migration and agreed the corresponding TP in [2].

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

However some aspects are missing, e.g, support of new services and requirements on inter-RAT mobility.

In this contribution, we continue the discussion on these aspects.

# Discussion

## Architecture requirements

### Requirement for RAN-CN interface

In RAN#109, RANP agreed “-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.”, however it is unclear whether it covers both the RAN internal interface and RAN-CN interface since RAN-CN interface is not just for 6G RAN. We believe it shall cover both RAN internal interfaces and RAN-CN interfaces. Therefore, it would be good to clarify it similar as 5G, i.e., “-	RAN-CN interfaces and RAN internal interfaces shall be open for multi-vendor interoperability”.

Proposal 1: RAN-CN interfaces and RAN internal interfaces shall be open for multi-vendor interoperability.

RAN WG 6G SID RP-251881 [3] has the following architecture related objectives:

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

5G NR signalling transfer between UE/RAN and CN, e.g., LMF is routed via AMF. There is scalability issue due to the load of AMF and potential single point of failure. These issues were already realized during 5G NR, and some workarounds were introduced. For example, originally LPP signalling was transferred via AMF in control plane, and enhancements were introduced to transport LPP signalling via user plane. For 6G, there is the opportunity to address above issues of RAN-CN interface in a systematic manner.

Proposal 2: The RAN architecture shall support efficient signalling transfer between UE/RAN and CN for diverse services, e.g., sensing, AI in efficient and scalable way.

### Support of new services (AI and sensing)

The issue has been discussed in last RANP as

New services - The design of the 6G RAN architecture shall allow the collection and transport of new service data (e.g. AIML data, sensing data) efficiently to fulfil new service requirements. - The design of the 6G RAN architecture shall support the generation, transport, and/or visibility of new service data (e.g., AIML data, sensing data) efficiently to fulfil new service requirements. - The design of the RAN architecture shall support the deployment of the new services e.g.AI/ML,sensing, rapidly and efficiently. - Support Efficient Data Transfer: Enable efficient data transfer for new services (e.g., AI/ML, sensing) across all network entities (UE, RAN node, CN) - Provide a Unified Framework: Ensure a consistent and unified data transfer design for all services and use cases. - Guarantee QoS: Strictly fulfill the diverse QoS requirements of these new services - The RAN shall support a management centric data collection framework enabling data collection and distribution to/from different parts of the 6G system. The framework shall enable local consumption of data at the RAN and data exposure from the RAN to data collection entities. - The design of the RAN architecture shall allow the deployment of new end-user services rapidly and efficiently. - 6G RAN architecture shall allow the introduction of new features rapidly and efficiently by enabling common architecture design. - The RAN architecture shall allow for RAN data plane. - The RAN architecture shall allow integrated sensing and communication function(s). - The RAN architecture shall support UE aggregation. - The RAN architecture shall allow for network digital twin (NDT).

To our understanding, to support new service, data collection is not the main requirement for RAN architecture design. Let’s use sensing as example. To support sensing, RAN needs to study:

- Phy design and evaluation for sensing, e.g., Waveform, numerology, frame structure, RS design, etc

- Define Measurement metrics

- Aspects of integration with communication services

- Overall architecture, procedure for Sensing, in coordination with SA2

- Interface/protocol between RAN and CN and internal RAN architecture, interface and protocol

This has been reflected in the SID [3] as

Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

Therefore, similar as 5G, the requirement to RAN architecture shall be:

- The design of the RAN architecture shall allow the deployment of new services (e.g., AI/ML, Sensing) rapidly and efficiently.

Proposal 3: The design of the RAN architecture shall allow the deployment of new services (e.g., AI/ML, Sensing) rapidly and efficiently.

There is large amount of data (e.g. training data, sensing data) exchanged between UE, RAN, and CN, as shown in Table 1 and Figure 1 below.

Table 1: Data transfer between UE, RAN and CN

|  | UE and RAN | UE and CN | RAN and CN |
| --- | --- | --- | --- |
| AI / ML | UE collects / reports training data (e.g., measurement results) for NW-side model training. | For AI / ML UE-side data collection, UE transfer its collected data for UE-side model training to its own OTT server via application layer (i.e., transparent to 3GPP). | RAN forward the received training data from UE to OAM. To support two-sided model, NW transfers collected/measured information (e.g., dataset / model parameter) to UE-side OTT server for two-sided model training. Efficient data exchange and computing sharing between RAN and Core for AI/ML RAN use cases. |
| Sensing | Sensing results measured by UE is reported to RAN for communication optimization. | Sensing UE reports sensing results to either SF in CN or to sensing client directly | RAN forward the received sensing results from UE to sensing client. When TRP-monostatic/TRP-TRP bistatic is used, RAN can report sensing results to SF in CN. |

Figure 1: Data transfer between UE, RAN and CN

Different types of data could have different QoS requirements. For example, as discussed in Rel-19 AI/ML, training data for offline training is delay tolerant while data for inference is delay sensitive.

Furthermore, it was also agreed in RAN2 #132 meeting to further study below aspects for data transfer framework:

Agreements

Study the follow aspects as a starting point:

Data types (non-real time)

-	AI/ML (data collection for offline training)

-	SON/MDT, QoE - (details on what is included in 6G depends on RANP)

-	Sensing (the detailed characteristics (including what cases are real time or not) will be discussed in later phase, after some RAN1 and RAN2 discussions in the sensing session)

QoS (e.g. data volume and latency)

-	Define some categories we want to base our discussion on.   Details of QoS for some cases may depend on other WGs.

Termination points (impacted group)

-	UE to RAN

-	UE to CN

-	UE to OAM

-	UE to UE server

Understand the consumer (may be different than the termination) of the data and whether data has to be decodable by RAN.

In light of above data transfer and processing requirements of AI/ML and sensing, RAN architecture shall allow for efficient transmission of large amount of data between UE, RAN and CN. It is envisioned that data plane between UE and RAN, UE and CN, and RAN and CN can be introduced with the following goals:

- Support innate data(e.g. AI / Sensing) transmission between UE and RAN, UE and CN, and RAN and CN.

- Reduce signaling overhead and transmission latency by bypassing AMF for AI/Sensing service.

Proposal 4: The RAN architecture shall support efficient transmission: 1) for diverse data types (e.g., AI, sensing, SON/MDT) and QoS requirements; 2) between different end point pairs (UE and RAN, UE and CN, RAN and CN).

## Migration requirements

For migration, RAN plenary has concluded that:

- The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. Additional migration options - RAN plenary start study in March 2026 - RAN plenary study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G) - RAN plenary to make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s) - Scope of normative work on migration functionalities to be done upon WG work item scoping. - Same applies to 6G-6G DC

Therefore, we only focus on the requirements for inter-RAT mobility between the 6GR and NR.

In RAN2#132, following agreements have been made:

Agreements

1	For inter-RAT mobility (5G and 6G), the following is supported as a starting point:

­	For RRC_CONNECTED state, both handover and re-direction can be supported for inter-RAT mobility

­	Inter-RAT cell reselection information can be provided to UE.

2	Wait for further mobility discussions on the intra-RAT solutions, before coming back to look at the need for further enhancements.  Consider MRSS.

During the discussion, some companies raised questions on whether features introduced in Intra-RAT mobility can also be applied for Inter-RAT mobility between 6GR and NR, e.g., early sync, early processing, etc.

To our understanding, considering the supported high data rate in 5G, to avoid the impact on user experience, NR has improved the mobility interruption a lot with LTM, etc. We expect that the similar mechanisms will be introduced in 6G as well to meet the mobility requirements. However, the interruption and the data loss during traditional inter-RAT handover, e.g., 4G-5G cannot be avoided. We do see the requirements to improve the inter-RAT handover as well in order to avoid downgrade of the use experience.

Proposal 5: The RAN architecture shall support high performing interworking between the 6GR and NR.

Proposal 6: Capture following architecture and migration requirements in the TR 38.914:

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support high performing inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	RAN-CN interfaces and RAN internal interfaces shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

-	The design of the RAN architecture shall allow the deployment of new services (e.g., AI/ML, Sensing) rapidly and efficiently.

-	The RAN architecture shall support efficient transmission of diverse service data (e.g., AI, sensing, SON/MDT) and QoS requirements between different end point pairs (UE and RAN, UE and CN, RAN and CN).

# Conclusion

In this contribution, we discuss requirements for architecture and migration and propose the following:

Proposal 1: RAN-CN interfaces and RAN internal interfaces shall be open for multi-vendor interoperability.

Proposal 2: The RAN architecture shall support efficient signalling transfer between UE/RAN and CN for diverse services, e.g., sensing, AI in efficient and scalable way.

Proposal 3: The design of the RAN architecture shall allow the deployment of new services (e.g., AI/ML, Sensing) rapidly and efficiently.

Proposal 4: The RAN architecture shall support efficient transmission: 1) for diverse data types (e.g., AI, sensing, SON/MDT) and QoS requirements; 2) between different end point pairs (UE and RAN, UE and CN, RAN and CN).

Proposal 5: The RAN architecture shall support high performing interworking between the 6GR and NR.

Proposal 6: Capture following architecture and migration requirements in the TR 38.914:

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support high performing inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	RAN-CN interfaces and RAN internal interfaces shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

-	The design of the RAN architecture shall allow the deployment of new services (e.g., AI/ML, Sensing) rapidly and efficiently.

-	The RAN architecture shall support efficient transmission of diverse service data (e.g., AI, sensing, SON/MDT) and QoS requirements between different end point pairs (UE and RAN, UE and CN, RAN and CN).

# 4 References

[1] RP-252909 Moderator's summary for RAN led 6G SI: Requirements for architecture and migration		NTT DOCOMO

[2] RP-252870 pCR for 38.914 on Requirements for architecture and migration for 6G	NTT DOCOMO (moderator)

[3] RP-251881, NTT DOCOMO (Moderator), “New SID: Study on 6G Radio”

# 5 Annex Requirements for architecture and migration of NR (TR 38.913 clause 8)

The RAN design for the Next Generation Radio Access Technologies shall be designed to fulfil the following requirements:

-	The RAN architecture shall support tight interworking between the new RAT and LTE.

-	Considering high performing inter-RAT mobility and aggregation of data flows via at least dual connectivity between LTE and new RAT. This shall be supported for both collocated and non-collocated site deployments.

-	The RAN architecture shall support connectivity through multiple transmission points, either collocated or non-collocated.

-	The RAN architecture shall enable a separation of control plane signalling and user plane data from different sites.

-	The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination.

-	Different options and flexibility for splitting the RAN architecture shall be allowed.

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable context aware service delivery, low latency services, etc...

-	The RAN architecture shall allow for C-plane/U-plane separation.

-	The RAN architecture shall allow deployments using Network Function Virtualization.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

-	The RAN architecture shall allow for the operation of Network Slicing3GPP TR 23.799 [7].

-	The RAN architecture shall support sharing of the RAN between multiple operators.

-	The design of the RAN architecture shall allow the deployment of new services rapidly and efficiently.

-	The design of the RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational).

-	The design of the RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services.

-	RAN-CN interfaces and RAN internal interfaces (both between new RAT logical nodes/functions and between new RAT and LTE logical nodes/functions) shall be open for multi-vendor interoperability.

-	The RAN architecture shall support operator-controlled sidelink (device-to-device) operation, both in coverage and out of coverage.