3GPP TSG RAN Meeting #110		RP-253240

Baltimore, USA, 8th – 11th December, 2025

Agenda item:	8.2.1.3.1

Source:	T-Mobile USA Inc., Nokia

Title:	Unlicensed operations for 6G

Document for:	Approval

# 1	Introduction

This document discusses and proposes 6G requirements for boosting 6G uplink performance with unlicensed spectrum operation. The document also includes a text proposal to be captured into the TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements.

# 2	Discussion

Traffic growth is accelerated by new and extended services and applications like XR, FWA and device AI, as well as technological advancements like use of edge computing with more processing being done at the edge of the network. Therefore, there is a need for efficient uplink data transmission from devices to edge servers. New and extended applications and services are likely to accelerate the growth of uplink traffic, while downlink traffic is also expected to continue to grow. Therefore, in 6G we see a need for faster uplink and more uplink capacity, but at the same time it is important to avoid sacrificing downlink performance.

One way to obtain additional uplink resources without impacting downlink capacity and downlink latency is by exploiting U-NII-7 unlicensed spectrum to boost the uplink. Uplink transmission is better suited to U-NII-7 unlicensed spectrum operation than downlink transmission due to its lower transmit power levels than downlink transmission. In U-NII-7, “Standard Power” operation under the control of Automatic Frequency Coordination (AFC) allows outdoor use with 30 dBm EIRP and 17 dBm/MHz EIRP PSD [2] without explicit channel access procedures.  (See other device classes for U-NII-7 defined in Table A-1 in ANNEX A.). 6G uplink on U-NII-7 unlicensed spectrum would complement 6G TDD operations on licensed spectrum (e.g. around 7 GHz), which would deliver enhanced uplink coverage, latency and capacity.

To avoid 6G technology and market fragmentation it would be beneficial to enable 6G uplink operation in unlicensed spectrum using mainstream 6G radio design like 6G carrier aggregation, flexible UL/DL pairing and UL Tx switching.

# 3	Conclusion

In conclusion, we propose to introduce 6G requirements for boosting 6G uplink coverage, latency and capacity by enabling 6G uplink transmission on unlicensed spectrum like U-NII-7 to support new and extended services and user demands. 6G uplink transmission on unlicensed spectrum U-NII-7 would complement regular licensed band TDD operations e.g. around 7 GHz frequency.

We propose the following requirement text to be introduced in TR 38.914.

--- Text Proposal starts ---

## 5.4	Requirements of New and Existing Services

To meet increasing uplink traffic demands of new and extended services like XR, FWA and device AI, as well as technological advancements like use of edge computing, the 6G system shall be able to support 6G uplink operation using mainstream 6G radio design on unlicensed spectrum, such as U-NII-7, which allows standard power operation.

--- Text proposal ends ---

# References

[1] RP-251395 Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

- [2] See 47 C.F.R § 15.407(a) and 47 C.F.R. § 15.407(a)(7)

# ANNEX A

- Table A-1

- Mobile Client includes 3GPP UE portables (portable designation includes smartphones as defined by FCC).  The Mobile Client is indirectly AFC controlled via its serving AFC controlled Base Station (BS).  An AFC controlled BS is an SP AP as is an AFC controlled FWA CPE.