3GPP TSG RAN Meeting #110			RP-253242

Baltimore, US, December 8-11, 2025

Agenda item:	8.2.1.1

Title:	Views on 6G key performance indicators

Source:	SK Telecom

Document for:	Discussion

1. Introduction

At RAN#109, pCR for TR 38.914 on key performance indicators(KPIs) for 6G based on the latest ITU working document  has been approved as follows [2]:

### 5.1.1 Peak data rate [Peak data rate is the theoretical maximum [achievable] data rate under ideal conditions, which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times). Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by: Rp = W × SEp Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands, then the total peak data rate is R=i=1Q Wi × SEpi where Wi and SEpi (i = 1, …, Q) are the component bandwidths and spectral efficiencies respectively. The minimum requirements for peak data rate are as follows: – Downlink peak data rate is TBD Gbit/s. – Uplink peak data rate is TBD Gbit/s.] …

As reflected in the discussions captured in the previous pCR, additional work is still required to reach agreement on how the Peak Data Rate should be defined or represented. Likewise, the topic of Coverage was not sufficiently addressed during the RAN#109 meeting, and no clear consensus was reached on the appropriate metric or evaluation methodology.

Therefore, through this contribution, we present our view on both Peak Data Rate and Coverage, with the objective of facilitating further discussion and helping the group converge toward a consistent and technically sound approach for 6G performance characterization.

2. Discussion

2.1 Peak data rate

To compute the Peak Data Rate, both the available frequency bandwidth and the corresponding Peak Spectral Efficiency need to be specified. However, determining a concrete Peak Data Rate value for 6G remains infeasible at this stage, as the IMT-2030 candidate spectrum has not yet been finalized. The identification and allocation of 6G frequency ranges will only be clarified after WRC-27, leaving the bandwidth assumptions highly uncertain until then.

It is also noted that the current work on Peak Spectral Efficiency already incorporates the expected gains from major 6G technology enablers, including advancements of modulation and channel coding, # of layers, etc. These metrics already capture improvements driven by physical-layer innovations without requiring premature assumptions about spectrum availability.

Given these uncertainties, specifying a numerical Peak Data Rate would not be technically justified and could unintentionally create unrealistic expectations for external stakeholders. The 5G experience further illustrates that the originally advertised Peak Data Rates have often been perceived as unattainable, and similar issues may arise again in 6G era if speculative numbers are included in early 6G documents.

Therefore, we propose the following options regarding the Peak Data Rate in the TR:

- We propose the following options regarding the Peak Data Rate in the TR:

- Option 1: Do not define a Peak Data Rate value at this stage, or, if the group believes a reference metric is still useful, designate it explicitly as a “Theoretical Peak Data Rate” to clarify that it is based on hypothetical bandwidth assumptions and does not represent deployable performance.

- Option 2: If the group prefers to retain some figures for Peak Data Rate in the TR, the proponent shall provide a specific numerical proposal together with the explicit assumptions used (e.g., bandwidth, spectral efficiency), ensuring transparency and avoiding misleading interpretation.

We believe the above approach can provide a more realistic and technically sound handling of Peak Data Rate while avoiding unnecessary speculation during the early phase of 6G standardization.

2.2 Coverage

Although “Coverage” is not currently defined as a KPI item, it continues to be an important topic of discussion within the group. In TR 38.914, the following statement has already been included:

Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

Given this context, it is reasonable and practical to assess comparable coverage using the same methodology that was applied for the 5G mid-band. Leveraging an established comparison framework will ensure consistency with prior IMT evaluations and facilitate a clearer understanding of 6G deployment expectations.

In 5G, coverage was represented by using the Maximum Coupling Loss (MCL) metric. We understand that the group is currently discussing whether 6G coverage should be expressed using MCL or the alternative Maximum Path Loss (MPL) metric. To the best of our knowledge, the definitions of these two metrics are as follows:

- MCL (Maximum Coupling Loss): The difference between the transmit and receive power levels measured at the respective antenna ports, representing the coupling loss between the transmitting and receiving antennas.

- MPL (Maximum Path Loss): The maximum loss observed along the propagation path, accounting for various environmental factors affecting signal attenuation.

While both metrics can theoretically be used to characterize coverage, adopting MPL would introduce a new baseline that is not directly aligned with the methodology previously used for 5G. In contrast, maintaining MCL would ensure compatibility with established 5G coverage evaluations and simplify direct comparison, which is essential given the “comparable coverage” requirement stated in TR 38.914.

- We propose that MCL, rather than MPL, be used for expressing 6G coverage, in order to maintain consistency with 5G mid-band evaluations.

It should be noted that if a proponent provides strong and technically justified rationale for MPL, we do not intend to preclude such consideration. However, from a comparability and continuity perspective, MCL remains the preferred metric.

3. Conclusion

In this contribution, the following proposals are provided.

- We propose the following options regarding the treatment of Peak Data Rate in the TR:

- Option 1: Do not define a Peak Data Rate value at this stage, or, if the group believes a reference metric is still useful, designate it explicitly as a “Theoretical Peak Data Rate” to clarify that it is based on hypothetical bandwidth assumptions and does not represent deployable performance.

- Option 2: If the group prefers to retain a Peak Data Rate figure in the TR, the proponent shall provide a specific numerical proposal together with the explicit assumptions used (e.g., bandwidth, spectral efficiency), ensuring transparency and avoiding misleading interpretation.

- We propose that MCL, rather than MPL, be used for expressing 6G coverage, in order to maintain consistency with 5G mid-band evaluations.

References

- RP-252947, “pCR for 38.914 on Key performance indicators for 6G”, CMCC (moderator), RAN#109.

- RP-252882, “R 38.914 v0.2.0 for Study on 6G Scenarios and requirements”, RAN#109.

- RP-252970, “Agenda for RAN #110,”  RAN Chair (Samsung), Beijing, China, September 15–18, 2025.