3GPP TSG-RAN Meeting #110		RP-253254

Baltimore, US, December 08-11, 2025

Source:	vivo

Title:	Discussion on 6G voice fallback

Agenda Item:	8.2.1.4

Document for:	Discussion and Decision

- Introduction

In [1,3], the following agreement were achieved in the last RAN meeting. This paper further provides views on supporting voice fallback for 6G day-1.

Moderator proposal 1: It is agreed to capture following into TR 38.914 section 5.4:

- Mobile broadband

- Immersive Communication

- Massive Communication (IoT)

- Sensing

- AI

- Voice

- Regulatory services

- Discussion

In our view, support of voice service is critical for 6GR. Voice services require minimum interruption during mobility scenarios, necessitating fast switching between 5G and 6G networks. Additionally, an effective fallback solution may be needed to ensure sufficient coverage and stable user experience, as it remains unclear whether 6G native voice would be lunched commercially in Day-1.

There are two potential scenarios to deploy voice services on 6G Day-1:

- Scenario 1: 6G Native Voice Deployment. 6G native voice deploys on Day-1 over 6G radio. If a UE moves out of 6G coverage, inter-RAT handover is required for service continuity for voice.

- Scenario 2: Voice Fallback to 5G. If no native 6G voice deploys for Day-1, voice falls back to 5G.

UE initiates voice services directly in 6G when UE camping on or connected to a 6GS, enabling 6G to fully support all service types and provide better performance, particularly for concurrent voice and other 6G services. To support 6G native voice on Day-1, some features are needed, such as voice indication in RRC Connection Request, paging priority, flow based QoS and mapping, SM signaling, UE capability, intra/inter RAT handover, suitable L2 configuration and mechanisms.

Table 1: RAN functions to support voice in 6G

| Feature Name | Protocol Layer | Category |
| --- | --- | --- |
| Voice, video, emergency indication in RRC Connection Request | RRC | Call setup success rate optimization |
| Paging priority | NGAP | Call setup success rate optimization |
| SSAC | RRC | Call setup success rate optimization |
| Flow based QoS, GBR, PBR, LCP, DRB combinations | MAC | Basic function |
| SM signaling | NGAP | Basic function |
| UE capability match | NGAP | Basic function |
| Intra/Inter RAT handover | system | Basic mobility |
| SPS | MAC | Power/Overhead optimization |
| RoHC | PDCP | Coverage/Overhead optimization |
| Short L2 SN | L2 | Coverage/Overhead optimization |
| MAC CE based rate adaption | MAC | Reliability and spectral efficiency optimization |

Another issue to consider is the call setup rate. Based on our experience and observations from 5G deployments, it took a long time, even more than one year, for VoNR to mature. According to our survey, statistics on successful call setup rate during early NR SA deployment are illustrated in figure 2. Even in areas with relatively good NR SA coverage, the successful call setup rate remains lower than that in LTE for most of the tested UE models. The deployment and performance of 6G native voice gradually maturing require accumulated time.

Figure 2 successful call setup rate for LTE and NR SA for Mobile-Originated and Mobile-Terminated cases

We see two issues for 6G Native Voice Deployment.

Issue1: IMS system deployment and maturation require time to accumulate

The deployment and maturation of the IP Multimedia Subsystem (IMS) core require considerable time. As a complex ecosystem of network functions, integrating the IMS with the new 6G core and ensuring carrier-grade reliability and seamless interworking with previous generations (5G) is a protracted process of testing and optimization that cannot be completed at launch.

Issue2: RAN parameters configuration for voice supporting requires time to accumulate

The Radio Access Network (RAN) necessitates highly precise configuration to support voice's stringent latency and reliability demands. Parameters for scheduling, mobility management, and especially beam management in 6G's envisioned high-frequency bands require extensive field data and iterative tuning in a live network environment.

The combined effect of the IMS's slow maturation cycle and the RAN's post-deployment parameter optimization will inevitably delay the availability of robust native voice, making it a service that matures after the initial 6G rollout. Similar with the 5G voice rollout, 6G native voice may not be deployed from Day-1. Hence, voice fallback to 5G would be enabled through IRAT handover or redirection.

Observation 1: 6G shall support natively voice from Day-1, however deploying 6GR voice across the entire network on Day-1 is challenging due to 6G RAN parameters configuration for voice and IMS system needs time for maturation. Without voice fall back to 5G, voice may not be adequately supported during early 6G deployments.

Hence, we propose that

Proposal 1：Voice fallback from 6GR to 5G should be supported.

- Conclusion

In this contribution, we discuss supporting voice fallback on 6G Day-1.  we have the following proposal:

Proposal 1：Voice fallback from 6GR to 5G should be supported.

- Reference

- RP-252971, Meeting Report for RAN #109;

- RP-251881, Revised SID: Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom;

- TR38.914, Study on 6G Scenarios and Requirements, V0.2.0，2025-09