3GPP TSG RAN Meeting #110                                                       RP-253256

Baltimore, USA, December 8-11, 2025

Source:	vivo

Title:	Discussion on 6G requirements for ISAC

Agenda Item:	8.2.1.4

Document for:	Discussion and Decision

Introduction

In 3GPP RAN#109, the following text proposal was agreed for 6GR sensing use cases.

Agreement： ### 5.4.4 Sensing Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

The agreed use cases are mainly from 5G sensing use cases. For 6GR sensing, there are more use cases worth studying. In this contribution, we further discuss more use cases and requirements for ISAC.

Discussion

Detection and/or tracking of UAVs, human, vehicles and AGVs

It is agreed 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of UAVs, human, vehicles and AGVs. Table 1 and Table 2 provide the deployment scenarios and KPIs for use cases of detection and/or tracking, respectively.

Table 1: Deployment scenarios for object detection and tracking

| Sensing target of detection and/or tracking | Related deployment scenario |
| --- | --- |
| UAV | Urban macro |
| Human | Outdoor: Urban macro/urban grid Indoor: Indoor hotspot/factory |
| AGV | Indoor factory |
| Vehicle | Urban grid |

Table 2: KPIs for object detection and tracking

| Use cases | Confidence level [%] | Positioning accuracy[m] |  | Radial velocity accuracy[m/s] | Sensing latency[ms] | Missed detection probability | False alarm probability |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal | vertical |  |  |  |  |
| UAV | [90%] | [2] | [2] | [2] | [<1000] | [5%] | [5%] |
| Human | [90%] | [2] | N/A | [0.5] | [<1000] | [5%] | [5%] |
| AGV | [90%] | [1] | N/A | [1] | [<1000] | [5%] | [5%] |
| Vehicle | [90%] | [2] | N/A | [2] | [<1000] | [5%] | [5%] |

Proposal 1: The deployment scenarios and KPIs in Table 1 and Table 2 for detection and/or tracking of UAVs, human, vehicles and AGVs are supported.

Sensing-assisted communication

Besides the use cases of detection and/or tracking of passive objects including UAVs, human, vehicles and AGVs, sensing-assisted communication should be prioritized for study in 6GR.

In the past, communication operated “blindly”. Now, thanks to the Integrated Sensing and Communication (ISAC), we can achieve “environment-aware” communication as shown in Figure 1. Assistant information, such as environmental information or channel knowledge maps, can be leveraged to enhance communication performance. This includes reducing beam management latency or improving handover success rates, or to optimize the power consumption of communication etc., depending on the different use cases of sensing-assisted communication. The related KPIs are communication performance metrics such as spectrum efficiency, user perceived throughput (UPT) etc., depending on the specific use cases. The deployment scenario is urban grid.

Proposal 2: Sensing-assisted communication should be prioritized in the study of sensing. The related KPIs are communication performance metrics such as spectrum efficiency, user perceived throughput (UPT) etc., depending on the specific use cases. The deployment scenario is urban grid.

Figure 1: Example of sensing-assisted communication

Micro-Doppler related use cases

Sensing targets or their components often exhibit micro-motion such as vibration, rotation, and acceleration. These micro-motions modulate the phase of the echo signal, resulting in corresponding frequency modulation, which is known as the micro-Doppler effect. To support the micro-motion detecting and tracking, the micro-Doppler model in current ISAC channel model should be further enhanced since the Doppler frequency is defined as:

fD,n',m',n,mk,pt=rrx,k,p,n',m'Ttvrxt+rk,p,n',m'Ttvk,ptλ0+rtx,k,p,n,mTtvtxt+rk,p,n,mTtvk,ptλ0

where the velocity of micro movement has not been defined. However, different micro movement type may lead to different mathematical model. For instance, the rotational local movement, such as the rotation of the wings from UAV in Figure 2, which is moving regularly, can be modeled as the circular model and the micro velocity can be modeled by the circular law. On the other hand, the non-circular movement with obvious regular movement like swinging of human arms or the chest cavity movements caused by breathing can be modeled as the mathematical model of trigonometric functions. Therefore, in order to provide the evaluation basis for the detection or tracking in the use case of micro motion, the detailed velocity model of micro movement can be modeled based on the sensing use case with several typical motion characteristics. The detailed discussion and the proposals about micro-Doppler formula can be found in [1].

Figure 2: Blade rotation of UAV causing micro-Doppler

For UAV use cases, micro-Doppler is primarily caused by the rotation of UAV blades. When modeling the Doppler shift of UAVs, in addition to considering the UAV's translational motion, it is necessary to model the micro-Doppler characteristics based on the angular velocity and rotational radius of the UAV blades. Micro-Doppler detection involves extracting time-domain signal sequences associated to detected paths (sampling points), followed by time-frequency analysis to derive corresponding micro-Doppler information, as illustrated in Figure 3. The spectrum reflects combined Doppler frequency related to the translational motion of the UAV and micro-Doppler induced by its two rotating blades.

Figure 3: Illustration of UAV micro-Doppler spectrum.

Micro-Doppler modeling is beneficial not only for identifying characteristics of different UAVs but also enhances UAV detection performance, particularly when the radial velocities of the UAV is low and its echo may be obscured by static clutter. Modeling the blade rotation-induced micro-Doppler improves detection probability as illustrated in Figure 4. The left plot without micro-Doppler modeling shows no detectable target in the delay-Doppler spectrum after static clutter suppression, whereas the right plot with micro-Doppler modeling demonstrates distinguishable target echoes from static clutter due to the micro-Doppler signature.

Figure 4: Delay-Doppler spectrum with and without micro-Doppler modeling.

- Observation 1: Micro-Doppler modeling improves detection probability of UAVs under low radial velocity conditions and facilitates identification of UAV-specific characteristics, enabling differentiation of UAV types and discrimination from other flying objects such as birds.

Another ISAC use case related to micro-Doppler is human respiration. In the field of healthcare and elderly care, ISAC can provide services including the monitoring of vital signs such as respiration (as shown in Figure 5) and heart rate. The detection of respiration and heart rate, etc., requires the detection of micro-Doppler caused by the human body.

The related deployment scenarios of blade rotation of UAV and human respiration can reuse the deployment scenarios for UAV and human in Table 1. The related KPI is accuracy of micro-Doppler.

Figure 5: Human respiration use case defined in 3GPP TR 22.837

Proposal 3: 	Micro-Doppler related use cases including blade rotation of UAV and human respiration should be prioritized in the study of sensing. The related deployment scenarios of blade rotation of UAV and human respiration can reuse the deployment scenarios for UAV and human in Table 1. The related KPI is accuracy of micro-Doppler.

Conclusion

In this contribution, we provide our considerations on 6G use cases and requirements and the proposals are as following:

- Observation 1: Micro-Doppler modeling improves detection probability of UAVs under low radial velocity conditions and facilitates identification of UAV-specific characteristics, enabling differentiation of UAV types and discrimination from other flying objects such as birds.

Proposal 1: The deployment scenarios and KPIs in Table 1 and Table 2 for detection and/or tracking of UAVs, human, vehicles and AGVs are supported.

Table 1: Deployment scenarios for Object detection and tracking

| Sensing target of detection and/or tracking | Related deployment scenario |
| --- | --- |
| UAV | Urban macro |
| Human | Outdoor: Urban macro/urban grid Indoor: Indoor hotspot/factory |
| AGV | Indoor factory |
| Vehicle | Urban grid |

Table 2: KPIs for Object detection and tracking

| Use cases | Confidence level [%] | Positioning accuracy[m] |  | Radial velocity accuracy[m/s] | Sensing latency[ms] | Missed detection probability | False alarm probability |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal | vertical |  |  |  |  |
| UAV | [90%] | [2] | [2] | [2] | [<1000] | [5%] | [5%] |
| Human | [90%] | [2] | N/A | [0.5] | [<1000] | [5%] | [5%] |
| AGV | [90%] | [1] | N/A | [1] | [<1000] | [5%] | [5%] |
| Vehicle | [90%] | [2] | N/A | [2] | [<1000] | [5%] | [5%] |

Proposal 2: Sensing-assisted communication should be prioritized in the study of sensing. The related KPIs are communication performance metrics such as spectrum efficiency, user perceived throughput (UPT) etc., depending on the specific use cases. The deployment scenario is urban grid.

Proposal 3: Micro-Doppler related use cases including blade rotation of UAV and human respiration should be prioritized in the study of sensing. The related deployment scenarios of blade rotation of UAV and human respiration can reuse the deployment scenarios for UAV and human in Table 1. The related KPI is accuracy of micro-Doppler.

Reference

[1] R1-2505767, Considerations on channel modeling and evaluation assumtions for 6GR air interface, BUPT, vivo.